3GPP TSG-RAN5 Meeting #105	R5-247834

Orlando, US, 18th – 22nd November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.905 | CR | 0278 | rev | 1 | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test point analysis for Inter-band 4DL & 5DL CA for REFSENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton International INC., Bureau Veritas ADT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Inter-band 4DL CA_14A-66A-66A-66A and 5 DL CA_2A-14A-66A-66A-66A is missing for REFSENs test cases in TS36.905 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the following analysis files as attachment in this CR: “TpAnalysis4DLRefSens_7.3A.9(14A-66A-66A-66A).zip” “TpAnalysis5DLRefSens_7.3A.10(2A-14A-66A-66A-66A).zip” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-band 4DL CA_14A-66A-66A-66A and 5 DL CA_2A-14A-66A-66A-66A will be still missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.6.4, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-1 CR 5526 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - Meeting date is correct. This is the final outcome of one revision of R5-246947. |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

4.2.6.4	Intra-band non-contiguous + Inter-band

In this test case, there are default test points for each intra-band non-contiguous band to be used unless CA configuration specific test points are over-ruling in table 4.2.6.4-1. The default test points are based on fallback non-contiguous CA test points, with the inter-band CC using Max NRB and Mid test frequency, as per default Inter-Band test points. For CA configurations where default test points are used and the 3DL fallback cases also use default test points there is no need for justification in Table 4.2.6.2-1.

Table 4.2.6.4-1: CA configuration specific test points for 4DL CA (Intra-band non-contiguous + Inter-band)

| CA config | Justification | Comments |
| --- | --- | --- |
| CA_2A-2A-4A-5A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-4A-5A).zip” | Added at RAN5#73 |
| CA_2A-2A-4A-71A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-4A-71A).zip” | Added at RAN5#82 |
| CA_2A-2A-5A-12A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-5A-12A).zip” | Added at RAN5#73 |
| CA_2A-2A-5A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-5A-66A).zip” | Added at RAN5#100 |
| CA_2A-2A-12A-30A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-12A-30A).zip” | Added at RAN5#75 |
| CA_2A-2A-12A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-12A-66A).zip” | Added at RAN5#100 |
| CA_2A-2A-29A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-29A-66A).zip” | Added at RAN5#102 |
| CA_2A-2A-30A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-30A-66A).zip” | Added at RAN5#100 |
| CA_2A-2A-66A-71A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-66A-71A)_v1.zip” | Added at RAN5#85 |
| CA_2A-4A-4A-5A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-4A-4A-5A).zip” | Added at RAN5#85 |
| CA_2A-4A-7A-7A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-4A-7A-7A).zip” | Added at RAN5#77 |
| CA_2A-4A-7A-7A_UL_2A-4A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-4A-7A-7A).zip” | Added at RAN5#77 |
| CA_2A-5A-66A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-5A-66A-66A).zip” | Added at RAN5#100 |
| CA_2A-12A-66A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-12A-66A-66A).zip” | Added at RAN5#75 |
| CA_2A-29A-66A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-29A-66A-66A).zip” | Added at RAN5#102 |
| CA_2A-66A-66A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-66A-66A-66A).zip” | Added at RAN5#101 |
| CA_2A-66A-66A-71A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-66A-66A-71A).zip” | Added at RAN5#82 |
| CA_3A-3A-7A-7A | See attachment “TpAnalysis4DLRefSens_7.3A.9(3A-3A-7A-7A).zip” | Added at RAN5#75 |
| CA_4A-4A-5A-30A | See attachment “TpAnalysis4DLRefSens_7.3A.9(4A-4A-5A-30A).zip” | Added at RAN5#73 |
| CA_4A-4A-12A-30A | See attachment “TpAnalysis4DLRefSens_7.3A.9(4A-4A-12A-30A).zip” | Added at RAN5#73 |
| CA_4A-4A-29A-30A | See attachment “TpAnalysis4DLRefSens_7.3A.9(4A-4A-29A-30A).zip” | Added at RAN5#73 |
| CA_5A-30A-66A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(5A-30A-66A-66A).zip” | Added at RAN5#100 |
| CA_12A-30A-66A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(12A-30A-66A-66A).zip” | Added at RAN5#100 |
| CA_14A-66A-66A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(14A-66A-66A-66A).zip” | Added at RAN5#105 |
| CA_29A-66A-66A-70A | See attachment “TpAnalysis4DLRefSens_7.3A.9(29A-66A-66A-70A).zip” | Added at RAN5#79 |
| CA_29A-30A-66A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(29A-30A-66A-66A).zip” | Added at RAN5#101 |
| CA_66A-66A-70A-71A | See attachment “TpAnalysis4DLRefSens_7.3A.9(66A-66A-70A-71A).zip” | Added at RAN5#79 |

4.2.6.5	Intra-band non-contiguous + Intra-band contiguous

Table 4.2.6.5-1: CA configuration specific test points for 4DL CA (Intra-band non-contiguous + Intra-band contiguous)

| CA config | Justification | Comments |
| --- | --- | --- |
| CA_2A-2A-12B | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-12B)_v2.zip” | Added at RAN5#75 |
| CA_2C-66A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2C-66A-66A).zip” | Added at RAN5#81 |
| CA_66A-66A-70C | See attachment “TpAnalysis4DLRefSens_7.3A.9(66A-66A-70C).zip” | Added at RAN5#79 |
| CA_5B-66A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(5B-66A-66A).zip” | Added at RAN5#101 |

4.2.6.6	Intra-band non-contiguous + Intra-band non-contiguous

In this test case, there are default test points for each intra-band non-contiguous band to be used unless CA configuration specific test points are over-ruling in table 4.2.6.6-1. The default test points are based on fallback non-contiguous CA test points For CA configurations where default test points are used and the 3DL fallback cases also use default test points there is no need for justification in Table 4.2.6.2-1.

Table 4.2.6.6-1: CA configuration specific test points for 4DL (Intra-band non-contiguous + Intra-band non-contiguous)

| CA config | Justification | Comments |
| --- | --- | --- |
| CA_2A-2A-4A-4A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-4A-4A).zip” | Added at RAN5#72 |
| CA_2A-2A-66A-66A | See attachment “TpAnalysis4DLRefSens_7.3A.9(2A-2A-66A-66A).zip” | Added at RAN5#101 |

4.2.7	Reference sensitivity level for 5DL CA

This section contains information on test point selection for test case 7.3A.10 Reference sensitivity level for 5DL CA.

[In the applicability conditions in TS 36.521-2 [4] it is defined that if the 5DL test is performed, testing of any 4DL or 3DL or 2DL fallbacks can be skipped specifically:

-	4DL fallback 5DL/1UL -> 4DL/1UL: Test of fallback can be skipped in many cases. Analysis supplied per CA configuration later in this clause.

-	4DL fallback 5DL/2UL -> 4DL/2UL: It is FFS if fallback need to be tested.

-	1UL fallback 5DL/2UL -> 5DL/1UL: Test of fallback is required due to different minimum requirements.]

This implies that the test point analysis described later in this section currently only considers 1UL.

4.2.7.1	Intra-band contiguous

FFS

4.2.7.2	Inter-band

In this case, there are default test points to be used unless CA configuration specific test points are over-ruling in table 4.2.7.2-1. The default rule of Mid test frequency and Highest NRB_agg is chosen for any CA configuration. For CA configurations where default test points are used and the 4DL fallback cases also use default test points there is no need for justification in Table 4.2.7.2-1.

Selection of test points should include some possible worst combinations and these can be based on operator preference in case of the CA configuration is operator specific. The number of test points should not exceed the total number of the maximum number of the test points to cover fallback of 2DL and 3DL and 4DL CA case.

Table 4.2.7.2-1: CA configuration specific test points for 5DL CA (Inter-band)

| CA config | Justification | Comments |
| --- | --- | --- |

4.2.7.3	Intra-band contiguous + Inter-band

In this test case, there are default test points to be used unless CA configuration specific test points are over-ruling in table 4.2.7.3-1. The default rule of Low and High test frequency for the band with 2 CC and Mid test frequency for the band with 1CC, Lowest and Highest NRB_agg for the band with 2 CC and Highest NRB_agg for the band with 1 CC is chosen for any CA configuration. For CA configurations where default test points are used and the 4DL fallback cases also use default test points there is no need for justification in Table 4.2.7.3-1.

Table 4.2.7.3-1: CA configuration specific test points for 5DL CA (Intra-band contiguous + Inter-band)

| CA config | Justification | Comments |
| --- | --- | --- |
| CA_1A-3A-18A-42C | See attachment “TpAnalysis5DLRefSens_7.3A.10(1A-3A-18A-42C).zip” | Added at RAN5#85 |
| CA_1A-3A-19A-42C | See attachment “TpAnalysis5DLRefSens_7.3A.10(1A-3A-19A-42C).zip” | Added at RAN5#75 |
| CA_1A-3A-41C-42A | See attachment “TpAnalysis5DLRefSens_7.3A.10(1A-3A-41C-42A).zip | Added at RAN5#83 |
| CA_1A-3A-41A-42C | See attachment “TpAnalysis5DLRefSens_7.3A.10(1A-3A-41A-42C).zip | Added at RAN5#83 |
| CA_1A-19A-21A-42C | See attachment “TpAnalysis5DLRefSens_7.3A.10(1A-19A-21A-42C).zip” | Added at RAN5#75 |
| CA_1A-41C-42C | See attachment “TpAnalysis5DLRefSens_7.3A.10(1A-41C-42C).zip” | Added at RAN5#76 |
| CA_3A-41C-42C | See attachment “TpAnalysis5DLRefSens_7.3A.10(3A-41C-42C).zip” | Added at RAN5#81 |
| CA_29A-66C-70C | See attachment “TpAnalysis5DLRefSens_7.3A.10(29A-66C-70C).zip” | Added at RAN5#79 |
| CA_66C-70C-71A | See attachment “TpAnalysis5DLRefSens_7.3A.10(66C-70C-71A).zip” | Added at RAN5#80 |

4.2.7.4	Intra-band non-contiguous + Inter-band

In this test case, there are default test points for each intra-band non-contiguous band to be used unless CA configuration specific test points are over-ruling in table 4.2.7.4-1. The default test points are based on fallback non-contiguous CA test points, with the inter-band CC using Max NRB and Mid test frequency, as per default Inter-Band test points. For CA configurations where default test points are used and the 3DL fallback cases also use default test points there is no need for justification in Table 4.2.7.2-1.

Table 4.2.7.4-1: CA configuration specific test points for 5DL CA (Intra-band non-contiguous + Inter-band)

| CA config | Justification | Comments |
| --- | --- | --- |
| CA_2A-2A-12A-66A-66A | See attachment “TpAnalysis5DLRefSens_7.3A.10(2A-2A-12A-66A-66A).zip” | Added at RAN5#101 |
| CA_2A-2A-29A-30A-66A | See attachment “TpAnalysis5DLRefSens_7.3A.10(2A-2A-29A-30A-66A).zip” | Added at RAN5#102 |
| CA_2A-2A-29A-66A-66A | See attachment “TpAnalysis5DLRefSens_7.3A.10(2A-2A-29A-66A-66A).zip” | Added at RAN5#102 |
| CA_2A-5A-30A-66A-66A | See attachment “TpAnalysis5DLRefSens_7.3A.10(2A-5A-30A-66A-66A).zip” | Added at RAN5#101 |
| CA_2A-14A-66A-66A-66A | See attachment “TpAnalysis5DLRefSens_7.3A.10(2A-14A-66A-66A-66A).zip” | Added at RAN5#105 |

4.2.7.5	Intra-band non-contiguous + Intra-band contiguous

Table 4.2.7.5-1: CA configuration specific test points for 5DL CA (Intra-band non-contiguous + Intra-band contiguous)

| CA config | Justification | Comments |
| --- | --- | --- |

4.2.7.6	Intra-band non-contiguous + Intra-band non-contiguous

In this test case, there are default test points for each intra-band non-contiguous band to be used unless CA configuration specific test points are over-ruling in table 4.2.7.6-1. The default test points are based on fallback non-contiguous CA test points For CA configurations where default test points are used and the 4DL fallback cases also use default test points there is no need for justification in Table 4.2.7.2-1.

Table 4.2.7.6-1: CA configuration specific test points for 5DL (Intra-band non-contiguous + Intra-band non-contiguous)

| CA config | Justification | Comments |
| --- | --- | --- |

4.2.7.7	Intra-band non-contiguous + Intra-band contiguous Inter-band + Inter-band

Table 4.2.7.7-1: CA configuration specific test points for 5DL CA (Intra-band non-contiguous + Intra-band contiguous Inter-band + Inter-band)

| CA config | Justification | Comments |
| --- | --- | --- |
| CA_29A-66A-66A-70C | See attachment “TpAnalysis5DLRefSens_7.3A.10(29A-66A-66A-70C).zip” | Added at RAN5#79 |
| CA_66A-66A-70C-71A | See attachment “TpAnalysis5DLRefSens_7.3A.10(66A-66A-70C-71A).zip” | Added at RAN5#80 |

## << End of changes >>

The following zipfile is added to TS 36.905.

TpAnalysis4DLRefSens_7.3A.9(14A-66A-66A-66A).zip

TpAnalysis5DLRefSens_7.3A.10(2A-14A-66A-66A-66A).zip