3GPP TSG-RAN5 Meeting #105 	R5-247837

# Orlando, USA, 18th Nov. 2024 - 22nd Nov. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1868 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MU for FR1 configured output power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MTSU/TT for FR1 configured transmitted power is based on incorrect UL power assumption. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MTSU/TT values for FR1 configured transmitted power were updated considering test points with UL power < 0 dBm. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will not be tested with correct MU and TT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.4.1.1, 6.2B.4.1.2, 6.2B.4.1.3, 6.2B.4.1.3_1, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on the outcome of MU discussion R5-247168 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-247170 and the outcome of 1 revision to this document. r1: MTSU/TT for TC 6.2B.4.1.1 was updated in Annex F. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.2B.4.1.1	Configured Output Power Level for Intra-Band Contiguous EN-DC

<<Unchanged sections skipped>>

Table 6.2B.4.1.1.5-3: Test Tolerance (UE configured UE output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| ENBW ≤ 40MHz | 0.7 dB1 1.0 dB2 | 1.0 dB | 1.0 dB |
| 40MHz < ENBW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |  |

6.2B.4.1.2	Configured Output Power for Intra-Band Non-Contiguous EN-DC

<<Unchanged sections skipped>>

Table 6.2B.4.1.2.5-3: Test Tolerance for UE configured UE output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | N/A |
| NR | BW ≤ 40MHz | 0.7 dB1 1.0 dB2 | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |  |  |

Table 6.2B.4.1.2.5-4: Test Tolerance for UE configured UE output power (Combined measurements of E-UTRA and NR CCs)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB1 1.0 dB2 | 1.0 dB | 1.0 dB | 0.7 dB1 1.0 dB2 | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |  |  |  |  |  |  |  |  |  |

6.2B.4.1.3	Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC)

<<Unchanged sections skipped>>

Table 6.2B.4.1.3.5-3: Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | N/A |
| NR | BW ≤ 40MHz | 0.7 dB1 1.0 dB2 | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |  |  |

Table 6.2B.4.1.3.5-4: Test Tolerance for UE maximum output power (Combined measurements of E-UTRA and NR CCs)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB1 1.0 dB2 | 1.0 dB | 1.0 dB | 0.7 dB1 1.0 dB2 | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band EN-DC configuration, ΔTIB,c in 6.2B.4.2 applies where unless otherwise stated, the same ΔTIB,c is applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔTIB,c is set to zero.

6.2B.4.1.3_1	Configured Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC)

<<Unchanged sections skipped>>

Table 6.2B.4.1.3_1.5-3: Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | N/A |
| NR | BW ≤ 40MHz | 0.7 dB1 1.0 dB2 | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |  |  |

Table 6.2B.4.1.3_1.5-4: Test Tolerance for UE maximum output power (Combined measurements of E-UTRA and NR CCs)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB1 1.0 dB2 | 1.0 dB | 1.0 dB | 0.7 dB1 1.0 dB2 | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band EN-DC configuration, ΔTIB,c in 6.2B.4.2 applies where unless otherwise stated, the same ΔTIB,c is applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔTIB,c is set to zero.

<<Unchanged sections skipped>>

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| <<Unchanged rows skipped>> |  |  |
| 6.2B.4.1.1 Configured Output Power Level for Intra-Band Contiguous EN-DC | MAX (MULTE, MUSA) | MULTE is MU of LTE specified in clause 6.2.4 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.2.4 in TS 38.521-1 [8]. |
| 6.2B.4.1.2 Configured Output Power for Intra-Band Non-Contiguous EN-DC | MAX (MULTE, MUSA) | MULTE is MU of LTE specified in clause 6.2.4 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.2.4 in TS 38.521-1 [8]. |
| 6.2B.4.1.3 Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) | MAX (MULTE, MUSA) | MULTE is MU of LTE specified in clause 6.2.4 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.2.4 in TS 38.521-1 [8]. |
| 6.2B.4.1.3_1 Configured Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) | MAX (MULTE, MUSA) | MULTE is MU of LTE specified in clause 6.2.4 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.2.4 in TS 38.521-1 [8]. |
| <<Unchanged rows skipped>> |  |  |

<<Unchanged sections skipped>>

F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| <<Unchanged rows skipped>> |  |  |
| 6.2B.4.1.1 Configured Output Power Level for Intra-Band Contiguous EN-DC | MAX (TTLTE, TTSA) | TTLTE is TT of LTE specified in 6.2.4 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.4 in TS 38.521-1 [8]. |
| 6.2B.4.1.2 Configured Output Power for Intra-Band Non-Contiguous EN-DC | MAX (TTLTE, TTSA) | TTLTE is TT of LTE specified in 6.2.4 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.4 in TS 38.521-1 [8]. |
| 6.2B.4.1.3 Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) | MAX (TTLTE, TTSA) | TTLTE is TT of LTE specified in 6.2.4 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.4 in TS 38.521-1 [8]. |
| 6.2B.4.1.3_1 Configured Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) | MAX (TTLTE, TTSA) | TTLTE is TT of LTE specified in 6.2.4 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.4 in TS 38.521-1 [8]. |
| <<Unchanged rows skipped>> |  |  |

<<End of change>>