3GPP TSG-RAN WG5 Meeting # 105	R5-247850

# Orlando, US, November 18 – 22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3141 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on definition of symbols and abbreviations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Tejet, SRTC, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To align with TS 38.101-1 v15.27.0, some symbols and abbreviations in clause 3.2 and 3.3 are missing or incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the symbols and abbreviations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The symbols and abbreviations will remain incorrect or missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-247402r1: (1) Typo correction on symbol “UTRAACLR”. (2) Add R&S as co-source company. (3) Resolve overlapping with R5-247003. R5-247402r2: - Add the reference core spec version in cover sheet. R5-247402r3: - Remove the changes not belong to Rel-15. |  |  |  |  |  |  |  |  |

<< Start of changes >>

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Δ Frequency of default TX-RX separation of the FDD operating band

∆MPRc	Allowed Maximum Power Reduction relaxation for serving cell c

ΔPPowerClass	Adjustment to maximum output power for a given power class

ΔR1R	Reference sensitivity adjustment due to support for 1 antenna ports

RB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWGB	max(BWGB,Channel(k))

BWGB,Channel(k)	Minimum guard band defined in clause 5.3A.1 of carrier k

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	Center frequency of a carrier for a numerology defined by the RF reference frequency on the channel raster mapped to the carrier according to sub-clause 5.4.2.2

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC, low	The Fc of the lowest carrier, expressed in MHz

FC, high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset., low

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset., high

Fedge, low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured). For intra-band contiguous CA, the FInterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the centre frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC, high to the higher edge or FC, low to the lower edge

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF,Shift	RF reference frequency for Supplementary Uplink (SUL) bands and for the uplink for all FDD bands, and TDD bands

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

GBChannel	Minimum guard band defined in clause 5.3.3, expressed in kHz

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=i=1jNRBi∗2μi for carrier 1 to j, where μ is defined in TS 38.211 [8]

Highest NRB_agg	The highest NRB_agg across Bandwidth Combination Sets supported by the UE as part of the CA Configuration.

Lowest NRB_agg	The lowest NRB_agg across Bandwidth Combination Sets supported by the UE as part of the CA Configuration.

NRB_agg=1jNRBj∗2μj

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj∗2μj for carrier j, where μ is defined in TS 38.211 [8]

NRB,cj=NRBj∗2μj

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRB,low	The transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	The transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PPowerClass	The nominal UE power (i.e., no tolerance)

Pmax,c,AC	Maximum output power defined as the sum of measurement of all antenna connectors

Pmax,c,TABC	Maximum carrier output power defined as the sum of measurement of all TAB connectors

Prated,c,AC	Rated maximum output power defined as the sum of power over all antenna connectors

Prated,c,TABC	Rated maximum output power defined as the sum of power over all TAB connectors

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBStart	The lowest RB index of transmitted resource blocks

RBstart_CA	Indicates the lowest RB index of transmitted resource blocks for intra-band contiguous CA

SCSc	SCS for the component carrier c, expressed in kHz

SCSlargest BW	SCS for the largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in kHz

SCSlow	SCS for the lowest assigned component carrier in section 5.3A.1, expressed in kHz

SCShigh	SCS for the highest assigned component carrier in section 5.3A.1, expressed in kHz

tp	Transient Period value signalled by the UE

tpstart	Start position of transient period relative to the symbol boundary

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

TL,c	Absolute value of the lower tolerance for the applicable operating band as specified in section 6.2.1

SSREF	SS block reference frequency position

UTRAACLR			UTRA ACLR

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

ATG	Air-To-Ground

BS	Base Station

BW	Bandwidth

BWP	Bandwidth Part

CA	Carrier Aggregation

CA_nX-nY	Inter-band CA of component carrier(s) in one sub-block within Band nX and component carrier(s) in one sub-block within Band nY where nX and nY are the applicable NR operating bands

CC	Component Carriers

CG	Carrier Groups

CM	Cubic Metric

CORESET	Control Resource Set

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DC	Dual Connectivity

DCI	Downlink Control Information

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DL	Downlink

DM-RS	Demodulation Reference Signal

DTX	Discontinuous Transmission

EIRP	Equivalent Isotropically Radiated Power

E-UTRA	Evolved UTRA

EVM	Error Vector Magnitude

FFT	Fast Fourier Transformation

FR	Frequency Range

FRC	Fixed Reference Channel

GNSS	Global Navigation Satellite System

GSCN	Global Synchronization Channel Number

HARQ	Hybrid Automatic Repeat Request

IBB	In-band Blocking

IDFT	Inverse Discrete Fourier Transformation

IE	Information Element

ITS	Intelligent Transportation System

ITU-R	Radio communication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MCG	Master Cell Group

MOP	Maximum Output Power

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

MU	Measurement Uncertainty

NR	New Radio

NR/5GC	NR connected to 5GC

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NS	Network Signalling

NSA	Non-Standalone

OCNG	OFDMA Channel Noise Generator

OOB	Out-of-band

PBCH	Physical Broadcast Channel

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

P-MPR	Power Management Maximum Power Reduction

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PSCCH	Physical Sidelink Control Channel

PSSCH	Physical Sidelink Shared CHannel

QAM	Quadrature Amplitude Modulation

RB	Resource Block

RE	Resource Element

RedCap	Reduced Capability

REFSENS	Reference Sensitivity

RF	Radio Frequency

RMC	Reference Measurement Channel

RMS	Root Mean Square (value)

RNTI	Radio Network Tempory Identity

RoT	Rise Over Thermal

RSRP	Reference Signal Receiving Power

Rx	Receiver

SA	Standalone

SC	Single Carrier

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SDL	Supplementary Downlink

SEM	Spectrum Emission Mask

SL	Sidelink

SL-MIMO	Sidelink-Multiple Antenna transmission

SNR	Signal-to-Noise Ratio

SRS	Sounding Reference Symbol

SUL	Supplementary uplink

SS	Synchronization Symbol

SS	System Simulator

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TAG	Timing Advance Group

TPC	Transmit Power Control

TT	Test Tolerances

Tx	Transmitter

TxD	Tx Diversity

UE	User Equipment

UL	Uplink

UL MIMO	Uplink Multiple Antenna transmission

ULFPTx	Uplink Full Power Transmission

V2X	Vehicle to Everything

==============================================================

<< End of changes >>