3GPP TSG-RAN5 Meeting #105	R5-247904

# Orlando, USA, 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3579 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of NR SA FR2 PCell uplink TCI state switch for two known TCI states test case including TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | # NR_MIMO_evo_DL_UL-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NR SA FR2 uplink TCI state switch for two known TCI states test case is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are being done: - Addition of test tolerances for the test case and measurement uncertainty - Addition of message contents |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case would be incomplete in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.13.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 0910 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Updated the editor’s notes based on offline comments. |  |  |  |  |  |  |  |  |

-------- Start of change 1--------

#### 7.5.13.6	MAC-CE based active uplink TCI state switch for single-DCI mTRP

##### 7.5.13.6.1	NR SA FR2 MAC-CE based active uplink TCI state switch for 2 known TCI states

Editor's note: This test case is incomplete. The following aspects are either missing or TBD:

- The setup under OTA test parameters table may require update

7.5.13.6.1.1	Test purpose

The purpose of this test is to verify fulfilment of the uplink TCI switch delay requirement defined in TS 38.133 [6] clause 8.23.3 by a UE capable of beam correspondence without the need for UL beam sweeping. The test scenario comprises one PCell (Cell 1) with two TRPs.

7.5.13.6.1.2	Test applicability

This test applies to all types of NR UE release 18 onwards and supporting unified TCI state operation for single-DCI based intra-cell multi-TRP with multiple activated TCI states.

7.5.13.6.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.13.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.13.6.1.

7.5.13.6.1.4	Test description

There is one NR PCell (Cell 1) configured in this test. This test consists of two successive time periods, with time duration of T1 and T2 respectively.

7.5.13.6.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.13.6.1.4.1-1.

Table 7.5.13.6.1.4.1-1: Supported test configurations for NR SA MAC-CE based active uplink TCI state switch for two known TCI states

| Config | Description |
| --- | --- |
| 7.5.13.6.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 7.5.13.6.1.4.1-2.

Table 7.5.13.6.1.4.1-2: Initial conditions for NR SA MAC-CE based active uplink TCI state switch for two known TCI states

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.3-1 and TS 38.508-1 [14] clause 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.13.6.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 7.5.13.6.1.4.1-3.

2.	Message contents are defined in clause 7.5.13.6.1.4.3.

3.	There is one NR cell specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to clause C.1.1 and C.1.2 for this test.

Table 7.5.13.6.1.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| L1-RSRP reporting period | slot | 160 | Periodic L1-RSRP reporting configured |
| L1-RSRP measured RS |  | SSB0, SSB1, SSB2, SSB3 | L1-RSRP measurements of SSB0, SSB1, SSB2 and SSB3. |
| Number of reported RS |  | 3 | L1-RSRP reporting of measurements on SSB0, SSB1, SSB2, and SSB3. |
| T1 | s | 0.2 |  |
| T2 | s | 2 |  |

7.5.13.6.1.4.2	Test procedure

During the test PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE would have ACK/NACK sending. Prior to the start of the time duration T1, the UE shall be fully synchronized to NR PCell. The UE shall be configured with 4 different uplink TCI states for PCell: Uplink TCI state 0 (associated with SSB0),  uplink TCI state 1 (associated with SSB1),  Uplink TCI state 2 (associated with SSB2), and Uplink TCI state 3 (associated with SSB3) in Cell 1 before starting the test. Uplink TCI state 0 is indicated as the active uplink TCI-state and the TCI state 1, TCI state 2, TCI state 3 is not in the active TCI state list for PDSCH/PDCCH. The UE shall be configured with a unified DL TCI state 0, and SSB0 is configured as QCL source for the TCI state. At the start of test UE is connected to DL TCI state 0 and UL TCI state 0, and UL TCI state 1.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.4.

2.	Set the parameters according to T1 in Table 7.5.13.6.1.5-2. Propagation conditions are set according to clause C.2.2. T1 starts. During T1 only SSB0, SSB1 to which uplink TCI state 0, TCI state 1 is QCL'd is transmitted.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.5.13.6.1.5-2. T2 starts. At the beginning of T2, the SSB2 corresponding to uplink TCI state 2, and the SSB3 corresponding to uplink TCI state 3 starts transmitting.

4.	The SS transmits an RRCReconfiguration message to configure periodic L1-RSRP reporting.

5.	The UE transmits an RRCReconfigurationComplete message.

6.	The SS sends a MAC-CE to indicate switch to uplink TCI-state 2 and to uplink TCI-state 3 in slot n which is within 1280ms of UE providing L1-RSRP report with results for SSB0, SSB1, SSB2, and SSB3.

7.	If the SS:

a)	Receives ACK/NACK on each UL transmission occasion scheduled on uplink TCI state 0 and uplink TCI state 1 until slot n+THARQ+3Nslotsubframe,µ, and

b)	Receives ACK/NACK on each UL transmission occasion scheduled on uplink TCI state 2 and uplink TCI state 3 after slot n+ THARQ +3Nslotsubframe,µ + max{NM1* (Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms), NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS2 + 2ms)} / NR slot length. NM1 is equal to 1. NM2 is equal to 1.

the number of successful tests is increased by one, otherwise the number of failed tests is increased by one.

8.	When T2 expires the SS shall sends a MAC-CE to indicate switch to uplink TCI state 1 and uplink TCI state 2.

9.	Wait 1s for the UE to switch to uplink TCI state 1 and uplink TCI state 2. If the SS receives ACK/NACK on each UL transmission occasion scheduled on uplink TCI state 0 continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.4.

11.	Repeat steps 2-10 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.13.6.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.5.13.6.1.4.3-1: Common Exception messages for NR SA MAC-CE based active uplink TCI state switch for two known TCI states

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with condition SSB |

Table 7.5.13.6.1.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| mimoParam-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| additionalPCI-ToAddModList-r17 | Not present |  |  |
| additionalPCI-ToReleaseList-r17 | Not present |  |  |
| unifiedTCI-StateType-r17 | separate |  |  |
| uplink-PowerControlToAddModList-r17 SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControl-r17 { | 1 entry |  |  |
| uplink-powerControl-r17[1] SEQUENCE { |  | Entry 1 |  |
| ul-powercontrolId-r17 | 1 |  |  |
| p0AlphaSetforPUSCH-r17 SEQUENCE { |  |  |  |
| p0-r17 | 0 |  |  |
| alpha-r17 | Alpha08 |  |  |
| closedLoopIndex-r17 | i0 |  |  |
| } |  |  |  |
| p0AlphaSetforPUCCH-r17 SEQUENCE { |  |  |  |
| p0-r17 | 0 |  |  |
| alpha-r17 | Alpha08 |  |  |
| closedLoopIndex-r17 | i0 |  |  |
| } |  |  |  |
| p0AlphaSetforSRS-r17 SEQUENCE { |  |  |  |
| p0-r17 | 0 |  |  |
| alpha-r17 | Alpha08 |  |  |
| closedLoopIndex-r17 | i0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| uplink-PowerControlToReleaseList-r17 | Not present |  |  |
| sfnSchemePDCCH-r17 | Not present |  |  |
| sfnSchemePDSCH-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.5.13.6.1.4.3-3: BWP-UplinkDedicated

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| ul-TCI-StateList-r17 CHOICE { |  |  |  |
| explicitlist SEQUENCE { |  |  |  |
| ul-TCI-ToAddModList-r17 SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF TCI-UL-State-r17 { |  |  |  |
| TCI-UL-State-r17 [1] SEQUENCE { |  |  |  |
| tci-UL-StateId-r17 | 0 |  |  |
| servingCellId-r17 | Not present |  |  |
| bwp-Id-r17 | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index of SSB #0 |  |  |
| } |  |  |  |
| additionalPCI-r17 | Not present |  |  |
| ul-powerControl-r17 | 1 |  |  |
| pathlossReferenceRS-Id-r17 | 0 |  |  |
| } |  |  |  |
| TCI-UL-State-r17 [2] SEQUENCE { |  |  |  |
| tci-UL-StateId-r17 | 1 |  |  |
| servingCellId-r17 | Not present |  |  |
| bwp-Id-r17 | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index of SSB #1 |  |  |
| } |  |  |  |
| additionalPCI-r17 | Not present |  |  |
| ul-powerControl-r17 | 1 |  |  |
| pathlossReferenceRS-Id-r17 | 0 |  |  |
| } |  |  |  |
| TCI-UL-State-r17 [3] SEQUENCE { |  |  |  |
| tci-UL-StateId-r17 | 2 |  |  |
| servingCellId-r17 | Not present |  |  |
| bwp-Id-r17 | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index of SSB #2 |  |  |
| } |  |  |  |
| additionalPCI-r17 | Not present |  |  |
| ul-powerControl-r17 | 1 |  |  |
| pathlossReferenceRS-Id-r17 | 0 |  |  |
| } |  |  |  |
| TCI-UL-State-r17 [4] SEQUENCE { |  |  |  |
| tci-UL-StateId-r17 | 3 |  |  |
| servingCellId-r17 | Not present |  |  |
| bwp-Id-r17 | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index of SSB #3 |  |  |
| } |  |  |  |
| additionalPCI-r17 | Not present |  |  |
| ul-powerControl-r17 | 1 |  |  |
| pathlossReferenceRS-Id-r17 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-TCI-ToReleaseList-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-powerControl-r17 | Not present |  |  |
| pucch-ConfigurationListMulticast1-r17 | Not present |  |  |
| pucch-ConfigurationListMulticast2-r17 | Not present |  |  |
| pathlossReferenceRSToAddModList-r17 SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17 { |  |  |  |
| PathlossReferenceRS-r17 [1] SEQUENCE { |  |  |  |
| pathlossReferenceRS-Id-r17 | 0 |  |  |
| referenceSignal-r17 CHOICE { |  |  |  |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | Resource #4 in TRS.2.1 |  |
| } |  |  |  |
| additionalPCI-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| pathlossReferenceRSToReleaseList-r17 | Not present |  |  |
| } |  |  |  |

7.5.13.6.1.5	Test Requirement

Tables 7.5.13.6.1.4.1-3, 7.5.13.6.1.5-1 and 7.5.13.6.1.5-2 define the primary level settings including test tolerances for NR SA MAC-CE based active TCI state switch for two known TCI states.

During T2, the UE shall send L1-RSRP report with results for SSB0 and SSB1. After receiving MAC-CE command in slot n, the UE shall:

-	Continue transmitting using UL TCI state 0 and UL TCI state 1 up to and including slot n + THARQ + 3Nslotsubframe,µ

-	Start transmitting using UL TCI state 2 and UL TCI state 3, from slot n + THARQ + 3Nslotsubframe,µ + max{Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms, Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS2 + 2ms} / NR slot length and onwards.

The rate of correct events observed during repeated tests shall be at least 90%.

Table 7.5.13.6.1.5-1: OTA related test parameter for NR SA MAC-CE based active uplink TCI state switch two known TCI states

| Parameter | Unit | Cell 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  | SSB2 |  | SSB3 |  |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  | Setup 7 according to clause A.9.7 |  |  |  |  |  |  |  |
|  |  | AoA1 |  | AoA2 |  | AoA1 |  | AoA3 |  |
| Assumption for UE beams Note 6 |  | Rough |  |  |  |  |  |  |  |
| NocNote 1 | dBm/15 kHz | -92.1 |  |  |  |  |  |  |  |
| NocNote 1 | dBm/SCS | -86.1 |  |  |  |  |  |  |  |
| Ês/Noc | dB | 3.4 |  | -infinity | 3.4 | -infinity | 3.4 | -infinity | 3.4 |
| SS-RSRP Note 2 | dBm/120 kHz Note3 | -82.7 |  | -infinity | -82.7 | -infinity | -82.7 | -infinity | -82.7 |
| IoNote2 | dBm/95.04 MHz Note4 | -56.47 |  | -61.51 | -56.47 | -61.51 | -56.47 | -61.51 | -56.47 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone. Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |

Table 7.5.13.6.1.5-2: NR Cell specific test parameters for NR SA MAC-CE based active uplink TCI state switch for two known TCI states

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Data RBs allocated |  | 24 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.5 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| DL TCI State |  | DLorJoint TCI.State.0 |
| PL-RS Configuration for CSI-RS#0 |  | Resource #4 in TRS.2.1 TDD for UL TCI.State.2 |
| PL-RS Configuration for CSI-RS#1 |  | Resource #4 in TRS.2.2 TDD for UL TCI.State.3 |
| PL-RS Configuration for CSI-RS#2 |  | Resource #4 in TRS.2.4 TDD for UL TCI.State.4 |
| PL-RS Configuration for CSI-RS#3 |  | Resource #4 in TRS.2.5 TDD for UL TCI.State.5 |
| UL TCI State-0 Configuration |  | UL TCI.State.2 |
| UL TCI State-1 Configuration |  | UL TCI.State.3 |
| UL TCI State-2 Configuration |  | UL TCI.State.4 |
| UL TCI State-3 Configuration |  | UL TCI.State.5 |
| reportConfigType |  | ssb-Index-RSRP |
| reportConfigType |  | periodic |
| timeRestrictionForChannelMeasurements |  | configured |
| TRS Configuration |  | TRS.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote 1 |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

-------- End of change 1--------