3GPP TSG-SA WG2 Meeting #165	S2-2410791

# Hyderabad, India, October 14 - 18, 2024	(was S2-2410375)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1408 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix the descriptions on QoS information for PDU Set QoS Parameters AF provisions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is unclear what AF provides QoS information to the network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ‘QoS parameters/QoS Reference’ is replaced with ‘individual QoS parameters/QoS Reference’ as their intention. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoS information provided from AF is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** 1st Changes **********

##### 6.1.3.27.3	Support for delivery of multi-modal services

Enablers to support interactive media services that require high data rate and low latency communication, e.g. cloud gaming, AR/VR/XR services and tactile/multi-modal communication services are defined in TS 23.501 [2] clause 5.37.2.

For the delivery of multi-modal services, the AF may request to the NEF multiple data flows for a single UE or for multiple UEs via multiple PDU Sessions and separate request(s) per PDU Session, to be setup with specific QoS requirements, as described in clause 6.1.3.22. Following additional attributes are supported where the AF provides specific information for multi-modal service:

-	Multi-modal Service ID: an identifier that refers to the multi-modal communication service. Data flows belonging to the same multi-modal service share the same Multi-modal Service ID.

Multi-modal Service Requirements consisting of the following existing attributes are supported for the AF to provide specific information for a data flow of the multi-modal service and the AF can provide them multiple times, i.e. once per data flow:

-	QoS monitoring requirements for each data flow.

-	QoS information and service requirements for each data flow, that consist of: Flow description information of the data flow and individual QoS parameters/QoS Reference as described in clause 6.1.3.22.

NOTE:	The attributes for multiple data flows can be provided via a single or multiple AF requests.

The request to the PCF includes Multi-modal Service ID as well as service requirements for each data flow that belongs to the multi-modal service. The PCF determines whether the request from the NEF or AF is authorized, derives the required QoS parameters based on the information provided and provisions in the SMF the PCC rules with updated policy control information for the affected PDU Session.

The AF may provide QoS monitoring requirements for each data flow associated with the same Multi-modal Service ID. If the PCF has received the QoS monitoring requirements from the AF, the PCF generates the QoS monitoring policy for the PCC rule corresponding to the data flow.

When PCF receives further AF request with the same Multi-modal Service ID and PCF authorization fails, the PCF rejects the AF rejects. Application may decide how to deal with the data flows with already authorized AF request with the same Multi-modal Service ID (e.g. stop them, adjust the AF request).

********** End of Changes **********