3GPP TSG SA Meeting #106                                                                                                           SP-241679

# Madrid, Spain, December 10 – 13, 2024

# Source:	Ericsson

# Title:	New WID on Study on Improved ePDG EPS/5GS Interworking

# Document for:	Information

# Agenda Item:	22.2.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Improved ePDG EPS/5GS Interworking

Acronym: 	FS_IePI

Unique identifier:		TBD

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X |  | X |  | X |
| Don't know |  | X |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 5GS_Ph1 | Stage 2 of 5G System - Phase 1 | 5GS_Ph1 defined the current architecture for interworking between ePDG/EPC and 5GS |

# 3	Justification

The ePDG was defined as part of the EPS architecture and is widely deployed by operators, primarily to support Voice over Wi-Fi (VoWiFi). Therefore, the industry would highly benefit from an improved and more optimized interworking solution for ePDG in 5GS.

The interworking architecture between ePDG/EPC and 5GS was defined in rel-15 where EPC interfaces (Diameter-based SWm, SWd, SWx) and network entities (3GPP AAA Server (and 3GPP AAA proxy for roaming) and HSS) are used to connect the ePDG to an EPC/5GS interworking architecture.

The current architecture with SWm/SWx requires the operator to maintain a 3GPP AAA Server (and a 3GPP AAA proxy for roaming scenarios with SWd) in order to support ePDG use cases with an interworking core network. The IWK architecture has already been improved in previous releases to reduce the need for a 3GPP AAA Server and move to service based interfaces:

- 	The Diameter-based S6b interface (between PGW-C+SMF and 3GPP AAA Server) can be avoided by using the N10 interface (SBI between PGW-C+SMF and UDM) instead (see TS 23.501, clause 4.11.4.3.6)

- 	The architecture for Non-Seamless WLAN Offload (NSWO) has been modernized to support 5G security and work without a 3GPP AAA Server (see e.g. TS 23.502, clause 4.2.5, and TS 33.501, Annex S)

The SWm/SWd/SWx interfaces for ePDG are the last remaining interfaces requiring a 3GPP AAA Server/proxy to support ePDG/EPC interworking with 5GS.

From an SA2 perspective, it is assumed that legacy ePDG authentication method applies, i.e. with no UE impacts. This allows the modernized architecture to be used to support also legacy UEs.

By allowing an option where ePDG can connect to a EPS/5GS IWK architecture without Diameter interfaces and without 3GPP AAA Server allows operators to continue to support IMS voice over Wi-Fi with reduced OPEX and CAPEX.

# 4	Objective

The aim of this study work is to investigate and identify potential architecture and system level enhancements for EPS/5GS interworking system to support non-3GPP access via ePDG without the need for 3GPP AAA Server or Diameter based protocols. Specifically, the objectives include:

- 	WT#1:	Architecture impacts to support ePDG connected to AUSF, UDM and NRF, without intermediate 3GPP AAA Server.

NOTE 1: 	It is assumed that no new NF is to be defined as part of the study.

- WT#2: 	Impacts to procedures for Attach and PDN Connection Establishment via ePDG when interfacing AUSF and UDM.

NOTE 2: 	Security aspects are to be addressed by SA3.

- WT#3: 	Potential impacts to UDM subscription data to support access via ePDG.

NOTE 3: 	It is assumed that legacy ePDG authentication method applies, i.e. with no UE impacts. This is however to be confirmed by SA3.

NOTE 4: 	The study scope does not include a replacement of the S2b interface between ePDG and PGW-C+SMF.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 0.5 | 0.5 | No | WT#1 is self-contained |
| WT#2 | 1 | 1 | No | WT#1 |
| WT#3 | 0.5 | 0.5 | No | WT#1 |

Total TU estimates for the study phase: 2

Total TU estimates for the normative phase: 2

Total TU estimates: 4

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.XXX | Study on improved ePDG EPS/5GS interworking | TBD | TBD | TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

{Secondary responsible Working Group(s) are possible. In this case, list them here}

# 8	Aspects that involve other WGs

SA3 shall provide feedback regarding the security aspects of the solutions.

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

Ericsson