3GPP TSG-SA3 Meeting #119	S3-244905

Orlando, US, 11 -15 November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0222 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to AAnF response without UE Identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This proposal is to correct AAnF response without UE Identity. As description in clause 6.2.2, TS 33.535, In some scenarios, anonymous user access to the AF is desirable (e.g., UE identification is not required at the AF). For allowing such anonymous user access to the AF, the procedure detailed in clause 6.2.1 of the present document is used with the following changes: - in step 2, instead of Naanf_AKMA_ApplicationKey_Get request, Naanf_AKMA_ApplicationKey_AnonUser_Get request is used by the AF; and - in step 6, the AAnF sends Naanf_AKMA_ApplicationKey_AnonUser_Get response to the AF with KAF and the KAF expiration time. The AAnF shall store the KAF expiration time as part of UE’s AKMA context. The A-KID functions as a temporary user identifier. In clause 6.2.1, the AAnF should send Naanf_AKMA_ApplicationKey_AnonUser_Get response to the AF in step8, not in step6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to correct AAnF response without UE Identity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for AAnF response without UE Identity is mistaken. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of Change ***

6.2.2	AAnF response without UE Identity

In some scenarios, anonymous user access to the AF is desirable (e.g., UE identification is not required at the AF). For allowing such anonymous user access to the AF, the procedure detailed in clause 6.2.1 of the present document is used with the following changes:

-	in step 2, instead of Naanf_AKMA_ApplicationKey_Get request,  Naanf_AKMA_ApplicationKey_AnonUser_Get request is used by the AF; and

-	in step 8, the AAnF sends Naanf_AKMA_ApplicationKey_AnonUser_Get response to the AF with KAF and the KAF expiration time. The AAnF shall store the KAF expiration time as part of UE’s AKMA context.

The A-KID functions as a temporary user identifier.

*** End of Change ***