3GPP TSG-SA WG1 Meeting #108 	S1-244038

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Source:	IPLOOK

pCR Title:	Use case on sensing for fiber-optic vibration service over satellite

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.2

Document for:	Approval

Contact:	Weizhi Wu  (wu.weizhi@iplook.com)

Abstract:

1. Introduction

Integrated Sensing and communications is one of the key areas according to the usage scenarios of IMT-2030 in ITU-R M.2160. This pCR introduces a new use case on sensing for fiber-optic vibration service over satellite.

2. Reason for Change

To provide a new use case on sensing for fiber-optic vibration service over satellite.

3. Conclusions

The vibration sensor send the vibration data to the 5G operator over satellite. The 5G core network then can quickly locate and analyze the causes of the vibration.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

5.x	Use case on sensing for fiber-optic vibration service over satellite

5.x.1	Description

The disconnection of fiber-optic caused by sabotage or accidental operations will interrupt communication services and cause huge economic losses. This use case describes a scenario where underground fiber-optic is equipped with vibration sensors, which can collect and transmit vibration data and determine the location and type of vibration area through analysis. Fiber-optic vibration monitoring technology is widely used in various scenarios, such as oil and gas pipeline monitoring, fence intrusion detection, etc. By identifying different vibration behaviors, the monitoring and alarm of specific events can be realized, such as construction excavation, vehicle passing, machine operation, etc.

Figure 1: An example of sensing operation of vibration sensors

5.x.2	Pre-conditions

Each area is equipped with one or more fiber-optic vibration sensor.

The satellite provides coverage for ground fiber-optics.

For operator A, the sensing data of fiber-optics from different locations can be obtained through satellite.

The core network of operator A analyzes the sensing data and feeds the results to the fiber-optic operator.

5.x.3	Service Flows

- Each area covered by a satellite beam is equipped with multiple vibration sensors that are connected to the operator's 5G system via the satellite, so that the sensor can send the vibration sensing data through the satellite to the 5G system.

- When the vibration occurs, the sensor data is transmitted to the 5G core network through the satellite. The 5G core network analyzes and processes the sensing results, and the operator informs the local fiber maintenance worker Bob. According to the sensing analysis results, Bob knows the cause and location of the vibration.

- If the analysis results in storms, branches, and other occasional vibration events without threat, no special treatment is required. However, if it is caused by malicious damage or mechanical operation, Bob needs to go to the site as soon as possible to stop the fiber damage.

5.x.4	Post-conditions

By analyzing the causes of the fiber vibration, send a warning to local maintenance personnel to prevent the fiber from being damaged in time.

5.x.5	Existing features partly or fully covering the use case functionality

None.

5.x.6	Potential New Requirements needed to support the use case

[PR.5.x.6-1] The system shall provide emergency warning broadcast services over satellite.

[PR.5.x.6-2] The system shall provide pre-scheduled emergency uplink services over satellite.

[PR.5.x.6-1] Subject to operator policy, the 5G system shall enable the core network to collect and aggregate 3GPP sensing data data from RAN.

[PR.5.x.6-1] Subject to operator policy, the 5G system shall enable the core network to expose a suitable API to provide the information regarding sensing results to authorized third parties.

* * * End of Change * * * *