3GPP TSG-SA WG1 Meeting #108 	S1-244041

Orlando, Florida, USA, 18-22 November 2024

Source:	SK Telecom

pCR Title:	Use case on Interworking and simplification using Dynamic CRUD controls

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.1

Document for:	Approval

Contact:	SeongJun Lee <seoul.lee@sk.com>

DongJin Lee <dongjin@sk.com>

Abstract: This document provides a new use case on Interworking and simplification using Dynamic CRUD controls .

1. Introduction

This contribution proposes a new use case on Interworking and simplification using Dynamic CRUD controls .

2. Reason for Change

TR 22.870 v0.0.0 does not yet contain any use case that covers Interworking and simplification using Dynamic CRUD controls.

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.0.0.

# *** First Changes ***

## 5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

## 5.x	Use case on Interworking and simplification using Dynamic CRUD controls

### 5.x.1	Description

As the network evolves and continues to support interworking with previous generations, the physical and logical functional fragmentation of commercially operated networks has intensified. With CapEx/OpEx considerations, the complexity of the structure will further escalate as new networks require continuous support for legacy systems with different interworking scenarios. Therefore, functional enhancements are essential to mitigate network complexity.

A new network should be able to take individual legacy system’s function dynamically. For instance, a support of Dynamic create, retrieve, update, delete (CRUD) control for individual NF’s sub-functionalities during the service procedures can be achieved for enhanced interworking. For example, if a specific service is deemed unused, it can be updated/removed and reduce its own load (e.g., energy consumption) automatically.

### 5.x.2	Pre-conditions

There is a Network A that supports call processing. The operator B, who operates Network A, provides services for various purposes.

### 5.x.3	Service Flows

- When operating the network, it is possible to continuously provide existing services.

- A request for a new service is made to the existing network system.

- The operating network system requests new functionalities to provide the requested service.

- The existing system can immediately support the requested service by providing additional functionalities.

- After a certain period, if a specific service is deemed unused, the network system can reduce its own load (e.g., power consumption) by automatically deleting the specific service

### 5.x.4	Post-conditions

Providing functional scalability to the network system can significantly reduce the time-to-market for service deployment. Dynamic reduction of unused operations maximizes network efficiency.

### 5.x.5	Existing features partly or fully covering the use case functionality

Currently, dynamic CRUD control is not provided.

### 5.x.6	Potential New Requirements needed to support the use case

### [PR.5.x.6-1] The 6G network shall support Dynamic CRUD control for individual NF’s sub-functionalities during the service procedures.

# *** End of Changes ***