3GPP TSG-SA WG1 Meeting #108 	S1-244042

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Use case on big events assurance

Agenda Item:	8.1.7

Source:	ZTE

Contact:	Chen Lin   chen.lin23@zte.com.cn

Abstract: <provide a short description of the content>

This contribution proposes a new use case about big events assurance, which uses AI agents and LLM in 6G network to integrate the communication knowledge, structured data and network atomic capabilities to achieve the big event assurances.

************************ New Use Case ***********************************

W.x	Use case on big events assurance

W.x.1	 Description

An artificial intelligence (AI) agent refers to a system that is capable of autonomously performing tasks on behalf of a user or another system by designing its workflow and utilizing available tools. In addition to the natural language processing, AI agent can perform decision-making, problem-solving, and executing actions. AI agent can be used in 6G network to solve complex tasks such as big event assurance.

The big events, such as Olympic games, concerts, marathon, etc., usually poses stringent requirements for network operations. Large concurrent users, big volume data transmission and frequent real-time operations are assumed. With Asian games in 2022 as an example, there are 48 matches and 1.5million users over 16 days. During these events, full network coverage, good user experience, and zero complaints are expected.

In order to alleviate the burden of operational teams, AI agent and LLM(Large Language model) can be used to integrate the communication knowledge, structured data and network atomic capabilities to achieve the big event assurances. For example, operational team may raise the big event assurance task and requirements, the service types, bandwidth requirement, the number of users that needs to be supported, etc. Through the multi-modal human-computer interaction, user agent can understand user intent, decompose the user intent into several sub-tasks and calling service agents. The service agent makes action plan for efficient network configuration and resource allocation, personalized guarantee of user experience, real-time performance KPI monitoring, network coverage and interference evaluation, risk prediction and avoidance, alarm and root cause analysis, etc. Finally, resource agents may execute the specific action by using various tools, including built-in tools, API interfaces, models, etc., to complete given sub-tasks step by step.

Based on the assistance of LLM and multi-agent collaboration in 6G network operation, it is expected that the network operation efficiency is improved and the labor workload can be reduced.

W.x.2	 Pre-conditions

National game will be held with millions of athletes and visitors. Operator A is responsible for the communication service provision with 6G network around the national game area. Operator A has realized the LLM and AI agent mechanism for the efficient 6G network operation and maintenance. Various AI agents are created when the 6G network is deployed.

W.x.3	 Service Flows

- Before the national game, the operation team inputs the requirements that national game needs to be prepared, such as the service types, bandwidth requirement, and the number of users, etc.

- Upon receiving such input, the user intent is analyzed by AI agent and divided into several sub-tasks. The sub-tasks may cover efficient network configuration, personalized guarantee of user experience, real-time performance KPI monitoring, risk prediction and avoidance, etc.

- Each AI agents for sub-tasks will make detailed action plan for the sub-tasks. The action plan will be evaluated and tested until it works smoothly before the national game.

- During the game, AI agents involved in the big event assurance take actions to provide the autonomous network operation. Moreover, the operation team creates the monitoring dashboard so that the status of user experience and network performance can be continuously monitored.

- If pre-warning is received during the game, the corresponding AI agent can make optional solution based on pre-set intents. If this solution is evaluated as feasible, the network re-configuration will be carried out by AI agents automatically.

W.x.4	 Post-conditions

With the intelligent AI agents from all domains, the workload on complex routine network configuration and risk prediction/avoidance can be reduced.

W.x.5	 Existing features partly or fully covering the use case functionality

In TS 22.261, clause 6.51.2 on monitoring of network elements interactions in 5G include the following requirements:

NOTE 1: The monitoring system is outside of the 5G network. Both the monitoring system and the monitored network elements in the requirements below are fully under the control of the MNO.

The monitored network elements in the 5G network shall support the transmission of a secured copy of the outgoing and incoming signalling traffic to a monitoring system.

The 5G network shall enable the MNO to configure network monitoring, e.g., switching on/off per network element, selecting what type of elements and what type of signalling from these elements is the target for monitoring.

The 5G network shall allow the monitoring (i.e., transmit secured copies of outgoing and incoming signalling traffic) of a transmitting network element and, separately, the monitoring of the receiving network element while facilitating correlation of the information received from both network elements by the external system.

NOTE 2: These requirements do not imply/assume any design of the network elements. How the copies are created within the element, e.g., physical, virtual or container based, is expected to be implementation specific.

The signalling traffic shall be securely transmitted from the monitored network elements of the 5G network to the monitoring system while minimizing the degradation of network performance.

NOTE 3: The monitoring system is not integrated with the key management scheme of the 5G core.

The transmission of signalling traffic from the monitored network elements of the 5G network to the monitoring system shall be compliant with privacy legislation, data protection regulations and protection of confidential system internal data.

The transmission of signalling traffic from the monitored network elements of the 5G network to the monitoring system shall be limited regarding the number of file formats (e.g., JSON, PCAP, etc.) to assist with the ingestion of traffic feeds.

W.x.6  Potential New Requirements needed to support the use case

[PR.w.x.6-2]	Subject to operator’s policy, the 6G system shall be able to create, modify and delete an AI agent.

[PR.w.x.6-1]	Subject to operator’s policy, the 6G system shall be able to support AI agent to understand the user intent and divide the user intent into several sub-tasks.

[PR.w.x.6-3]	Subject to operator’s policy, the 6G system shall be able to support multiple AI agent collaboration to complete the sub-tasks automatically.

[PR.w.x.6-4]	Subject to operator’s policy, the 6G system shall be able to support the AI agent to monitor the real-time network performance and user experience.