3GPP TSG-SA WG1 Meeting #108 	S1-244085

Orlando, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	New use case on Immersive Service Discovery

Agenda Item:	8.1.4

Source:	Nokia

Contact:	Hideaki Takahashi; hideaki dot takahashi at nokia dot com

Abstract: This contribution proposes a new use case to discover service instances of immersive communication present in the network.

---------- Use Case template ----------

## 8.X	Use case on Immersive Service Discovery

### 8.X.1	Description

Immersive communication is an emerging paradigm to enable immersive user experience. For immersive communication services, multiple computing and data storage resources are deployed at various locations aimed at e.g., reduced latency, bandwidth efficiency, enhanced security, reliability, etc. XR is a prominent application where multiple compute resources are needed to perform edge/cloud rendering, i.e., rendering at the closer site to a user or a central site in the cloud. Each computing service instance, depending on its allocated compute resources, can handle a specific number of users. Whilst these computing resources can be scaled up to accommodate more users, the capacity of computing site ultimately limits the scalability.

The existing 5GS is capable of offering networking assistance to Edge Computing services, with the primary focus being fulfilling or optimizing a target network performance, e.g., latency. With this approach each user (UE, application client) is provided with compute resources on a site closest to the user. This may lead to situations where some compute sites get overloaded while others are underutilized. In order to optimally reserve or balance the use of computing resources across the compute sites while ensuring the application’s end-to-end performance and user experience, the compute site or application server selection should be performed based on both network and compute metrics, with any site or server meeting the application’s networking requirements considered a candidate. Figure x.1.1 illustrates a conceptual scenario that an optimal compute site is discovered by means of both network and compute metrics.

In a conventional edge computing scenario, a computing instance is always in-operation and running at least in one of the multiple computing sites. In the future, an advance scenario can be envisaged that a computing instance can be deployed and in-operation in an on-demand basis as per service request from applications or users. The motivations behind the on-demand service are as follows:

-	The application servers required by each individual application need not be pre-deployed on every computing site, but this can be done on-demand especially for the applications that are more rarely used.

--	 To save energy for running computing servers, sites, etc.

-

The on-demand instance deployment use case aims to select an optimal compute site, to deploy a computing service instance, that meets network and compute requirements of the application or the user. For instance, in AI co-inference scenarios, a UE could utilize a pruned model (i.e., a model with less parameters to reduce its size) locally that is suited for most frequent tasks, while the large model is stored to be deployed on-demand as per the request of the user. When the application starts requesting non-frequent tasks, the large model could be deployed at compute site that meets both network and compute requirements of the user. This approach would reduce resource consumption for both the UE (reduced local processing), the compute site (activated only when needed), and the application provider (cost incurred only during active use). Figure x.1.1 also covers the on-demand use case that some computing service instances are in-operation in the on-demand basis in addition to the pre-deployed service instances running all the time.

Figure x.1.1:	Computing Service Discovery by means of network and compute metrics

Potential sustainability impacts of the use case

|  | (the UN SDGs/GDC matching goals of each aspect within 3GPP context) | Potential benefits of the use case (added value) | Potential areas of attention of the use case (risks to be mitigated) |
| --- | --- | --- | --- |
| Environmental sustainability aspects (UN SDGs 12, 13, 14, 15 and indirectly 6, 7 & 11 UN GDC “Develop principles for environmental sustainability of digital technologies”) | Energy resources (UN SDG 7, 11, 12) | - Helping improving energy efficiency in 6G network by means of energy consumption as a metric of service discovery |  |
|  | Emissions (UN SDG 6, 7, 11, 12, 13, 14, 15) | - Enabling CO2 emission reduction due to improved energy efficiency. |  |

### 8.X.2	Pre-conditions

A Computing Service Provider (CoSP) is a provider to offer their computing resources, e.g. for immersive communications. MNO or the 3rdy party can be a CoSP. The CoSP deploys one or more application service instances (e.g., XR rendering, etc.) at various physical sites.

Subject to CoSP’s policy, split rendering service instances are always in-operation, or instead, not all of them are always in-operation and some of them are deployed only when a service is requested by users, i.e., on an on-demand basis.

In CoSP’s platform, while a split rendering service instance is active, its computing resource status (e.g., CPU/memory load of the computing service) is measured, which is to be used as a metric of computing performance. Additionally, computing resource status of the compute site (e.g., available CPU/memory of the compute site) is also measured, which is to be used for on-demand deployment. In case MNO plays a role of CoSP, the MNO network is also capable of measuring network performance (e.g., latency between a UE and an application service instance at a given compute site) when requested for a given UE.

A user subscribes application services (e.g., XR applications, etc.) provided by the MNO and/or an Application Service Provider (ASP) by leveraging CoSP’s application service instances at their compute sites.

### 8.X.3	Service Flows

1.	The user invokes the subscribed immersive application and request the service to MNO’s or ASP’s application host.

2.	The ASP or MNO requests the CoSP to provide the network metrics per split rendering service instance or site where the target immersive application can be provided for user’s UE requesting the application service.

3.	In case a slit rendering service instance or site is in-service, User’s immersive application or ASP collects from CoSP, the computing resource status of the split rendering service instance or the compute site as well as the network performance in relation to user’s UE. In case MNO plays a role of CoSP, the network performance is obtained from 6G network, whereas compute performance is obtained via a different layer than the one where 6G network encompasses.

5.	User’s immersive application or ASP selects an optimal split rendering service instance, taking into account the computing and network metrics for each application service instance or compute site obtained from CoSP.

6.	If the selected split rendering instance is provided on an on-demand basis, user’s application or ASP requests on-demand computing service provisioning.

### 8.X.4	Post-conditions

The immersive application user can enjoy optimal performance and user experience by means of the metrics of computing and network. In addition, the MNO and ASP can contribute to save energy consumption by means of the on-demand basis service provisioning and enhance their annal ESG report.

### 8.X.5	Existing features partly or fully covering the use case functionality

None.

### 8.X.6	Potential New Requirements needed to support the use case

[PR 8.x.6-1] Subject to operator’s policy and an authorized 3rd party and/or user consent, the 6G system shall be able to assist immersive applications in selecting a split rendering service instance or site which is optimal, including based on network and compute metrics, and application’s networking and compute requirements.

[PR 8.x.6-2] The 6G network shall be able to receive information about split rendering service instances and sites for which its selection assistance applies (e.g., identification, addressing and capability information of the split rendering service instances and sites).

NOTE:	The capability information of the split rendering service instances and sites include information on which metrics exposure or determination methods (e.g., measurement result of metrics) the split rendering service instances and compute sites support towards the 6G Network.

[PR 8.x.6-3] The 6G network shall be able to determine, e.g., by measurements using the measurement results of metrics supported by the split rendering service instances and sites, the network metrics (e.g., latency) between a UE and each potential split rendering service instance or site.

[PR 8.x.6-4] The 6G network shall be able to expose network metrics (e.g., latency) between a UE and each potential split rendering service instance or site to an authorized 3rd party.