3GPP TSG SA WG 1 Meeting #108 	S1-244087

Orlando, USA, 18-22 November 2024	(revision of S1-24xxxx)

Source:	NTT DOCOMO

pCR Title:	Pseudo-CR on Fault analysis through multi-orbit satellite access

Draft Spec:	3GPP TR 22.870 v0.0.0

Agenda item:	8.1.3

Document for:	Approval

Contact:	azem@docomolab-euro.com

Abstract: <provide a short description of the content>

1. Introduction

pCR on Fault Analysis and Reporting for multi-orbit satellite access.

2. Reason for Change

Define the requirements for 6G system to support fault analysis and reporting for multi-orbit satellite communication system.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.0.0.

* * * First Change * * * *

# 7	Ubiquitous Connectivity

Editor's Note: "Ubiquitous Connectivity" is intended to enhance connectivity to bridge the digital divide and enhance user experience, to address presently uncovered or scarcely covered areas, particularly rural, remote and sparsely populated areas, and indoor connectivity

## 7.x	Use Case x: Fault Analysis and Reporting for multi-orbit satellite access

### 7.x.1	Description

Multi-orbit satellite networks offer a wide range of services, utilising combined access from low-Earth orbit satellites and high-altitude satellites in medium-Earth and geostationary orbits. In scenarios when communication failure occurs, the available fault information at the UE and/or satellite can be forwarded to the satellite supporting the fault analysis capability, so that the fault information report can be provided to the MNO accordingly. The provided information can be analysed by the MNO afterwards to identify the cause of communication failure, which can be used to enhance the system performance and improve the user experience.

The fault analysis mechanism should be deployed on certain satellites (e.g. a GEO satellite), which will be utilized by other satellites through forwarding the available fault information (locally occurred or received from a UE) over ISLs. Hence, in multi-orbit satellite networks, communicating the fault information might follow multi-paths until it reaches the core network, e.g. fault information sent from UE to a satellite A that forward it to another satellite B supporting the fault analysis capability, as shown in figure 7.x.1-1.

Figure 7.x.1-1: Example of fault analysis through multi-orbit satellite access

### 7.x.2	Pre-conditions

- Operator A’s network supports satellite access and provide automated satellite fault analysis capabilities onboard on Sat B.

- A user subscribed to operator A’s network, using a device UE1 supports satellite access and the UE1 supports satellite fault information handling.

### 7.x.3	Service flows

1. UE1 connects to the operator A network via Sat A

2. in case of communication failure/error at UE1, the device stores this information locally

3. UE1 sends satellite fault information to Sat A

4. Sat A sends the received information to Sat B, which has the fault analysis capability and reporting the failure communication information to the core network

5. upon reception of the report at the core network from Sat B, the information can be used for further analysis and can be forwarded and used by a trusted third party or authorized users.

### 7.x.4	Post conditions

The operator A can optimize performance through monitoring and analysing the failure information.

### 7.x.5	Existing features partly or fully covering the use cases functionality

none.

### 7.x.6	Potential New Requirements needed to support the use case

[PR 7.x.6-001] 	Subject to operator’s policies, a 6G system with satellite access shall be able to support satellite fault analysis operations onboard on specific satellites for collecting failure communication information within the multi-orbit satellite network.

[PR 7.x.6-002] 	Subject to operator’s policies, a 6G system with satellite access supporting satellite fault analysis operations for multi-orbit satellite network, shall be able to support forwarding of the stored fault information from one satellite to another through ISLs.

* * * End of Change * * * *