3GPP TSG-SA WG1 Meeting #108	S1-244166

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Source:	KDDI, NTT DOCOMO

pCR Title:	Use case on mitigating IMS failure

Draft Spec:	TR 22.870

Agenda item:	8.1.1

Document for:	Approval

Contact:	Kenichi Yamamoto <kc-yamamoto@kddi-std.jp>
Yusuke Nakano <you-nakano@kddi.com>

Abstract: This document provides a new use case on mitigating IMS failure.

1. Introduction

This contribution proposes a new use case on mitigating IMS failure.

2. Reason for Change

TR 22.870 v0.0.0 does not yet contain any use case that covers mitigating IMS failure.

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.0.0.

# ***1st Changes ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[y]	3GPP TS 22.173: " Multimedia Telephony Service and supplementary services; Stage 1".

[z]	3GPP TS 23.380: "IMS Restoration Procedures".

# ***2nd Changes (All new texts)***

5.x	Use case on mitigating IMS failure

5.x.1	Description

When a country deploying the communication service is broken down due to IMS-related / IMS-affected failures, massive subscribers de-register network and then attempt to re-register to the network in cycles. Nowadays, the IMS have no response or return a response with error code which does not contain the appropriate reason for the failures. If the IMS and/or EPC/5GC (e.g. PGW/SMF, PCRF/PCF) providing MMTEL service is under failure condition, UEs may not be able to receive SIP messages and initiate re-attach or re-registration procedure to the EPC/5GC. Therefore, it’s difficult for UEs to activate appropriate error handling based on the response from the IMS. This leads the UEs to initiate re-attach or re-registration procedure, causing unexpected overload, accelerating the signalling storm in the network and affecting data services for the EPC/5GC dependent on UE’s implementation. It eventually takes a longer time to recover the communication services.

In this use case, it is assumed that the communication service with minimum interrupt can be provided in case of IMS-related / IMS-affected failures. It is highly expected that such mitigation is required not only for existing EPS/5GS but also for the future 6G system.

5.x.2	Pre-conditions

User A and User B subscribe the communication and the data services of Operator A.

Operator A’s network deploys a restoration mechanism for IMS failure.

5.x.3	Service Flows

- User A makes a phone call to user B, and starts to communicate with the user B.

- IMS-related / IMS-affected failures, such as failures of IMS function(s) or failures of 6G network function(s) providing the IMS service, hit the network of Operator A. As a result, User A cannot communicate with the user B.

- Operator A’s network initiates the restoration procedure for IMS failure, considering the prevention of unnecessary re-attach or re-registration by UEs.

- Operator A’s network provides an IMS status indicating the failure condition (e.g., system failures, network congestion) to the UEs.

- User A’s and User B’s UEs receive the IMS status and recognize that the IMS is under network congestion.

- User A’s and User B’s UEs do not initiate any signalling.

- Operator A’s network recovers from the failure and provides an IMS status indicating the return to normal operation to the UEs.

- User A’s and User B’s UEs receive the IMS status and recognize that the IMS has returned to normal operation.

- User A’s and User B’s UEs initiate signalling to Operator A’s network (e.g., re-attach or re-registration).

- User A makes a phone call to user B again and start to communicate.

5.x.4	Post-conditions

User A and User B are not heavily impacted and continue to consume the service from Operator A.

Operator A avoids a large-scale network failure, recovers the network in a short period.

5.x.5	Existing features partly or fully covering the use case functionality

The Minimization of Service Interruption requirements have been specified in clause 6.31 of TS 22.261 [x]. However, it focuses on enabling a UE of a given PLMN to obtain connectivity service from another PLMN in areas affected by a disaster condition.

The IMS Multimedia Telephony Service and supplementary services requirements have been specified in TS 22.173 [y]. However, it has not any consideration on IMS-related / IMS-affected failure.

The IMS restoration procedures for the P-CSCF/S-CSCF failure scenario have been specified in TS 23.380 [z]. However, it has not any consideration on the UE’s behaviours based on various IMS-related / IMS-affected failures and the failures of core network providing IMS service.

5.x.6	Potential New Requirements needed to support the use case

[PR.5.x.6-1] The 6G system shall support a mechanism to prevent a UE from invoking re-attach or re-registration in cycles, if the IMS or 6G network providing IMS service is under failure condition.

[PR.5.x.6-2] The 6G system shall support a mechanism to avoid unnecessary re-attach/re-registration to 6G network and minimize the load on 6G network if the IMS or 6G network providing IMS service is under failure condition.

[PR.5.x.6-3] The 6G system shall support a mechanism to provide appropriate error handling between IMS and UEs.

[PR.5.x.6-4] The 6G system shall be able to support mechanisms that enable the 6G network to provide an IMS status indicating failure conditions to UEs, with security consideration (e.g. preventing fraud from exploiting the status information) based on the operator’s policy.

# *** End of Changes ***