3GPP TSG-SA WG1 Meeting #108	S1-244167

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Source:	KDDI, NTT DOCOMO, TOYOTA MOTOR CORPORATION

pCR Title:	Use case on network congestion control

Draft Spec:	TR 22.870

Agenda item:	8.1.1

Document for:	Approval

Contact:	Kenichi Yamamoto <kc-yamamoto@kddi-std.jp>
Yusuke Nakano <you-nakano@kddi.com>

Abstract: This document provides a new use case on network congestion control

1. Introduction

This contribution proposes a new use case related to network congestion control.

2. Reason for Change

TR 22.870 v0.0.0 does not yet contain any use case that covers network congestion control.

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.0.0.

# ***1st Changes ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[y]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

# ***2nd Changes (All new texts)***

5.x	Use case on network congestion control

5.x.1	Description

For 5GS/EPS deployment, NAS level congestion control is achieved by providing a Back-off Timer and/or cause values to UEs. However, followings issues are identified:

- UEs need to be switched OFF/ON after receiving a Back-off Timer IE from a network (incl, IMS). So, the network using a Back off Timer does not solve the issue of the manual OFF/ON caused by the variety of UE implementation. If UEs receive a Back-off Timer IE, how the UEs apply to the Back-off Timer depend on vendor’s implementation. The network also needs to identify the UE's implemented release to assess their support of a Back off Timer.

- The flexibility allowed by 3GPP standards for vendors to decide on signalling behavior after receiving cause values can lead to inconsistent UE behavior, some of which a mobile operator does not expect.

For the 6G system, it would be beneficial to:

- Develop more robust and standardized congestion control mechanisms that account for the diverse range of devices and their capabilities.

- Implement more intelligent and adaptive network management techniques to handle congestion dynamically and efficiently.

5.x.2	Pre-conditions

Operator A and UE A deploys a mechanism for network congestion control.

UE A subscribes to Operator A.

5.x.3	Service Flows

- Service interruption such as system failures and network congestion hit the Operator A’s network.

- The Operator A’s network provides a network status indicating the failure condition (e.g., system failures, network congestion) to the UEs.

- UE A receives the network status and triggers configurations (e.g., maximum allowed number of signalling events, timing of signalling) based on the network status information.

- Operator A’s network recovers from the failure and provides a network status indicating the return to normal operation to the UEs.

- UE A receives the network status and triggers configurations (e.g., returning to normal mode) based on the network status information.

5.x.4	Post-conditions

User A is not heavily impacted and continue to consume the service from Operator A.

Operator A avoids a large-scale network failure, recovers the network in a short period.

5.x.5	Existing features partly or fully covering the use case functionality

General NAS level congestion control and the values for Back-off Timer and cause have been specified in TS 23.501 [x] and TS 24.501 [y], respectively. However, how the UEs apply to the Back-off Timer depend on vendor’s implementation.

5.x.6	Potential New Requirements needed to support the use case

[PR.5.x.6-1] The 6G system shall be able to support mechanisms for the 6G network to provide a network status information (e.g., network congestion) to UEs and to control the UEs effectively in terms of signalling frequency and timing of signalling based on the operator’s policy.

[PR.5.x.6-2] The 6G system shall make a UE trigger the configurations based on the network status information from 6G network, in accordance with the operator’s policy (e.g., maximum allowed number of signalling events, timing of signalling).

# *** End of Changes ***