3GPP TSG-SA WG1 Meeting #108 	S1-244193

Orlando, Florida, USA, 18 - 22 November 2024

Title:	Use case on low-altitude logistics via the UAVs supported by the NTN

Agenda Item:	8.1.3

Source:	ZTE Corporation

Contact:	    xu.ling@zte.com.cn, zheng.shuang@zte.com.cn

Abstract: This contribution describes a new use case about the low-altitude logistics delivery via the UAVs supported by the NTN communication, sensing and data processing.

* * * First Change * * * *

7.x	Use case on low-altitude logistics via the UAVs supported by NTN

7.x.1	Description

For remote regions with inconvenient transportation, complex terrain and rapidly changing weather, traditional logistics delivery methods struggle to meet the needs of local residents. By utilizing unmanned aerial vehicles (UAVs) and other low-altitude aircraft, the low-altitude logistics can overcome limitations posed by terrain and other factors to swiftly deliver the medicine, food, and emergency supplies to the residents live in such remote region, which exhibits its tremendous potential and advantages.

The development and operation of the low-altitude economy heavily rely on robust communication networks to ensure the safety and reliability of UAV systems. However, depending solely on traditional terrestrial networks (such as 4G/5G cellular networks) presents several critical challenges, for example:

- The density of ground base station deployments is limited, particularly in remote areas or challenging terrains such as mountains or forests. This can lead to insufficient or unstable signal coverage, limiting the expansion of the low-altitude economy into these regions.

- UAVs often operate in environments with high mobility and dynamic changes. Ground networks, which are primarily designed to serve relatively stationary or slowly moving objects, may struggle to maintain stable connections and experience higher data transmission delays.

- The terrestrial networks may reach their capacity limits when considering the large-scale and high-density traffic demands introduced by the low-altitude economy. Moreover, the flexibility and scalability of ground networks are limited, which can hinder their ability to respond to sudden spikes in emergency situations.

To address these challenges, the low-altitude economy requires reliance on Non-Terrestrial Networks (NTNs), such as Low Earth Orbit (LEO) satellite networks, High-Altitude Platform Stations (HAPS), and other space-based or non-conventional communication infrastructures. NTN can offer broader and more stable signal coverage, accommodate high-dynamics flight scenarios, and provide enhanced security and regulatory capabilities.

Additionally, in some challenging environment, such as steep mountains, deep valleys, and dense forests, it may bring significant difficulties for UAV fight. The rapidly changing weather conditions in remote mountainous regions, such as sudden strong winds, rain, or fog, further complicate UAVs’ operations. And the UAVs that flights over different areas in a remote region may be affected by different weather conditions as following Figure 1, e.g., some UAVs flight over the area 1 may be affected by the heavy rain, while the UAVs flights over the area 2 may be affected by the dense fog. Under such scenarios, the NTN can provide critical support through satellite-based communication, enabling real-time data exchange between UAVs and leveraging advanced sensing and processing capabilities to assist optimizing flight paths and ensure task operation.

Figure 7.x.1-1. Illustration of logistics delivery in remote mountain areas.

Following is an example of service flow to describe the low-altitude logistics delivery via the UAVs supported by the NTN networks .

7.x.2	Pre-conditions

To improve the delivery efficiency, the logistics company A bought a large number of UAVs for the delivery, and it has a dispatch center that is responsible for scheduling and optimizing the flying route of UAVs.

The UAVs have been configured with the camera and sensor, which can be used to sense and collect the environmental data.

The UAVs of logistics company A have been configured with modules that support communication via the NTN and have already requires the flight permission from local aviation authority.

The logistics company A has received many delivery orders in a remote region A  from different customers, and one order is from the customer Alice to deliver the medicine to the Jack’ home, who also lives in this remote region.

The operator B has deployed NTN network to provide the stable NTN signal coverage in this remote region A, and provide the ‘NTN communication service’ to the customers.

The NTN is equipped with the processing, sensing and AI network functions, which can sense and process the high-resolution image data, as well as performing the intelligent analysis of image data directly on the satellite.

The logistics company A has subscribed the ‘NTN communication service’ from operator B. And all UAVs have registered in the operator B.

7.x.3	Service Flows

1. The logistics company A inputs all delivery tasks of this remote region into the dispatch center, different delivery tasks have different targeted locations in this region. Then the dispatch center selects some UAVs and distributes a suitable initial route and delivery task for each UAV taking into account environmental conditions. And Alice’s order will be delivered by the UAV with number ‘1-1’.

- The UAVs take their corresponding expresses and begin the delivery according to the planned initial route.

- The NTN network provides coverage through satellites or high-altitude platform systems, ensuring network connectivity in this remote region. The communication between UAVs and between the UAVs and the dispatch center are supported by the NTN.

- During the delivery, UAVs that configured with camera and sensor will continuously sense and collect the condition of the airspace traffic, and the environmental condition.  The communication between UAVs and between UAVs and the dispatch center can be supported by the NTN to enable the direct data exchange. Multiple UAVs can share flight status, environmental sensing data, and mission progress information through NTN.

- During the flight of UAVs, the collected data of UAVs can also be transmitted to the NTN. And combined with the collected data that sensed by NTN itself, the NTN can process all collected data. According to the processed results,  the NTN detects that there exists a steep cliff ahead of UAV’1-1’, which may pose threat to UAV ‘1-1’ s flight. Thus the NTN send alert to the UAV ‘1-1’ with the processed result, the UAV ‘1-1’ autonomously adapts its flight path to avoid such cliff, ensuring safe navigation.

- Additionally, the NTN can also monitor the real-time weather condition. Through on board sensing and processing, the NTN gathers the weather data in area A and detects that there will have a strong winds ahead in area A. then NTN  immediately sends the weather status to the dispatch center. The dispatch center can dynamically assess whether to update the UAVs’ route that will flight over the area A based on the weather information.

- Finally, all the UAVs safely arrives at the targeted location as scheduled, and Jack gets the delivered medicine from UAV ‘1-1.

7.x.4	Post-conditions

The UAVs can dynamically update their flight routes and successfully finish their own delivery task in such harsh mountainous region through the support of NTN network.

The logistics company A can finish all delivery orders from all customers securely and efficiently.

7.x.5	Existing features partly or fully covering the use case functionality

None.

7.x.6	Potential New Requirements needed to support the use case

[PR.7.x.6-1] The 6G system with satellite access shall provide the communication service that can simultaneously satisfy the requirement of capacity, data rate, latency and reliability.

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Latency | reliability |
| --- | --- | --- | --- | --- | --- | --- |
| Low-altitude logistics in remote mountain area | [50]Mbps | [100]Mbps | TBD | TBD | <10ms | 99.9999% |

[PR.7.x.6-2] The 6G network shall be able to support detect and tracing sensing object (e.g. UAVs and its flying environment) while 6G sensing transmitter and/or 6G sensing receiver is/are on board satellite.

[PR.7.x.6-3] The 6G network shall be able to support sensing data processing on board satellite.

Note:	The sensing data can be 3GPP sensing data or non-3GPP sensing data.

[PR.7.x.6-4] Subject to operator’s policy and user’s consent, the 6G network shall be able to expose the processed results from the on board satellite to a trusted 3rd party.

* * * End of the First Change * * * *