3GPP TSG SA WG 1 Meeting #108 	S1-244236

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Reply to LS on further guidance to SA1 on Ambient IoT Service

Response to:	N/A

Release:	Rel-19

Study Item:	FS_Ambient_IoT_Sec

Source:	SA1

To:	SA, SA3

Cc:	SA2

Contact person:	Yang Xu

E-mail Address:	xuyang (at) oppo (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA1 would like to answer the questions in SP-241419/S3-243842 as below:

- Further guidance and clarification regarding stage 1 requirement for A-IoT taking aspects such as the business models and scope of the lifecycle management into account.

Are there any stage 1 requirements for AIoT on aspects of business model? Are there any stage 1 re-quirements on scope of lifecycle management?

Background:  SA3 was not able to reach a consensus on key issues related to authentication and communication security.  Though some progress has been made in the authentication key issue, further guidance is still needed. Some companies believe that in order to resolve this, more clarification related to business models of AIoT service, e.g., who owns the AIoT device, and how AIoT devices are provisioned or onboarded.  This has direct impact on security aspects related to the endpoint (e.g., an entity in the operator’s network or outside of operator’s network) of authentication and communication, i.e. whether authentication is between the AIoT device and another entity, protection of communication between the AIoT device and another entity.

Answer from SA1:

Regarding business model, SA1 assumes an operator-managed model where the network operator provides subscription and network resources, e.g. the operator network owns the AIoT device and responsible for the on-board provisioning. It is in the spirit of SA1 to continue the discussion on the operator-managed model and other possible models.

For the end-point, there are three communication modes defined in TS22.369 clause 4.4, namely Ambient IoT Direct Network Communication, Ambient IoT Indirect Network Communication, and Ambient IoT device to UE direct Communication. The communication modes implied which entity as the end-point.

2.     Consideration for capabilities of AIoT device that impact security as there are no service requirements in TS 22.369 linking these capabilities and security.

Are there any service requirements in TS 22.369 linking the limited capabilities of AIoT devices and security? If not, could SA1 provide such requirements?

Background:  SA3 was not able to reach consensus whether limited capabilities of AIoT devices should be considered when security key issues are analysed. Some companies believe the limited capabilities of the AIoT devices impact security requirements, security design and security solutions.  Some examples of such limited capabilities are related to the support of cipher algorithms as specified in TS 35.215 and TS 35.221, hash algorithms as specified in TS 33.501, and secure storage on the AIoT device.

Answer from SA1: TS 22.369 clause 4.2 has clearly described the characteristics of Ambient IoT as below.

Not all Ambient IoT devices are the same. Nevertheless, Ambient IoT devices have the following characteristics.

-	Energy harvesting. An Ambient IoT device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor). Energy is provided through harvesting of radio waves, light, motion, heat, or any other suitable power source. Energy harvesting can be continuous or incidental (e.g. based on the vibration that a vibration sensor has to report). It cannot be assumed that Ambient IoT devices always have enough power to initiate or receive communication.

-	Low complexity. Ambient IoT devices are expected to have lower complexity, smaller size, reduced capabilities and lower power consumption than previously defined 3GPP IoT devices (e.g. NB-IoT/eMTC devices). Low complexity of Ambient IoT devices is also expected to be reflected in efficient use of network resources. In general, Ambient IoT applications will deploy very large numbers of Ambient IoT devices.

-	Low data rates. Generally, Ambient IoT data transmissions contain only a low amount of data.

-	Life span. Ambient IoT devices can be maintenance free and can have long life span (e.g. more than 10 years). However, the life span of an Ambient IoT device can also be relatively short, e.g. when tracking a package through a logistics chain.

-	Communication characteristics. Ambient IoT devices can have a variety of communication characteristics, different from other IoT devices, based on how the Ambient IoT devices are powered by energy harvesting and whether / how the harvested energy can be stored. Ambient IoT devices will only be able to communicate when they have enough power. This can be an issue, especially when communication is initiated towards the Ambient IoT device, while it is not known whether the Ambient IoT device has enough power to receive this communication. For communications initiated by the Ambient IoT device, the Ambient IoT device cannot transmit data until it has harvested / stored enough energy. Some Ambient IoT devices can be powered on demand when they need to communicate. Additionally, some Ambient IoT devices will be able to communicate on a regular basis and have communication characteristics similar to regular IoT devices.

SA1 also would like to respond that the Ambient IoT requirements have been captured in TS22.369.

2	Actions

To SA, SA3

ACTION: SA1 kindly asks SA3 to continue the work on Ambient IoT security, taking into account the clarification in this LS response.

3	Dates of next TSG SA meetings

SA1#108 		November 18th – 22th 2024  	Orlando, US

SA1#109		February 17th – 21th 2025  	Athens, GR