3GPP TSG SA WG 1 Meeting #108 	S1-244292

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Use Case on multi-media services with deterministic experience via collaborative processing among UE-network-cloud

Agenda item:	8.1.4

Source:	Huawei, HiSilicon

Contact:	Alice Li, alice.li1@huawei.com

Abstract: This contribution provides a new use case on handling multi-media services processing with deterministic experience with collaborative processing among UE(s), network, and cloud (e.g. imaging processing, gaming service, XR rendering).

1. Introduction

“Immersive Communication” has been identified as one of key areas for 6G study. This contribution proposes a new use case on multi-media services with deterministic experience via collaborative processing among UE-network-cloud.

2. Reason for Change

Offloading rendering task to network/cloud is a promising trend to support advanced multi-media services in the future. However, the latency for offloading rendering task to network/cloud is quite difficult to be always guaranteed in anytime/anywhere due to the varied radio channel condition and network load/coverage. UE-Network-Cloud synergized multi-media operation is a collaboration mode which allows mobile device to dynamically offload part of rendering task to the cloud to improve the user experience.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1] 	https://en.wikipedia.org/wiki/Adreno

[x2] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/

[x3] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects)

[x4] 	https://phlearn.com/tutorial/how-many-exposures-hdr/

[x5]	 https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw

[x6]	https://www.dotphoton.com/products/jetraw-core

[x7] 	https://www.lambdatest.com/learning-hub/web-performance-testing

[x8] 	Static 3D triangle mesh compression overview, Marcos Aviles, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing

[x9] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

* * * Second Change (new clause) * * * *

## 8.x	Use case on multi-media services with deterministic experience via collaborative processing among UE-network-cloud

### 8.x.1	Description

Usually, the limited image rendering capability of mobile devices poses challenges for services that require high-quality image processing capability like high order super-resolution and de-noising algorithm, as well as high-quality rendering capability like ray tracing effect for gaming. As shown in figure 8.x.1-1, the GPU computing power for image rendering increasing in last ten years and the predication up to 2030 for mobile phone platform as well as the computing power required for advanced services has been summarized as below based on [x1] [x2] [x3]:

Figure 8.x.1-1: increasing GPU computing power for mobile platform

It can be observed that with the failure of Moore's law, the growth of mobile platform computing power gradually slows down. Offloading rendering task to network/cloud is a promising trend to support advanced multi-media services in the future. However, the latency for offloading rendering task to network/cloud is quite difficult to be always guaranteed in anytime/anywhere due to the varied radio channel condition and network load/coverage. UE-Network-Cloud synergized multi-media operation is a collaboration mode which allows mobile device to dynamically offload part of rendering task to the cloud to improve the user experience based on communication link status and maintains deterministic rendering task processing latency by local backup processing in case communication link is not good as shown in figure 8.x.1-2. There’s an expectation for 3GPP systems to consider such dynamic rendering offload framework to adapt to the synergized multi-media service processing trend.

Figure 8.x.1-2: UE-Network-Cloud synergized multi-media operation framework

The rendering task split function is a function provided by the UE operating system (OS) to map the rendering task request from the APP to local or remote rendering resources. Upon a rendering task is requested to be executed, the task split function can select the local and/or the remote rendering resources based on the relevant factors, such as required processing quality, radio link status, estimated latency, to ensure the deterministic user experiences for the requested multi-media service. The typical services which can be benefited from UE-Network-Cloud synergized multi-media operation include photo enhancement and ray tracing for gaming and so on.

The UE-Network-Cloud synergized operation raises several new issues, as follows:

-	uplink data rate: The rendering task offloading operation usually requires high uplink data rate to achieve higher service availability, considering the metadata to be uploaded for the rendering task is usually large with tight latency, e.g. 5Mbits/50ms, 150Mbits/1.5s (see detail in service flow part). However, the current network typically cannot guarantee sufficient uplink data rate especially at cell edge.

-	Real time communication link capability awareness: The rendering task split function might not know the exact communication link capability such as data rate, latency, and UE power consumption for data transmission, therefore it is hard for the UE to make the split decision.

-	Computing task level PDB requirement guarantee: The uplink metadata burst for a rendering task offloading arrives in a random way, and the latency requirement is intended for the whole data burst (i.e. the metadata of the task) rather than an individual packet. As shown in figure 8.x.1-3, the current GBR QoS flow framework guarantees the data rate in a fixed averaging window and fulfils latency in a per packet basis which is suitable for the periodic traffic. For the burst data carrying the metadata for rendering tasks, the latency and data rate should be guaranteed toward the whole data set rather than a specific packet. Therefore, the fixed averaging window as for periodic traffic is not suitable for bursty traffic. There is no suitable QoS type to guarantee the deterministic transmission latency for unpredictable uplink busty data traffic.

Figure 8.x.1-3: Burst latency requirement

### 8.x.2	Pre-conditions

1. Rendering resources are deployed in the cloud.

2. The UE operation system supports to map the rendering task from the APP to the local (in the mobile phone) and/or the remote (in the cloud) rendering resources.

### 8.x.3	Service Flows

Synergized photo enhancement case:

1.	User A is in a tourism attraction, taking photos with his mobile phone. Once he clicks the photo button, the phone senor produces raw data of the photo, including up to 6 frames (4K per frame) of raw files spanning 300ms for HDR reconstruction. Raw files retain the original uncompressed image data from the camera sensor, which is usually larger than compressed image format such as JPEG and PNG, and has unique advantages for imaging processing in practice.

NOTE 1: 2 to 10 frames are needed for HDR reconstruction based on [x4]; 6 frames are assumed in this use case.

2.	Upon reception of the rendering task request from camera APP for photo enhancement, the rendering task split function inside the UE OS determines whether or not to upload the raw data to the cloud for more powerful post-processing to enhance the photo quality, for which some factors would be considered such as potential delay and UE power consumption for data transmission.

3.	If the rendering task split function decides to upload the photo to the cloud, the raw data for the photo is compressed to around 150Mb (based on the assumption of 6 frames × 4K raw data and compressed ratio assumption in [x5] [x6]) and uploaded to the cloud within around 1.5s and downloaded the processed photo from the cloud. The UE can reduce the number of frames for uploading based on communication link capability. Therefore, the required uplink data rate in radio layer would be 150M/1.5s=100Mbps.

NOTE 2: E2E latency of 3s is assumed based on users' patience statistics as shown in [x7], where 1.5s is allocated for uploading to the cloud while 1.5s for processing in the cloud and downloading to the UE.

4.	If the rendering task split function decides not to upload the photo to the cloud, the local post-processing will be performed.

5.	When the user A views the photos, the downloaded photo or the local enhanced photo can be shown to the user.

Synergized gaming enhancement case:

1.	User A starts a gaming, the gaming APP produces the scene metadata, including 3D objects, lighting data, and materials, user actions etc. The typical data size after compression can be 5-20Mb (assuming middle to high complexity 3D model including 0.35 to 2 million vertexes (200 bits per vertex) 3D models and compression ratio assumption for 3D model in [x8]).

2.	Gaming APP submits the metadata to rendering task split function inside the UE OS for rendering, the rendering task split function can seek to utilize the more powerful computing in the cloud for higher quality such as ray tracing effect. The rendering task split function determines how to split the metadata to the cloud by considering some factors such as potential delay and UE power consumption for data transmission. For example, the UE selects to offload the slow changed scene background rendering to the cloud while performing rendering for fast moving objects using local rendering resources.

3.	The UE uploads the metadata allocated for the cloud rendering within 50ms [x9] and download the rendering result.

4.	The UE combines the local rendered result and cloud rendered result to the user A.

5.	Once the radio condition goes worse, the full local rendering can be activated to maintain user experience.

### 8.x.4	Post-conditions

User A continues enjoying high quality photo taking and high-quality gaming experience.

### 8.x.5	Existing features partly or fully covering the use case functionality

The split rendering architectures defined in TR 26.928 and TS 26.565, and the corresponding QoS requirements in TS 22.261, allow a UE to offload rendering requirement to the cloud for XR rendering.  These architectures assume that the metadata including 3D models of the gaming and APP logic is pre-installed in the cloud, therefore only user pose information is needed to be transferred in the uplink and there is no dynamic offload decision on whether to render a specific scene in the cloud or UE local resources (i.e. fully rely on the rendering result in the cloud). Therefore, the uplink requirement and dynamic offload decision requirement are not covered by the existing split rendering use cases and part of the downlink requirements on rendering result delivery are covered by the existing split rendering use cases.

It is worth noticing that Edge Enabler Client (EEC) is defined in TS 23.558, which provides supporting functions needed for Application Clients. There is some similarity between EEC and the “rendering task split function” described in this use case.

### 8.x.6	Potential New Requirements needed to support the use case

[PR 8.x.6-1] The 3GPP system shall support burst type QoS with the KPI requirements summarized below：

| Use cases | Burst size (note 1) | Max Allowed latency for a burst (note 2) | Service bit rate: user-experienced data rate | UE speed | Service Area |
| --- | --- | --- | --- | --- | --- |
| Synergized photo enhancement | UL: 150Mbits | UL: [1500ms] | UL: 100 Mbit/s | Stationary or Pedestrian | Countrywide |
| Synergized gaming enhancement | UL: 5-20Mbits | UL: [50ms] | UL: 100 – 400 Mbit/s | Stationary or Pedestrian | Countrywide |
| NOTE 1: assuming 6 x 4K raw pictures and compression ratio in [x6] for photo enhancement. Assuming 3D models including 0.35 to 2 million vertexes and compression ratio assumption for 3D model in [x8] for gaming enhancement. NOTE 2: 1500ms is derived from the E2E latency of 3s (based on users' patience statistics as shown in [x7]) and 1.5s for processing in the cloud and downloading. 50ms uplink latency is derived from [x9]. |  |  |  |  |  |

[PR 8.x.6-2] The 3GPP system shall enable a trusted third-party application in the UE to be aware of the real time and predicated communication link capability so that it can make decision on split rendering.