3GPP TSG-SA WG1 Meeting #108 	S1-244310

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Privacy and User Consent Considerations

Agenda Item:	8.1.1 System and Operation Aspects

Source:	Apple

Contact:	Mona Mustapha at apple dot com

Abstract: This contribution reviews a number of key privacy definitions (privacy, personal data and consent) and their implications to a 6G system. A gap analysis of 4G and 5G SA1 privacy and user consent requirements is provided, along with a number of proposed requirements for the 6G system.

The main observations on privacy and user consent:

1) Personal data includes information that can directly identify a user, and also any information that can distinguish one user from others.

2) Personal data includes information, that by itself cannot identify or distinguish a user, but when combined with other information, can identify or distinguish a user.

3) In the evolution of the 3GPP system over a number of releases, more features touch upon Personal data to provide new services.

The main proposals are summarised below:

A) Considering Privacy and User Consent as a service, the service requirements should be clearly defined.

B) Requirements related to Privacy and User Consent exist in 5G, but there are gaps identified

C) Privacy and User Consent requirements are proposed to have a holistic set of service requirements and also to fill in the identified gaps, including enabling user consent to be set by the user via the 6G system.

---------- Proposed changes ----------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	https://www.edps.europa.eu/data-protection/data-protection/glossary/p_en#privacy

[3]	https://www.edps.europa.eu/data-protection/data-protection/glossary/c_en#consent

[4]	Regulation (EU) 2016/679 of the European Parliament and of the Council of 27 April 2016, https://eur-lex.europa.eu/eli/reg/2016/679/oj

[5]	https://ico.org.uk/for-organisations/uk-gdpr-guidance-and-resources/personal-information-what-is-it/what-is-personal-information-a-guide/

[6]	3GPP TR 22.949: "Study on Generalised Privacy Capability"

[7]	3GPP TR 33.849: "Study on subscriber privacy impact in 3GPP"

[8]	3GPP TR 33.867: "Study on User Consent for 3GPP services"

[9]	3GPP TS 22.278 v9.0.0: "Service requirements for the Evolved Packet System (EPS)"

[10]	3GPP TS 22.261 v15.0.0: "Service requirements for the 5G system"

[11]	3GPP S1-110172: LS from SA1 to SA3 on Reply LS for MDT

[12]	3GPP TS 33.501: "Security architecture and procedures for 5G system"

[13]	OECD Privacy Principles https://legalinstruments.oecd.org/en/instruments/OECD-LEGAL-0188

[14]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services"

[15]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)"

---------- Use Case template ----------

## 5.1	Considerations on Privacy and User Consent

### 5.1.1	Description

Privacy is an integral part of the 3GPP system, with requirements on the protection for the communication content (e.g. content of emails, web pages), as well as any identities and UE location, from being exposed to unauthorised parties. This is considered as a 3GPP built-in feature by default, and solutions have been specified in previous releases that deal with the protection from unauthorised exposure of the communication content, user-related identities and UE location.

Key privacy definitions are reviewed below, highlighting aspects relevant to 3GPP systems, both in past releases and those foreseen for Rel-21 and beyond.

Definition of Privacy

Looking a definition of privacy from an EU perspective [2]:

Privacy is the ability of an individual to be left alone, out of public view, and in control of information about oneself. […] the ability to control the collection and sharing of information about oneself ("informational privacy”).

From this definition, the following interpretations could be derived from a 3GPP perspective:

1.	the ability of an individual to be left alone ("do not disturb"). This applies e.g. to ProSe discovery as defined in Rel-12 in TS 22.278

2.	the ability of an individual to be out of public view ("private"). This applies to confidentiality of Personal data

3.	the ability of an individual to be in control of collection and sharing of information about oneself ("control")

-	this applies by the user giving specific Consent for the intended usage; and

-	the network processing the user's Personal data according to the consented usage

The 3GPP requirements on privacy in releases leading up to 6G focused mainly on item number 2 above. In 5G, the 3GPP requirements started to cover to user consent, which covers item 3 to some degree.

Definition of Consent

Consent is a way to ask permission / authorisation from the user to collect / process information about the user, e.g. UE location, ID, and is defined in [3] as follows:

  - Consent refers to any freely given, specific and informed indication of the wishes of a data subject (identified or identifiable natural person), by which he/she agrees to personal data relating to him/her being processed

From the definition above, specific consent refers to a particular intended usage that the user has consented to e.g. to use UE-generated measurements to optimise the service provided to the UE. This means that consent needs to be linked to a particular usage, which needs to be indicated to the user.

Definition of Personal Data

As defined by EU GDPR [4]:

Personal data: any information relating to an identified or identifiable natural person ('data subject'); an identifiable natural person is one who can be identified, directly or indirectly, in particular by reference to an identifier such as a name, an identification number, location data, an online identifier or to one or more factors specific to the physical, physiological, genetic, mental, economic, cultural or social identity of that natural person

Furthermore, a person is identified or identifiable if this person can be distinguished from other persons [5].

Extracting further the interpretation of Personal data from the definitions above:

-	Identified directly [5] is where a network entity can identify (or distinguish) a person solely on the user-related information being processed by the network entity, e.g. a UE ID

-	Identified indirectly [5] is where a network entity can only identify (or distinguish) a person by a combination of the user-related information being processed with other user-related information held by the network.

NOTE:	SA1 in [6] previously used the EU GDPR definition, although a number of definitions of Personal Data exist, as captured in [7] Annex C.

Personal data in the 3GPP context covers the following types of information related to the user, that can be used to identify the user (or distinguish the user from other users), directly or indirectly:

-	Communication content: data sent or received by the user via the network, e.g. contents of a text message / voice call, data downloaded from a webpage

-	Non-communication content: data sent by the UE or derived by the network that is used to provide 3GPP services such as sensing, computing

-	User-related information: any information (excluding Communication content and Non-communication content) either sent by the UE, derived by the network, e.g. user location, UE ID

Evolution of SA1 privacy requirements in 4G and 5G

Based on the privacy definitions above, the following is observed:

1.	early 4G (Rel-8) and early 5G (Rel-15) SA1 Privacy requirements in [9] and [10]:

-	focused on avoiding access to or disclosure of Communication content and User-related information (i.e. user identities and user location) to unauthorised parties

-	no requirements enabling user control over their Personal data (i.e. User-related information)

2.	early 4G (Rel-8) and early 5G (Rel-15) SA1 requirements on Consent in [9] and [10]:

-	When 4G work started in Rel-8, there were no user consent requirements

-	in Rel-10 for MDT: SA1 agreed the need for user consent, but this was considered out of 3GPP scope [11]

-	in Rel-12 for ProSe: SA1 introduced the need for user consent in several ProSe functional requirements, which was to enable the user to agree to establish / move a traffic session, see TS 22.278 clause 7A

-	When 5G work started in Rel-15, user consent was first raised as part of 'Context-aware network' use case, enabling a broad set of User-related information (in addition to UE location and identities) to be used for network optimisation and efficient network resource utilisation.

-	Since Rel-15:

-	a number of features introduced in SA1, e.g. Ranging, Sensing, EnergyServ, AIML model transfer, have been added to the number of requirements involving user consent, as the potential collection and processing of a wider range of User-related information became involved

-	there are features that have enabled User-related information (e.g. UE location, UE ID) to be exposed by the network to authorised third parties via APIs

-	a requirement was introduced in Rel-18 enabling a UE to indicate consent for user data intended to be shared with a third party

3.	SA1 requirements can indicate that Consent is needed for a specific functionality, e.g. "Subject to user consent, the 5G system shall…", but there are no requirements defining user consent itself

Opportunity for the network operator

SA1 privacy requirements have focused mainly on avoiding unauthorised access or disclosure of Personal data to unauthorised parties. Privacy of Personal data can also be further protected by ensuring privacy principles in e.g. [13] are considered in the system design, for example, collection / processing of personal data for a specific purpose needs user consent, and the data collection is limited to and relevant for this specific purpose.

In the evolution of the 3GPP system functionality from 4G to 5G, user consent started to take more prominence in SA1 requirements, as more functionalities touch upon User-related information. A growing number of requirements introduced the need for user Consent, e.g. 16 instances in TS 22.261 v19.7.0 vs 1 instance in TS 22.261 v15.0.0.

As it is expected that potential 6G new services and / or functionalities will likely need access to more Non-communication content and User-related information, it is proposed that 3GPP take a more holistic approach where user privacy and consent is a service in its own right, and for service requirements for user privacy and consent to be defined in Stage 1.

By building out a more complete privacy-preserving system in 6G on top of what is defined in 5G, the added privacy protection and user consent capabilities could be considered much more than just a basic system function. This holistic privacy approach itself could be leveraged by network operators as a key service differentiator or value-add service for their customers, both in the consumer space and, more importantly, in the enterprise / verticals space, where control and usage of their data is one of the highest priorities.

### 5.1.2	Existing features partly or fully covering the use case functionality

SA1 studied privacy in Release 6 [6], SA3 also studied privacy and user consent in [7] in Release 14 and [8] in Release 17, where a number of privacy and user consent concepts were studied in more detail, and a number of normative security architecture requirements were defined in TS 33.501 Annex V [12]. SA2 introduced user consent management for Network Analytics in Rel-17 in TS 23.288 [14], and for Location Services in TS 23.273 [15].

### 5G Stage 1 requirements gap analysis: privacy

Requirements for privacy for 4G are defined in TS 22.278 [9]:

The Evolved Packet System shall provide several appropriate levels of user privacy including communication confidentiality, location privacy, and identity protection.

The privacy of the contents, origin, and destination of a particular communication shall be protected from disclosure to unauthorised parties.

The Evolved Packet System shall be able to hide the identities of users from unauthorised third parties.

It shall be possible to provide no disclosure, at any level of granularity, of location, location-related information, e.g. geographic and routing information, or information from which a user's location can be determined, to unauthorised parties, including another party on a communication.

In addition to the requirements above, requirements for privacy for 5G are defined in TS 22.261 [10]:

### The 5G system shall support network resource utilization efficiently and network optimization based on system information, including […]

### -	subject to user consent, enhanced traffic characteristic of UE (e.g. Mobility information (e.g. no mobility, nomadic, spatially restricted mobility, full mobility), location, sensor-level information (e.g. direction, speed, power status, display status, other sensor information installed in the UE), application-level information (e.g. foreground applications, running background application, and user settings).

### The 5G system shall support a secure mechanism to collect system information while ensuring end-user and application privacy (e.g., application level information is not to be related to an individual user identity or subscriber identity and UE information is not to be related to an individual subscriber identity).

### The requirements above mainly cover the privacy of the Communication content, there are no privacy requirements covering the data generated as a result of other Non-communication services.

### Requirements that cover privacy principles have not been clearly identified and defined (e.g. as those in [13] for example, data collection is relevant and limited to the given purpose) for the 3GPP system, although certain privacy principles are supported by certain features (e.g. obtaining user consent, identify purpose for data collection).

### 5G Stage 1 requirements gap analysis: user consent

A requirement for user consent for user data intended to be shared with a third party is defined in TS 22.261 [10] clause 6.10:

The 5G system shall be able to: […]

-	allow the UE to provide/revoke consent for information (e.g., location, presence) to be shared with the third-party.

### Apart from the requirement above, which relates to sharing data with a third party, there are no Stage 1 requirements to enable user consent to be set by the user. Currently, the setting of user consent (apart from data sharing with third party) is deemed out of 3GPP scope. Potentially, this involves the user contacting the operator to ask to enable / disable consent e.g. via operator-specific apps, operator user account portal / webpage, contacting operator call centre. The way user consent is obtained needs to be aligned, and as more features are touching personal data, obtaining consent from the user should be a capability of the 6G system.

### There are no Stage 1 requirements to propagate user consent revocation in the network, so if the user withdraws consent, the actions in the network are not clear. As a result, entities in the network do not know that consent was revoked, so they are not aware of the need to take corresponding action e.g. to stop collecting / using data from that user.

### The Stage 1 requirements related to user consent imply that the consent settings are defined on a per feature level in the user's subscription. This means that the user is unable to provide more granular consent for specific usage type within a feature.

### There are no Stage 1 requirements that enable user to understand the current status of user consent, i.e. whether user consent was provided or withdrawn. Furthermore, the user does not have any visibility of the collection or processing of their personal data.

### 5.1.3	Potential New Requirements needed to support the use case

Requirements for privacy

[PR 5.1.3-001] Subject to regulatory requirements or operator policy, the 6G system shall protect from unauthorised access and disclosure to unauthorised entities any information at any level of granularity that could be used to locate, identify or distinguish a UE or user, e.g.:

-	communication content;

-	non-communication service content (e.g. sensing, AI/ML, computing);

-	any user-related information associated with the content above, e.g. origin, and destination of the content; and

-	any other information (excluding communication content and non-communication content) either sent by the UE, derived by the network;

during the following stages:

-	the collection, storage, processing and transportation of the information above within the 6G system; and

-	the transportation / exposure of the information above from the 6G system to entities outside the 6G system

[PR 5.1.3-002] The 6G system shall support the following privacy principles (e.g. based on a selected set of privacy principles [13]):

-	Limit the collection of Personal data to what is needed for the specific purpose;

-	Obtain consent of the user to collect Personal data;

-	Specify the purpose(s) for which Personal data is collected to the user; and

-	Personal data collected needs to be relevant to the purpose(s) for which it is to be used.

Requirements for user consent

NOTE:	The Man-Machine Interface (MMI) aspects of the following requirements are out of 3GPP scope.

[PR 5.1.3-003] Subject to regulatory requirements, the 6G system shall enable the user to set or update user consent settings based on the following:

-	the feature or the service for which this user consent applies;

-	whether or not consent was granted;

-	the user-related information to be processed;

-	the purpose(s) for the information processing (e.g. for service provisioning, service optimisations);

-	whether the entity using the information is the network operator or a third party;

-	the actions taken by the network to process the information (e.g. collection, storage).

NOTE:	There are certain types of user information that is essential to be used by the 6G system to e.g. enable a service to be provided to the user, and without this user information, the service does not work. For these types of user information, it is assumed user consent does not need to be provided.

[PR 5.1.3-004] Subject to regulatory requirements, the 6G system shall collect and process user-related information according to the applicable user consent expressed by the user. For example, if the user has consented for the network to collect a type of UE-generated measurements to provide a certain service, the 6G network then collects and processes this specific UE measurement to provide the service.

[PR 5.1.3-005] Subject to regulatory requirements, the 6G system shall enable the user to obtain information of instances where the network or trusted third party has processed user-related information for a specific granted user consent. For example, this can be done periodically or upon user request. Examples of information that could be shared with the user are the name of the related feature / service, the number of times user-related information was processed, whether the information was shared with a third party, etc.

[PR 5.1.3-006] Subject to regulatory requirements, the 6G system shall enable the user to revoke user consent at any time. The 6G system shall enable the user to check the latest status of the user consent setting(s) in the 6G system.

[PR 5.1.3-007] Subject to regulatory requirements, upon revocation of user consent, the 6G network shall:

-	stop the collection and processing of user-related information; and

-	provide a confirmation to the user once the 6G network has taken action to comply with the revocation of user consent (e.g. termination of user-related information collection, deletion of user-related information).