3GPP TSG-SA WG1 Meeting #108 	S1-244312

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Use Case On Glasses-free 3D Communication

Agenda Item:	8.1.4

Source:	China Mobile

Contact:	Shenxiu Du <dushenxiu@chinamobile.com>

Xiaonan Shi <shixiaonan@chinamobile.com>

Abstract: This document proposes a new use case on glasses-free 3D communication.

* * * First Change* * * *

8.x	Use case on glasses-free 3D communication

8.x.1	Description

As a video display effect enhancement technology, glasses-free 3D technology can be widely applied in video display scenarios, which also means that glasses-free 3D technology has broad application prospects in the field of communications. At present, various video communication services such as video calls, video conference, video CAT and Video CRS already have a large user scale. Introducing glasses-free 3D technology into the 6G system and making it one of the characteristic functions of 6G will effectively enhance the video display effect of video services and give users a brand-new usage experience.

Glasses-free 3D technology generally refers to a display technology in which users can directly see 3D videos on a display device without wearing wearable devices. However, different from general video technology, there are different formats for glasses-free 3D video streams, such as left-right format and top-bottom format. Therefore, the display device needs to start a specific decoder to decode and play according to the format of the glasses-free 3D video stream.

Figure 8.x.1-1  Left-right format:The images of the left eye and the right eye are transmitted in the same frame, and the horizontal resolution is halved.

Figure 8.x.1-2  Top-bottom format:The images of the left eye and the right eye are transmitted in the same frame, and the vertical resolution is halved.

8.x.2	Pre-conditions

1. UEA supports decoding and playing the glasses-free 3D video.

2. UEB does not support glasses-free 3D communication.

8.x.3	Service Flows

- User A uses UEA to request to establish a glasses-free 3D video call with UEB. UEA reports to the 6G system that only glasses-free 3D video in left-right format is supported .

- The 6G system conducts glasses-free 3D video media negotiation with UEB. UEB reports to the 6G system that  glasses-free 3D communication is not supported.

- User B uses UEB to answer this video call.

- 6G system converts the video stream from UEB into a glasses-free 3D video stream in left-right format.

- The 6G system sends the converted glasses-free 3D video stream to UEA.

- UEA receives the glasses-free 3D video stream and decodes and plays the stream.

8.x.4	Post-conditions

User A can see glasses-free 3D videos on UEA without the help of any wearable device.

8.x.5	Existing features partly or fully covering the use case functionality

None

8.x.6	Potential New Requirements needed to support the use case

[PR 8.X.6-1] The 6G system should allow UE to report the supported glasses-free 3D video stream formats.

[PR 8.X.6-2] The 6G system should support converting 2D video streams into glasses-free 3D video streams in the specified format.

[PR 8.X.6-3] The 6G system should notify the UE of the video stream format before sending glasses-free 3D video streams to the UE.

* * * End of Change * * * *