3GPP TSG SA WG 1 Meeting #108 	S1-244338

Orlando, USA, 18-22 November 2024

Source:	Nokia, ETRI

pCR Title:	Pseudo-CR on updates of use case 5.18 to reconsider potential new requirements

Draft Spec:	3GPP TR 22.887-020

Agenda item:	7.3

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 5.18 in FS_5GSAT_Ph4 TR.

1. Introduction

This contribution proposes to update the description, existing features and potential new requirements in use case “using GEO to land data in a desirable geographical location”.

2. Reason for Change

This use case seems to be related to ‘geofencing’ w.r.t. service link and feeder link. This seems not related to satellite orbit type as it can be achieved with both GSO and  NGSO (driven by SatCo ephemeris data satellite beam activation). Feeder link routing is managed by SatCo ephemeris data too.

Also some format cleanup for pre-conditions.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887-020.

* * * First Change * * * *

## 5.18	Use case on using GEO to land data in a desirable geographical location

### 5.18.1	Description

NGSO satellites typically have a much smaller field of view (FOV) than a GEO satellite.  Thus NGSO satellites have to either download traffic from a UE to a gateway within its FOV, or transfer via inter satellite links (ISL) the data to another NGSO satellite in order to land the data outside the FOV of the satellite connecting to the UE.  These ISLs sometimes often do not have as much bandwidth as the primary links.  ISLs may also be unavailable under various conditions due to satellite duty cycle constraints or routing conflicts.

It is also desirable for many customers to download data to network based in a specific geographic areas.  This may be for intellectual property, security, or national security requirements.  Many customers may want to avoid areas that belong to adversarial nations or have geopolitical instability. We will refer to this as data with a preferred satellite ground station location.

Using a multi-orbit satellite system allows differentiated data landing based on customer needs.  E.g., the UE can connect to a NGSO and then have its data sent to a GEO satellite from which the data can be landed anywhere in the field of view of the GEO.  It may also be the case that the UE with data with preferred satellite ground station location, directly communicates with a GEO satellite bypassing the NGSO entirely.

### 5.18.2	Pre-conditions

- The network supports both GEO and NGSO satellites

- The network supports handover between GEO satellite(s) and NGSO satellite(s)

- A network plan that deconflicts frequency allocations between the GEO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The network must be able to determine which data has a preferred satellite ground station location.

- The network must be able to determine whether each satellite network has access to the preferred satellite ground station location.

### 5.18.3	Service Flows

- During communication, the network or UE determines that data will be exchanged that has a preferred satellite ground station location

- The network shall determine the optimal path to route traffic and send traffic through that path.

### 5.18.4	Post-conditions

Data with preferred satellite ground station location is received securely at preferred location(s).

### 5.18.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 5.18.6	Potential New Requirements needed to support the use case

[PR 5.18.6-001] A 5G system with multi-orbit satellite access shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location.

NOTE: 	The link between satellite and satellite ground station location is out of 3GPP scope

Editor's Note:	it is FFS how it is related to multi-orbit as it seems to be covered by geofencing supported by single-orbit satellite access.