3GPP TSG-SA1 Meeting #108	S1-244403

# Orlando, United States, 18-22 Nov 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.268 | CR | 0085 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PWS for IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Novamint, Qualcomm, SyncTechno Inc., InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-11-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN2 has started work on the support of broadcast of PWS messages for IoT NTN. Current requirements for CMAS and EU-ALERT are not updated with regard to supported RATs, i.e. Satellite E-UTRAN In addition, the note in clause 4.2 related to UEs “supporting eDRX may not support all requirements for PWS” needs to be updated since eDRX is a mandatory feature when connected to an NB-IoT RAN, the UE sends no capability indication for eDRX. In addition other RATs are missing - NG-RAN is missing for CMAS. - Satellite NG-RAN is missing for CMAS and EU-ALERT Other editorial fixes: CMAS “Presidential” alert has been renamed by the FCC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the note related to support of eDRX and fixed NOTE numbering. Added NG-RAN for CMAS Added Satellite NG-RAN and Satellite E-UTRAN for CMAS and EU-ALERT CMAS “Presidential” alert renamed to “National |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing or misleading requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 6.1, 6.2, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S1-244403 |  |  |  |  |  |  |  |  |

## =============== First Change =============

## 4.2	High level general requirements for Warning Notification delivery

The following list gives the high level general requirements for Warning Notification delivery:

-	PWS shall be able to broadcast Warning Notifications to multiple users simultaneously with no acknowledgement required.

-	PWS shall be able to support concurrent broadcast of multiple Warning Notifications.

-	Warning Notifications shall be broadcast to a Notification Area which is based on the geographical information as specified by the Warning Notification Provider.

-	PWS capable UEs (PWS-UE) in idle mode shall be capable of receiving broadcasted Warning Notifications.

NOTE 1:	A bandwidth reduced low complexity UE, an NB-IoT UE or a UE supporting eDRX does not necessarily meet all requirements for PWS, including ETWS, CMAS, EU-Alert and KPAS

-	PWS shall only be required to broadcast Warning Notifications in languages as prescribed by regulatory requirements.

-	Warning Notifications are processed by PWS on a first in, first out basis, subject to regulatory requirements.

-	Reception and presentation of Warning Notifications to the user shall not pre-empt an active voice or data session.

-	Warning Notifications shall be limited to those emergencies where life or property is at imminent risk, and some responsive action should be taken.

NOTE 2: 	This requirement does not prohibit the use of the operator’s network (i.e. broadcast technology) implemented for Warning Notifications to be used for commercial services.

## =============== Next Change =============

## 6.1	Introduction to CMAS

The Warning Alert and Response Network (WARN) Act was passed by the United States Congress in September 2006 and was signed into law on October 13, 2006, known then as CMAS. CMAS was later renamed as Wireless Emergency Alert (WEA).

The Federal Communication Commission (FCC) released several Report and Order rulings for the Commercial Mobile Alert System. [1], [2], [3], [8] and [10].

As a result of these legislative and regulatory actions the Alliance for Telecommunications Industry Solutions (ATIS) and the Telecommunications Industry Association (TIA) agreed to develop ATIS and TIA standards to support the deployment of CMAS. This standard is listed in Clause 2.

The scope of these specifications includes the support of Commercial Mobile Alert System (CMAS) alert message via the GSM/UMTS Cell Broadcast functionality and warning message delivery via E-UTRAN, Satellite E-UTRAN, NG-RAN and Satellite NG-RAN. These specifications covers the mapping of CMAS messages onto the 3GPP-defined Cell Broadcast functionality and warning message delivery functionality, as well as the CMAS application layer functionality from the reception of the CMAS alert message from the Warning Notification Provider to the point of reception by the CMAS capable mobile device.

The CMAS functionality does not require modifications to the 3GPP-defined cell broadcast functionality.

The following functional reference model is based on the diagram from Section III.B.10 of the FCC First Report and Order for the Commercial Mobile Alert System, FCC 08-99 [1]:

Figure 6.1-1: CMAS Reference Architecture

## 6.2	Additional PWS Requirements Specific to CMAS

In addition to the General Requirements specified in Clause 4, the following requirements are specified for the deployment of CMAS. These CMAS specific requirements are based on the FCC Report and Orders listed in clause 6.1 and further specified in [4].

Warning Notifications shall support up to 360 characters of GSM 7 bit Default Alphabet [9] for E-UTRAN, Satellite E-UTRAN, NG-RAN and Satellite NG-RAN.

The following classes of Warning Notifications shall be supported: National, Imminent Threat, Public Safety, Child Abduction Emergency (e.g. AMBER) , and State/Local WEA Test.

When a National Warning Notification is received, it shall always be presented to the user whenever Cell Broadcast Service via GSM/UMTS or warning notification delivery via E-UTRAN, Satellite E-UTRAN, NG-RAN and Satellite NG-RAN is enabled on the UE.

If the National Warning Notification is received in English, then it shall be displayed by the UE. If the National Warning Notification is received in a language other than English, then it shall only be displayed by the UE if the User has selected that language.

A CMAS-capable User Interface (UI) shall support the ability for the user to opt-out of only the Imminent Threat, Public Safety and Child Abduction Emergency Warning Notifications. A CMAS-capable User Interface (UI) shall support the ability for the user to opt-in to the State/Local WEA Test notification.

If the User has opted out of a Warning Notification class, then the Warning Notification for that class shall not be displayed by the UE.

If the User has not opted out of a Warning Notification class, then any Warning Notifications for that class received in English shall be displayed by the UE. This also applies to Warning Notifications received in any additional languages that may be selected by the User.

Warning Notifications may contain URLs and/or phone numbers.

A CMAS-capable User Interface (UI) shall support the ability (e.g., a “click” touch input) for the user to navigate to the URL or initiate a voice call to the phone number which may be included in the Warning Notification [8].

The PLMN broadcasting CMAS Warning Notifications shall also broadcast corresponding CMAS Warning Notification Area(s) when the CMAS Warning Notification is required to adhere to the maximum 0,1 of a mile CMAS Warning Notification Area(s) overshooting as specified in [10]. This requirement applies only to LTE and NR accesses.

A CMAS-capable UE shall utilize UE-Based Location Calculation (see 3GPP TS 22.071 [11]) to determine whether it is located within the corresponding CMAS Warning Notification Area(s) in order to minimize overloading network LCS resources.

Subject to opt-out and opt-in settings described above, a CMAS-capable UE shall present the CMAS Warning Notification sent if there is a corresponding CMAS Warning Notification Area(s) and if the CMAS-capable UE is able to determine that its location is within the corresponding CMAS Warning Notification Area(s). Otherwise, if a CMAS-capable UE is unable to or fails to determine its location, the CMAS-capable UE shall present the Warning Notification to the user.

A CMAS-capable UE shall store presented Warning Notifications for at least 24 hours unless explicitly deleted by the user.

## =============== Next Change =============

## 7.2	Additional PWS Requirements Specific to EU-ALERT

In addition to the General Requirements specified in Clause 4, the following requirements are specified for the deployment of EU-ALERT. These EU-ALERT specific requirements are further specified in ETSI TS 102 900 [6].

EU-ALERT shall support three types of Warning Notifications: Alert messages to warn citizens of an imminent emergency situation, Advisory messages of lesser urgency, and Amber alerts.

The Alert messages shall be supported with three levels of severity. EU-Alert level 1 shall have no opt-out; levels 2 and 3 shall allow opt-out by the user. All levels of EU-ALERT messages shall be associated with a dedicated alerting indication.

NOTE: 	EU-Alert level 1 is compatible with the Presidential Alert in CMAS. EU-Alert level 2 is compatible with CMAS Extreme Alerts and EU-Alert level 3 is compatible with CMAS Severe Alert.

The Advisory messages have only one level. Advisory messages shall not be associated with any dedicated alerting indication.

Depending on national requirements, Amber alerts may need to be broadcast as part of the EU-ALERT service. Amber alerts shall not be associated with any dedicated alerting indication.

EU-ALERT shall support Warning Notifications in various languages. To support international roaming, it is expected that countries adopting EU-ALERT use the same Message Identifier for Warning Notifications in the local language. If Warning Notifications are broadcast in other languages besides the local language, then the Message Identifier for such Warning Notifications are expected to be the same across the countries adopting EU-ALERT.

EU-ALERT shall be supported on GERAN, UTRAN, E-UTRAN, Satellite E-UTRAN, NG-RAN  and Satellite NG-RAN.

EU-ALERT shall support that the PLMN broadcasting EU-ALERT Warning Notifications also broadcasts corresponding Warning Notification Area(s). This requirement applies only to E-UTRAN, Satellite E-UTRAN, NG-RAN and Satellite NG-RAN.

A EU-ALERT-capable UE shall utilize UE-Based Location Calculation (see 3GPP TS 22.071 [11]) to determine whether it is located within the corresponding Warning Notification Area(s) in order to minimize overloading network LCS resources.

Subject to opt-out and opt-in settings described above, a EU-ALERT-capable UE shall present the EU-ALERT Warning Notification sent if there is a corresponding Warning Notification Area(s) and if the EU-ALERT-capable UE is able to determine that its location is within the corresponding Warning Notification Area(s). Otherwise, if a EU-ALERT-capable UE is unable to or fails to determine its location, the EU-ALERT-capable UE shall present the EU-ALERT Warning Notification to the user.