3GPP TSG-SA WG1 Meeting #108	S1-244452

Orlando, Florida, USA, 18-22 November 2024								(revision of S1-244388)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0037 | rev | 2 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New use cases: Merging of two multi-train voice communications by the train driver |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph6 |  |  |  |  |  | Date: |  |  | 2024-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of merging use cases for multi-train voice communications, as per UIC functional FRMCS specifications (FU-7120). |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the continuation of the introduction of ‘merging’ feature two additional use cases are described: c) Multi-train voice communications initiated by drivers and merged by one of the drivers d) One multi-train voice communication initiated by driver and another by controller, both merged by driver. Initial and merged communications are using MCX Ad hoc Group Communications feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirements for downstream groups to specify ‘merging’ feature with MCX Ad hoc Group Communications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.7(new), 6.2.7.1(new), 6.2.7.2(new), 6.2.7.3(new), 6.2.7.4 (new), 6.2.7.5 (new), 6.2.8(new), 6.2.8.1(new), 6.2.8.2(new), 6.2.8.3(new), 6.2.8.4 (new), 6.2.8.5 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 6.2	Multi-train voice communication for Drivers and Ground FRMCS User(s)

### 6.2.1	Introduction

In this chapter the use cases related to the function of Multi-train voice communication from the Drivers and/or a Ground FRMCS User towards the Driver and/or Ground FRMCS Users are defined. Ground FRMCS Users may include Controllers. The following use cases are defined:

- Initiation of Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

- Join a Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

- Termination of Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

- Interworking GSM-R and FRMCS for Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

- Merging of two multi-train voice Ad hoc Group communications. There are different use cases when merging two multi-train voice communications depending on the ‘merging authority’ and the initiators of the two separate calls, as presented in the following:

- a) Two multi-train voice communications initiated by train drivers and merged by train controller

- b) A multi-train voice communication initiated by train driver and another initiated by train controller, merged by the train controller

- c) Two multi-train voice communications initiated by train drivers and merged by the train driver

- d) A multi-train voice communication initiated by train driver and another initiated by train controller, merged by the train driver.

Next change

### 6.2.7	Use case: Merging of two multi-train voice communications initiated by Train Drivers and merged by Train driver

#### 6.2.7.1	Description

The use case is derived from train driver’s operational activities. A train driver has initiated a multi-train voice communication with other train drivers and controllers in his/her area of movement. When the initiated multi-train voice communication is on-going, he/she receives another multi-train voice communication initiated by another driver impacting an overlapping addressed area or an addressed areas close to his/her addressed area. In this case, there is interesting information to be shared by the two groups participating in the two multi-train voice communications, so one of the two initiator drivers decides to merge the two multi-train voice communications. Moreover, it is more convenient for the initiator driver to handle only one communication.

#### 6.2.7.2	Pre-conditions

All involved FRMCS users (Controller A, Driver 1, Driver 2, Driver 3, Controller B, Driver 4, Driver 5, Driver 6) have been successfully registered to the FRMCS Service Server and selected their functional roles.

Driver 1 is authorized to initiate a multi-train voice communication.

Driver 4 is authorized to initiate a multi-train voice communication.

Driver 1 is authorized to merge two multi-train voice communications.

Driver 1 is authorized to terminate a multi-train voice communication.

There are two on-going multi-train voice communications. The participants of each one are selected by the FRMCS Service Server, based on the initiator’s location.

The participants of the two multi-train voice communications belong to the same FRMCS Service Server (same FRMCS domain).

Controller A benefits of the floor control priority (multi-user talker control of the FRMCS system) when involved in a multi-train communication.

Location information of all participants of multi-train communications can be provided by the FRMCS system.

The FRMCS system is preconfigured to define which controller is responsible for each addressed area.

#### 6.2.7.3	Service flows

- 1st multi-train voice communication initiated by Driver 1 (GC1: Group Communication1)

1) Driver 1 initiates a multi-train voice communication, with specific criterion of initiator’s location.

2) The FRMCS Service Server verifies if driver 1 is authorized to initiate a multi-train voice communication. If not authorized, the FRMCS Service Server rejects the multi-train voice communication initiated by driver 1.

3) Based on driver’s 1 location, the FRMCS Service Server determines the addressed area (GC1_AdA) using the initiating area.

4) Driver 2 and driver 3 are in the addressed area. Controller A is responsible for this addressed area.

5) The FRMCS Service server invites the participants fulfilling the criterion of GC1 (GC1_AdA), driver 2, driver 3 and controller A. Finally, the multi-train voice communication is established with set-up time specified as NORMAL (see 12.10).

6) All participants’ respective floor priority is considered when multi-user talker control is used during the multi-train voice communication of this group.

- 2nd multi-train voice communication initiated by Driver 4 (GC2: Group Communication2)

1) Driver 4 is also initiating a multi-train voice communication with the specific criterion of initiator’s location.

2) The FRMCS Service Server verifies if driver 4 is authorized to initiate a multi-train voice communication. If not authorized, the FRMCS Service Server rejects the multi-train voice communication initiated by driver 4.

3) Based on driver’s 4 location, the FRMCS Service Server determines the addressed area (GC2_AdA) using the initiating area.

4) Driver 5 and driver 6 but also driver 1 are in the addressed area which partially overlaps or is closed to the addressed area associated to driver’s 1 multi-train voice communication. Controller B is responsible for this addressed area.

5) Controller B benefits of the floor control priority (multi-user talker control of the FRMCS system) when involved in a multi-train communication.

6) The FRMCS Service server invites the participants fulfilling the criterion of GC2 (GC2_AdA), Driver 5, driver 6, controller B and driver 1. Finally, the multi-train voice communication is established with set-up time specified as NORMAL (see 12.10).

7) All participants’ respective floor priority is considered when multi-user talker control is used during the multi-train voice communication of this group.

- Driver 1 requests the FRMCS system for merging on-going GC1 and GC2

1) Driver 1 is now participating in two multi-train communications, GC1 and GC2. From the exchanged information, driver 1 realises that there is interesting information for both groups, moreover because they are sharing a partially overlapped addressed area or their addressed areas are neighbouring ones. Therefore, driver 1 decides to merge GC1 and GC2.

2) The FRMCS Service Server verifies if driver 1 is authorized to merge a multi-train voice communication. If not authorized, the FRMCS Service Server rejects the request for merging of multi-train voice communication initiated by driver 1.

3) If authorised, driver 1 initiates a new multi-train (merged) voice communication, named GC3, based on a new criterion, (GC3_AdA), which is the union of addressed areas of GC1 (GC1_AdA) and GC2 (GC2_AdA), with the updated participants of both addressed areas, at the time the ‘merging’ happened. The participants of the merged multi-train communications are notified.

4) The FRMCS Service server invites the participants fulfilling the criterion of (GC3_AdA), driver 1, driver 2, driver 3, driver 4, driver 5, driver 6 and controllers A and B. Finally, the multi-train voice communication is established with set-up time specified as NORMAL (see 12.10).

5) The FRMCS Service server continues to check the participants that satisfy the new criterion, corresponding to the addressed area of (GC3_AdA) and invites or removes FRMCS users, as usually in a multi-train communication based on criterion.

6) If the merged multi-train train voice communication based on the criterion of (GC3_AdA) must be terminated, this is to be performed by driver 1 who is authorised to do so in the initial group.

#### 6.2.7.4 	Post-conditions

A merged multi-train voice communication is established, including all participants of both multi-train voice communications GC1 and GC2, at the time the ‘merging’ was performed. Once the merged communication is established, the relevant participants keep their previous floor priorities and authorisations.

#### 6.2.7.5 	Potential requirements and gap analysis

Note: 	The gap analysis of both use cases 6.2.5.5 and 6.2.6.5 are also applicable to this use case because the same or different criteria for the initiating driver can apply.

### 6.2.8	Use case: Merging of one multi-train voice communications initiated by train driver and another initiated by the train controller, merged by the train driver

#### 6.2.8.1	Description

The use case is derived from train driver’s and controller’s operational activities. A concrete example is the banking use case (train traction manoeuvre) where the train driver initiates a multi-train voice communication to all drivers of the same train. The responsible controller is involved in an ongoing multi-user voice communication with other controllers who are also responsible for the movement of the train and wants to invite in this multi-user voice communication between controllers, the driver who has initiated the banking communication. Finally, the train driver initiator of the banking voice communication is involved in two multi-user voice communications dealing with the same banking manoeuvre and decides to merge them.

#### 6.2.8.2	Pre-conditions

All involved FRMCS users (Controller A, controller B, Driver 1, Driver 2, Driver 3) have been successfully registered to the FRMCS Service server and selected their functional roles.

Driver 1 is authorized to initiate a multi-train voice communication for banking.

Controller A is authorized to initiate a multi-train voice communication.

Driver 1 is authorized to terminate a multi-train voice communication for banking.

There are two on-going multi-train voice communication. The participants of each one are selected by the FRMCS Service server, based on different criteria (e.g., list of functional identities having the same vehicle identifier as location label for the driver-initiated call and list of participants based on addressed area for the controller-initiated call).

The participants of the two multi-train voice communications belong to the same FRMCS Service Server (same FRMCS domain).

Controller A benefits of the floor control priority (multi-user talker control of the FRMCS system) when involved in a multi-train communication.

Location information of all participants of multi-train communications can be provided by the FRMCS system.

The FRMCS system is preconfigured to define which controller is responsible for each addressed area.

#### 6.2.8.3	Service flows

- 1st multi-train voice communication initiated by Driver 1 (GC11)

1) Driver 1 initiates a multi-train voice communication, with specific criterion of list of participants with location label, the same vehicle identity, in their functional identities.

2) The FRMCS Service Server verifies if driver 1 is authorized to initiate a multi-train voice communication. If not authorized, the FRMCS Service Server rejects the multi-train voice communication initiated by driver 1.

3) Based on driver’s 1 location label, the FRMCS Service Server determines the list of participants (GC1_List_Functional Identities) using the same vehicle identity.

4) Driver 3 is included in the list.

5) The FRMCS Service server invites the participants fulfilling the criterion of GC1 (GC1_ List_Functional Identities), such as driver 3. Finally, the multi-train voice communication is established with set-up time specified as NORMAL (see 12.10).

6) All participants’ respective floor priority is considered when multi-user talker control is used during the multi-train voice communication of this group.

- 2nd multi-train voice communication initiated by Controller A (GC2)

1) Controller A is also initiating a multi-train voice communication with the specific criterion of selected addressed area (GC2_SAdA).

2) The FRMCS Service Server verifies if controller A is authorized to initiate a multi-train voice communication. If not authorized, the FRMCS Service Server rejects the multi-train voice communication initiated by controller A.

3) Driver 1 and controller B are located inside the selected addressed area

4) The FRMCS Service server invites the participants fulfilling the criterion of (GC2_ SAdA) driver 1 and controller B. Finally, the multi-train voice communication is established with set-up time specified as NORMAL (see 12.10).

5) All participants’ respective floor priority is considered when multi-user talker control is used during the multi-train voice communication of this group.

- Driver 1 requests the FRMCS system for merging on-going GC1 and GC2

1) Driver 1 is now participating in two multi-train communications, GC1 and GC2. Driver 1 considers that it is useful to merge GC1 and GC2, also considering the banking use case.

2) The FRMCS Service Server verifies if Driver 1 is authorized to merge two multi-train voice communication. If not authorized, the FRMCS Service Server rejects the request for merging of multi-train voice communication initiated by driver 1.

3) If authorized, Driver 1 initiates a new multi-train (merged) voice communication, named GC3, based on a new criterion, (GC3_ List_Functional Identities), which is the sum of list of participants of GC1 (GC1_List_Functional Identities) and participants corresponding to (GC2_SAdA), with the updated participants at the merging time. The participants of the two merged communications are informed about the merging.

4) The FRMCS Service server continues to check the participants that satisfy the new criterion, corresponding to the union of two criteria and invites or removes FRMCS users, as usually in a multi-train communication based on criterion.

#### 6.2.8.4 	Post-conditions

A merged multi-train voice communication is established, including all participants of both multi-train voice communications GC1 and GC2, at the time the ‘merging’ was performed. Once the merged communication is established, all the relevant participants keep their previous floor priorities and authorisations.

#### 6.2.8.5 	Potential requirements and gap analysis

Potential requirements derived from the gap analysis of §6.2.6.5 are also applicable for this use case. An additional one is:

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.2.8-001] | The FRMCS System shall enable an authorised FRMCS User to merge two multi-train voice communications, one based on list of participants (e.g., list of functional identities with same location label, as drivers of the same train) and another based on addressed area | A | 22.280 | Not covered. CR is needed for an authorised user to merge Ad hoc group communications, using list of functional identities and addressed area, as criteria. |

End of changes