3GPP TSG-WG SA1 Meeting #108	S1-244573

Orlando, US, 18th Nov – 22nd Nov, 2024	     								 (revision of S1-244376)

Source:	TNO

pCR Title:	Pseudo-CR on Use case on multi-orbit satellite connectivity to mobile base station relay (MBSR)

Draft Spec:	3GPP TR 22.887 v0.2.0

Agenda item:	7.3

Document for:	Approval

Contact:	Relja Djapic <relja.djapic@tno.nl>

Abstract: This document provides some updates for the use case on multi-orbit satellite connectivity to mobile base station relay (MBSR). The use case describes how a mobile base station relay exploits simultaneous connectivity through distinct satellite access paths.

1. Introduction

The aim of this contribution is to update the use case 5.20 proposed within FS_5GSAT_Ph4.

2. Reason for Change

According to the approved scope of FS_5GSAT_Ph4, this study focuses on multi-orbit satellite access for multiple services.

Use case 5.20 focuses on the secnario that mobile base station relay (MBSR) mounted on an airplane is connected through a terrestrial mobile network to the 5G Core when the airplane is on the ground. When the airplane becomes airborne the airplane mounted MBSR will connect to the 5G Core through simultaneous connectivity over two or more multi-orbit satellite access links.

3. Proposal

Proposal: It is proposed to update Use Case 5.20, in order to align it with the scope of FS_5GSAT_Ph4 focusing on satellite access and corresponding services.

* * * First Change * * * *

## 5.20	Use case on multi-orbit satellite connectivity to mobile base station relay (MBSR)

### 5.20.1	Description

Satellite access is a communication technique for provisioning of connectivity in rural and remote areas not covered by terrestrial access networks. Satellite access could be provided through satellites located in various orbits e.g. GEO, MEO, LEO. Each of these orbits has its own specificities related to coverage, data rates, capacity and latency. Similarly different services require different access network capabilities. In line with this, services requiring low latency and high data rates would optimally be served via LEO satellite access while delay tolerant services (e.g. txt, messaging) could be routed through a GEO satellite access.  Simultaneously combining multi-orbit satellite access links (LEO/MEO/GEO) could be of interest in cases when a single satellite access link cannot meet the prescribed requirements (e.g., capacity, latency). Specific characteristic of each satellite access link needs to be taken into account when combining it with another satellite access link in order to achieve required QoS of each user within a group of users (aggregate QoS).

Figure 5.20.1 shows an example of multi-orbit satellite access where access  links can be established through GEO and NGSO satellite constellation. Simultaneous usage of multi-orbit satellite accesses serves a mobile base station relay (MBSR) located on the airplane (or possibly on the ground on a remote location).

Gateway

Data

Network

ISL

5G NR

NGSO satellite

gNBGEO

gNBLEO

5G NR

MBSR

Figure 5.20.1: Illustration of multi-orbit satellite use-case simultaneously exploiting GEO  and NGSO satellite access links serving mobile base station relay (MBSR) on board satellite. ISL indicate intersatellite links.

GEO satellite backhaul

GEO satellite

Gateway

Data

Network

ISL

gNB

5G NR

NGSO satellite

NGSO satellite backhaul

ISL

5G NR

UE

UE

### 5.20.2	Pre-conditions

MLK is an airline company known for its high end on-board services. MLK takes good care of its passenger satisfaction and intends to upgrade its on-board entertainment services with on-line messaging, internet access, video streaming and gaming. MLK is looking for connectivity solutions that will enable the upgrade of its on-board entertainment. Cost efficiency of the connectivity solution is of great importance for MLK in order to remain competitive on the tight airline market.

### 5.20.3	Service Flows

A satellite operator SAOP having GEO and NGSO satellites in its satellite fleet offers satellite access connectivity solutions for the airplanes in MLK fleet anywhere around the globe. SAOP offers the most cost efficient solution for MLK as the different service types e.g., messaging, internet, video/gaming, will transfer their traffic over satellite access links optimal for the selected service e.g., GEO, MEO, LEO, respectively.

While on the ground the mobile base station relay (MBSR) mounted on an airplane is connected to the 5G Core through a terrestrial mobile access network. Passengers on board the airplane acquire wide variety of services (latency sensitive, latency tolerant, messaging, video or gaming) through the terrestrial mobile access network and the MBSR.

After the take-off, mobile base station relay mounted on the airplane loses the connectivity through terrestrial mobile access and switches over to satellite access through multiple satellite orbits i.e. LEO, MEO and/or GEO satellites. Satellite access through distinct orbits have different properties with respect to latency, data rate, spectrum, coverage and availability of the satellites in different orbits. 5G system is capable of differentiating traffic among various service types of the airplane passengers. For example, 5G system will route latency tolerant services’ traffic (e.g., messaging) carried via a mobile base station relay through GEO satellite backhaul, latency sensitive services’ traffic (e.g., mission critical communication, on-line gaming)  carried via a mobile base station relay through LEO satellite access and video traffic  carried via a mobile base station relay on demand through MEO satellite backhaul.

### 5.20.4	Post-conditions

MLK improved offer of their services in a cost efficient manner. MLK is able to provide wide range of services (latency sensitive, latency tolerant, high data rate and high reliability) by simultaneously exploiting multiple satellite access paths.

### 5.20.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] specifies the requirements for satellite backhauling i.e. that the 5G system shall support a mechanism to determine suitable QoS parameters for traffic over a satellite backhaul, based e.g., on the latency and bandwidth of the specific backhaul.

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mechanisms to differentiate charging information for traffic carried over satellite backhaul.

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mobile base station relays using 3GPP satellite NG-RAN (NR satellite access).

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mobile base station relays accessing to 5GC via NR satellite access and NR terrestrial access simultaneously.

The above listed requirement are focused on usage of a single satellite orbit. The cases of possible simultaneous usage of multi-orbit satellite connectivity are not explicitly considered.

### 5.20.6	Potential New Requirements needed to support the use case

[PR 5.20.6-001] The 5G system with satellite access supporting simultaneous connectivity over multiple satellite orbits shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the multi-orbit satellite accesses  (e.g., over GEO, MEO, LEO) to  provision required aggregated QoS for traffic relayed via a mobile base station relay.

[PR 5.20.6-002] The 5G system with satellite access supporting simultaneous connectivity over multiple satellite orbits shall be able to collect and distinguish charging information related to the multi-orbit satellite access  traffic carried via a mobile base station relay. .

Editor’s NOTE: whether the use of simultaneous connectivity to multiple orbit satellite for MBSR is within the scope of the FS_5GSAT_Ph4 is FFS.

* * * End of Changes * * * *