3GPP TSG SA WG 1 Meeting #108 	S1-244653

Orlando, USA, 18-22 November 2024	(revision of S1-244566, S1-244333)

Source:	Nokia, ETRI, Novamint

pCR Title:	Pseudo-CR on updates of use case 5.5 to consider data rate

Draft Spec:	3GPP TR 22.887-020

Agenda item:	7.3

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 5.5 in FS_5GSAT_Ph4 TR.

1. Introduction

This contribution proposes to update the description, post-conditions and potential requirement in use case “support for Mobile base station relays (MBSRs) through multi-orbit satellite networks”.

2. Reason for Change

Seamless connectivity in the context of multi-orbit means that UEs can maintain continuous and consistent connectivity over both orbits with minimal service interruption. In this use case, GEO cannot provide low-latency services and as a result MBSR’s connectivity is switched to LEO. Therefore the description and post-conditions should not mention seamless connectivity.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887-020.

* * * First Change * * * *

## 5.5	Use case on support for Mobile base station relays (MBSRs) through multi-orbit satellite networks

### 5.5.1	Description

Mobile base station relay (MBSR) is a good option for providing continuous connectivity to moving UEs in a vehicle. MBSRs can be connected to the network through non-terrestrial networks (NTN). NTN deployments are essential for providing connectivity to relays in isolated and remote areas. The 5G system supports a multi-dimensional heterogeneous architecture, incorporating terrestrial access, low altitude aerial devices (e.g., UAVs), High Altitude Platform Station (HAPS) and satellites (Low Earth orbit (LEO), Medium Earth Orbit (MEO) and GEO)) to meet varying connectivity requirements. Different types of satellites in various orbits can serve different services based on their latency requirements. For example, GEO satellites can support services like video streaming (buffered), email, chat, file sharing while LEO satellites are better suited for services requiring low latency like V2X messaging and real-time gaming. With the movement of relays and depending on the requirements of connected UEs, switching between multi-orbit satellites can ensure all use case scenarios covered using NTN deployments only (as shown in Figure 5.5.1-1).

Figure 5.5.1-1: Multi-orbit satellite networks support for Mobile base station relays (The moving vehicle in the figure represents various scenarios for a mobile relay, whether it is on a ship, train, or aeroplane)

### 5.5.2	Pre-conditions

MBSR is connected through GEO satellite.  The service provider’s 5G system supports switching from GEO to available LEO connectivity based on service requirements.

### 5.5.3	Service Flows

Initially, the MBSR is connected to the network through GEO satellite links. When the vehicle moves where LEO  coverage is  available, the MBSR remains connected to GEO. However, since GEO cannot provide low-latency services, the MBSR’s connectivity is switched to LEO, ensuring that NTN can deliver all types of services to the connected UEs.

### 5.5.4	Post-conditions

MBSR is able to provide continuous connectivity to the connected UEs in the remote areas with the help of multi-orbit satellite network (when terrestrial network is not available for MBSR).

### 5.5.5	Existing features partly or fully covering the use case functionality

TS 22.261 [2] specifies the following requirements associated with the use case:

- A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency.

NOTE: 5 ms network latency is assumed and added to satellite one-way delay.

- A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency.

NOTE: 5 ms network latency is assumed and added to satellite one-way delay. The 5G system shall support operation of downlink only broadcast/multicast over a specific geographic area (e.g., a cell sector, a cell or a group of cells).

- A 5G satellite access network connected to 5G core networks in multiple countries shall be able to meet the corresponding regulatory requirements from these countries (e.g., Lawful Interception).

- The 5G system shall be able to support mobile base station relays using 3GPP satellite NG-RAN (NR satellite access).

- The 5G system shall be able to support mobile base station relays accessing to 5GC via NR satellite access and NR terrestrial access simultaneously.

### 5.5.6	Potential New Requirements needed to support the use case

[PR 5.5.6-001] Subject to operator’s policy, the 5G network with multi-orbit satellite access shall be able to support mobile base station relays to access different orbit satellites considering e.g., availability of the coverage, required QoS.