3GPP TSG-SA WG1 Meeting #108	S1-244682

Orlando, Florida, USA, 18-22 November 2024	                                                         (revision of S1-244430)

Title:	Ubiquitous Computing service in Autonomous Delivery Drone Fleet

Agenda Item:	8.1.6

Source:	NEC, InterDigital Inc.

Contact:                       Hiroshi Dempo, hiroshidempo@nec.com

Deepak Rohilla, deepak.rohilla@india.nec.com

Debashish Purkayastha, debashish.purkayastha@interdigital.com

Abstract: It is expected that ubiquitous use of computing resources would expand throughout the IMT-2030. Roadmap to 6G [x2] also has introduced a goal where communications/connectivity services in 6G work will work together with unified computing that scales across devices, network computing resources, and data centers. For simplicity, this use case discusses a ubiquitous computing service in the context of an autonomous delivery drone fleet in order to induce general requirements for the ubiquitous computing.

* * * Start of 1st change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x1]	ITU-R M.2160: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

[x2]	Next G Alliance Report: Roadmap to 6G, Feb. 2022.

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * End of 1st change * * * *

* * * Start of 2nd change * * * *

# 10	Further Use Cases on Industry and Verticals

Editor's Note: This clause considers various capabilities and use cases needed to support the specific needs of different vertical markets. This includes e.g. Hyper Reliable and Low-Latency Communication, digital twin and other industrial communications use cases. This is not an exhaustive list

## 10.x	Use case on Ubiquitous Computing service in Autonomous Delivery Drone Fleet

### 10.x.1	Description

According to ITU-R M.2160 [x1], it is expected that ubiquitous use of computing resources would expand throughout the IMT-2030. In this context, the expansion of ubiquitous computing in the network infrastructure is closer to the origin of the data and support for proliferation of ubiquitous intelligence throughout the IMT-2030. The reference also described that the trend contributes to the improvements for applications requiring real-time responses and data transport. Ubiquitous computing disseminated across IMT-2030 is expected to enable efficient utilization of resources and optimal placement of workloads, as well as scales and manages the infrastructure to run the applications. Roadmap to 6G ［x2］ also has introduced a goal where communications/connectivity services in 6G work will work together with unified computing that scales across devices, network computing resources, and data centers.

Ubiquitous computing can be utilized by variety of application workloads such as self-driving in automobile, augmented reality (AR)/virtual reality (VR)/immersive application, industrial and medical robotics, drones and vision/control systems. For simplicity, this use case discusses a ubiquitous computing service in the context of an autonomous delivery drone fleet in order to induce general requirements for the ubiquitous computing. An autonomous delivery drone fleet consists of unmanned aerial vehicles designed to transport goods quickly and efficiently without human intervention. Leveraging ubiquitous computing in 6G, these drones navigate safely while optimizing routes through a fleet management system.

Drones are responsible for delivering packages in an urban area. Each drone is equipped with a variety of sensors that monitor its systems, including battery health, motor performance, GPS functionality, and environmental conditions. These drones can dynamically select optimized routes by utilizing ubiquitous computing in the 6G system.

Drones can use the compute capability available at the edge of the network to determine flight path, etc. Drones can also use assistance with real time information and use the compute capability available on the flight path in network elements such as remote gNB, repeaters, etc. This remote network elements, termed as “Far-edge”, provides compute capability for collision detection and avoidance, real time path change, etc.  The far-edge capability enables computation even closer to the drones, provides extremely low-latency comutations such as data processing and real-time analytics. This ubiquitous computing service for verticals enables workloads to navigate the drones in complex urban landscapes more effectively, reliable and responsive.

### 10.x.2	Precondition

John, Manager in a company got a delivery order in his area and he assigned job to an autonomous delivery drone.

### 10.x.3	Service Flows

- Before the beginning of the business hour, an edge computation supports a computational workload for the fleet management system. The computational workload on the edge computation generates a set of initial delivery plan. Far edge computations support computational workloads for localized fleet management. The computational workloads on the edge and far edge computation may be deployed or instantiated by the telecom operator or verticals in air control markets.

- The computational workload on the edge computation delivers the initial plan to drones and far edge computations for the localized fleet management.

- The computational workloads on far edge computations further generates initial detail route plans for drones in the assigned zone that would come to the zone. The assigned drone collects to parcel and left the warehouse to delivery location.

- The drone continuously collects data from its sensors during flight. For instance, it monitors battery voltage, temperature, and motor RPM and sending a report to far edge computation in the 6G network.

- The computational workload on the far edge computation collects reports from drones flying in the assigned zone. The reports may also be converted into the acceptable format if different verticals use individual report format. Then the computational workload on far edge computation analyses the converted reports to infer instructions (e.g. optimized route, altitude from an entry to exit point) in the assigned zone. After the computation, the computational workload on far edge computation responds each drone with an optimized route for the drone in the step 3.

- To enhance their operational efficiency, the step 3 and step 4 may be repeated at every specified time. Therefore, the optimized route is frequently updated to adapt conditional and environmental changes.

- The drone changes from the current far edge computation to another far edge computation which manages another zone when it comes to a border of the zone.

- The drone repeats the step3, step 4 and step 5.

- At each step, information for the far edge computation can be exposed to verticals so that the vertical can utilise the information for their business improvements.

### 10.x.4	Post-condition

John is notified the successful delivery of the order.

### 10.x.5	Existing features partly or fully covering the use case functionality

- Not applicable

### 10.x.6	Potential New Requirements needed to support the use case

- [PR 10.x.6-1] The 6G system shall be able to  support computation offload from a UE to mobile opertor’s far-edge network elements (e.g. gNB, repeaters, relays, and etc.) in order to support real time operations for the UE.

- [PR 10.x.6-2] The 6G system shall be able to support discovery of computation availability in the mobile operator’s far-edge network elements, which can support required QoS.

* * * End of 2nd change * * * *