3GPP TSG-SA1 Meeting #108	S1-244773

# Orlando, Florida, USA, 18-22 November 2024                	   (was S1-244630, S1-244304)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0822 | rev | 2 | Current version: | 18.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 PALS Requirements clean-up Part 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V., Futurewei |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | PALS |  |  |  |  |  | Date: |  |  | 2024-11-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some of the Rel-18 PALS requirements were addressed differently or not addressed in stage 2 & 3: - Clause 6.42.2.2: requirement on spatial limitation is not fully met. Stage-2 spec TS 23.501 clause 5.30.2.3 explictly states “the location assistance information is not used for any area restriction enforcement”. Through careful configuration limitation to a single cell is possible, but not a general area restriction. No third party interface for control of the spatial extent is specified. - Clause 6.42.2.2: ), requirement on coordination between home network and hosting network is not fully met. The location validity as implied by “and location” in the requirement is not specified, and should hence be removed. - Clause 6.42.2.4: requirement on support of 3rd party providing localized service policy to UE is not met. Although TS 23.501 Annex N.2.3 mentions QoS configuration, it only specifies reuse of existing framework to configure CN, not UE. Slicing is not controlled by 3rd party service provider. And in general the 3rd party service provider cannot provide UEs with a localized service policy as written in the requirement. Even if we would see the credential holder as the 3rd party service provider, then the credential holder does not directly provide policies to the UE, only can provisioning credentials as per TS 23.501. - Clause 6.42.2.4: requirement on authenticating and authorizing a user of a UE accessing a hosting network if UE has no subscription to the hosting network and still needs to get authorized to use localized services via the hosting network is not specified in stage 2 & 3. Requirement relates to authenticating a user (e.g. through an access portal), not about the use of default credentials used in current onboarding solution of SNPN. - Clause 6.42.2.5: requirement on UE being able to access a localized service without subscription to the 3rd party service provider is not met. At least the part behind the comma “or 3rd party service provider” is not addressed. That part of the requirement is about being able to obtain a subscription to the localized services (e.g. via an access portal) even if the UE doesn’t have any subscription to the 3rd party service provider yet. However, the concept of access portal has not been addressed during stage 2 and 3 work. Furthermore the text on supporting NPN for localized services is not complete and confusing, i.e. not clear that PLMN can also serve as the home network for localized services and the word “can” does not limit hosting network to only NPNs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of scope in clause 6.41.2.1 and removal of non-implemented requirements for PALS in clauses 6.41.2.2, 6.41.2.4 and 6.41.2.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.41.2.1, 6.41.2.2, 6.41.2.4, 6.41.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.41.2.1	General

In the requirements below, it is assumed that:

-	The home network is a PLMN or an NPN. The hosting network is an NPN.

-	Only subscribers of a public network can roam into a PLMN. Examples of interworking scenarios between network operators and application providers for localized services are indicated in Annex H.

#### 6.41.2.2	Configuration of Localized Services in Hosting Network

NOTE:	The term “Service provider of localized services” includes also 3rd party service provider.

The 5G system shall support suitable mechanisms to allow automatically establishing localized service agreements for a specific occasion (time and location) and building temporary relationship among hosting network operator and other service providers including network operators or 3rd party application providers.

The 5G system shall support means for the service provider to request the hosting network via standard mechanisms to provide access to 3rd party services at a specific period of time and location. This period of time shall be flexible, so that a change in service provision can be decided at any time (e.g., to cancel or prolong local services in the locality of service delivery) based on localized services agreements.

Based on localized services agreements, the 5G system shall provide suitable means to allow the service provider to request and provision various localized service requirements, including QoS, expected/maximum number of users, event information for discovery, network slicing, required IP connectivity etc, and routing policies for the application of the localized services via the hosting network.

The 5G system shall support means for a hosting network to create policies and configure resources for the requested time and location for the 3rd party services based on the received request.

The 5G system shall support means for a hosting network to notify the service provider of the accepted service parameters and routing policies.

Subject to regulatory requirements and localized service agreements, the 5G system shall allow a home network operator to automatically negotiate policies with the hosting network for allowing the home network’s subscribers to connect at a specific occasion, e.g., time and location, for their home network services.

Subject to the automatic localized services agreements between the hosting network operator and home network operator, for UE with only home network subscription and with authorization to access hosting networks the 5G system shall support:

-	access to the hosting network and use home network services or selected localized services via the hosting network,

-	seamless service continuity for home network services or selected localized services when moving between two hosting networks or a host network and the home network.

The 5G system shall support a mechanism to enable configuration of a network that provides access to localized services such that the services can be limited in terms of their spatial extent (in terms of a particular topology, for example a single cell).

NOTE: In this release of this specification, only spatial limitation per cell or group of cells is supported.

The 5G system shall support a mechanism to enable configuration of a network that provides access to localized services such that the services can be limited in terms of the resources or capacity available, to correspond to requirements that apply only to the locality of service delivery, as specified by a service provider of localized services.

The 5G system shall support means for a hosting network to provide a 3rd party service provider with information for automatic discovery of the hosting network by the UEs to allow access to specific 3rd party services.

The 5G system shall support secure mechanisms to allow a home network to coordinate with a hosting network for a subscriber to temporarily access the hosting network (e.g., based on temporary credentials) at a given time (start time and duration).

#### 6.41.2.4	UE Configuration, Provisioning, Authentication and Authorization

Subject to localized services agreements, the 5G system shall enable a home network operator to authorize a UE for using its home network services via a hosting network for a certain period of time and/or location.

The 5G system shall enable a UE to use credentials provided by the hosting network with or without coordination with the home network of the UE, to make use of localized services via the hosting network with a certain time (including starting time and the duration) and location validity.

The 5G system shall be able to allow the home network to steer its UE(s) to a hosting network with the consideration of the location, times, coverage of the hosting network and services offered by the home network and hosting network.

The 5G system shall provide support to enable secure means to authenticate and authorize a user of a UE accessing a hosting network.

The 5G system shall be able to authenticate and authorize the UE of a user authenticated to a hosting network to access the hosting network and its localized services on request of a service provider.

#### 6.41.2.5	UE Discovery, Selection and Access

Subject to operator’s policy and agreement between a 3rd party service provider and operator, the 5G system shall enable a UE to receive and use configuration provided by a 3rd party service provider to discover and access a hosting network and localized services, including the considerations of prior service agreement with a 3rd party service provider and no prior subscription to hosting network. If the UE is able to obtain services from two networks simultaneously, it may additionally select the hosting network. If the UE cannot maintain the connection to the home network while selecting the hosting network, the selection shall only be done on request by the user, i.e., using manual selection.

The 5G system shall support secure means for a UE to select and access localized services which may be provided by a 3rd party service provider via a hosting network, independent of prior subscription to the hosting network .

The 5G system shall enable the home network to allow a UE to automatically select a hosting network for accessing localized services when specified conditions (e.g., predefined time, location) are fulfilled.

The 5G system shall be able to prevent a UE to re-access the hosting network after the localized services were terminated if the authorization for the localized services is no longer valid (e.g., can be based on certain conditions such as time or location of the user).

The 5G system may support means for a UE which may or may not have prior subscription to the hosting network to display human readable information on how to gain access to the hosting network and available 3rd party services.

The 5G system shall support a mechanism to allow a user to manually select a specific local hosting network.

NOTE: 	Additional information can be presented to the user to facilitate the manual network selection.

The 5G system shall be able to limit access of specific UEs to a configurable area of a hosting network's coverage area.

The 5G system shall be able to maintain privacy of a user against the hosting network while the UE does not make use of the hosting network, for example, to prevent tracking of UEs by hosting networks.

The 5G system shall enable the home network to instruct a UE to select a hosting network with certain conditions (e.g., predefined time, location) based on the request from a service provider.

The 5G system shall enable the home network to allow a UE to select a hosting network or change to another hosting network, without any additional user intervention as long as the delivered services, both localized services and home routed services, are unchanged.