3GPP TSG-SA WG1 Meeting #108 	S1-244871

Orlando, US, 18-22 November 2024	         (was S1-244827,S1-244809, S1-244532 S1-244325)

Title:	Mixed Reality (MR) Gaming

Agenda Item:	8.1.4 Immersive Communication

Source:	Ericsson

Contact:	Daniel Lönnblad (Daniel.lonnblad at ericsson.com)

Abstract: A use case for Mixed Reality gaming proposed for chapter 8 in TR 22.870

# 8	Immersive Communication

Editor's Note "Immersive communication" require support for mixed traffic of video, audio, haptic and other environment data in a reliable and synchronous manner, combining low latency and high data rates.

## 8.1	Use case 1

## 8.x	Use case on Mixed Reality gaming

### 8.x.1	Description

Immersive communication use cases will continue to drive requirements on mobile networks. The need for slim light-weight Mixed Reality (MR) / Augmented Reality (AR) glasses has been emphasized announcement of device manufacturers. Multiplayer MR gaming is considered a target use case for 2030 timeframe.

We here consider a multiplayer game with users having a shared spatial map of the environment. The virtual scene is updated and placed in the same way and location for all players simultaneously. A 6G network should provide differentiated connectivity and beyond-connectivity services with consistency in performance.

For a shared gaming experience, the technical requirements are determined by the upstream of the environment from different players, the downstream of virtual objects, and a game logic that defines the connection between reality and virtual objects for multiple users.

The following gaming scenario is considered:

Users A and B are simultaneously acquiring and upstreaming a spatial map of the local environment to a cloud server. In the cloud, a shared virtual scene is generated and transmitted to Users A and B. The users are moving outdoors and indoors in a Dense Urban scenario.

Key value impact analysis

Energy resources: The increase usage of energy connected to the new MR equipment should be considered by focusing on energy performance and consumption.

Material resources: The material resources connected to new MR equipment should be considered by focusing on total material use.

Emissions: Emissions to air, water and soil connected to the entire lifecycle of new MR equipment can be addressed with the aim to be minimized, but not avoided entirely.

Education: MR gaming can be used in education such as teaching languages.

Health: On the positive side, MR gaming likely increase physical activities compared to other computer gaming. Furthermore, MR gaming can be fun and lead to well-being. On the negative side is the risk of addiction to gaming. Specific scenarios, e.g. MR gaming used in healthcare like might also be used for reintegration of difficult cases in society (e.g., post-traumatic stress syndrome PTSD).

Inclusion & equality: There is a possibility for improvement of the social / (gender) equality by e.g. helping chronically ill people to live a better life and thereby participate in society. From a inclusion point of view, affordability is important aspect to consider as those without a headset otherwise are excluded.

Trustworthiness: Privacy and integrity issues related to the is risk that sensitive data is collected; that personal data get into the wrong hands; or intrusion of bystander privacy.

### 8.x.2	Pre-conditions

The following pre-conditions are considered:

- Two or more users are co-located and collaborating on a game in a mixed digital and physical environment.

- The users are equipped with light-weight AR/MR devices and connected to a 6G network.

- Low latency sensor hardware is required on the devices for per-pixel accurate alignment.

### 8.x.3	Service Flows

The following service flow is considered:

- Immersive video (RGB and depth) and audio feed is captured on user device.

- The stream is partially compressed on device and offloaded over a 6G network to the cloud. The tradeoff depends on the level of details, latency, and accuracy.

- In the cloud, a shared virtual scene is generated and pre-rendered.

- The virtual scene potentially including coordinates, audio, video, and depth, is transmitted to user device.

### 8.x.4	Post-conditions

Users have an enjoyable gaming experience.

### 8.x.5	Existing features partly or fully covering the use case functionality

Remote or split rendering for gaming applications, such as rendering of 360-degree video tiles in the cloud and transmitting the images to device, is already a well-known concept.

Media Codecs, such as Versatile video coding (VCC), are commonly available and considered for the compression of uplink and downlink images.

In terms of network support, Time Critical Communications (TCC) and L4S are supported on 5G networks. In TS 22.156 Mobile Metaverse services requirement for association and coordination for objects related to a session.

In table 7.1.1 TS 22.261 requirements for interactive audio and video service is defined for indoor/hotspot scenario in note 3 “For interactive audio and video services, for example, virtual meetings, the required two-way end-to-end latency (UL and DL) is 2-4ms while the corresponding experienced data rate needs to be up to 8K 3D video [300Mbit/s] in uplink and downlink”.

### 8.x.6	Potential New Requirements needed to support the use case

The following KPIs should be supported simultaneously in Urban and Dense Urban and indoor/outdoor scenario:

| Profile | Characteristic parameter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate down link | Bit rate up link | Application latency | Communication service availability | Overall user density | Activity factor | UE speed |
| Mixed Reality gaming | [50] Mbit/s Note 1 | [50-100] Mbit/s Note 3 | [20] ms Note 2 | [99]% | [1000-10000]/km2 (Urban) [2500-25000]/km2 (Dense Urban) | 20% | 5km/h |
| Note 1: Remote or split rendering for gaming applications, such as rendering of 360-degree video tiles in the cloud and transmitting the images to device, Note 2: The local encoding and processing on the glasses is assumed Note 3: envisioned 4K resolution |  |  |  |  |  |  |  |