SA WG2 Meeting #166	S2-2411277

18 - 22 November, 2024, Orlando, USA

SA WG2 Meeting #165	S2-2409569

14 - 18 October, 2024, Hyderabad, India

SA WG2 Meeting #S2#164	S2-2407421

19 - 23 August, Maastricht, Netherlands

3GPP TSG-CT WG1 Meeting #149	C1-243693

# Hyderabad, India, 27-31 May 2024

# Title:	LS on local BDC setup on terminating side in case INVITE does not contain DC description

# Response to:	-

# Release:	Rel-18

# Work Item:	NG_RTC

Source:	CT1

To:	SA2

Cc:	CT4

Contact Person:

Name:	Xu Chen

Tel. Number:

E-mail Address:	chenxu@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT1 has discussed a use case that terminating network provides autodownload/autorun P2A DC applications (e.g. translation/transcription/subtitle joined) to terminating UE via local BDC and ADC, and could not reach a consensus on whether the terminating IMS AS can notify the DCSF based on the operator policy, when the SDP offer within an initial INVITE from the originating network does not contain DC description and the terminating UE has IMS DC capability and subscription.

CT1 would like to ask SA2 the following questions:

Q1: Is the presence of DC media description in the SDP offer of an initial INVITE request from the originating network mandatory condition for the terminating IMS AS to notify the DCSF?

Q2: If the answer on Q1 is No, the SDP offer in an initial INVITE request from the originating network does not contain DC media description and the terminating UE has IMS DC capability and subscription, can the terminating IMS AS notify the DCSF based on the operator policy?

Q3: If the answer on Q1 is No and Q2 is Yes, after the terminating IMS AS notifies the DCSF, is the DCSF allowed to instruct the IMS AS to set up bootstrap data channel between the terminating network and terminating UE if the SDP offer of an initial INVITE request from the originating network does not contain DC media description?

2. Actions:

To SA2

ACTION: 	CT1 kindly asks SA2 to provide answers to the above questions.

3. Date of Next CT1 Meetings:

CT1#150                           19th August- 23rd August 2024..............Maastricht, NL

CT1#151                           14th October- 18th  October 2024..............China, CN

CT1#152                           18th November- 22nd  October 2024..............Orlando, US