SA WG2 Meeting #S2-166	S2-2411288

18 - 22 November, 2024, Orlando, USA

3GPP TSG-CT WG1 Meeting #151	C1-246002

# Hefei, China, 14-18 October 2024

# Title:	LS on the supporting 5G ProSe multi-hop Relays

# Response to:	N/A

# Release:	Rel-19

# Work Item:	5G_ProSe_Ph3

Source:	CT1

To:	SA2

Cc:	CT3, CT4

Contact Person:

Name:	Xiaoxue Zhao

E-mail Address:	zhaoxiaoxue@cictmobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	N/A

1. Overall Description:

While working on the implementation of CR0445 (S2-2409086) which enhances the 5G ProSe Capability for multi-hop Relays, CT1 has observed that the CR has introduced 5 new 5G Prose Capabilities (only for layer-3 case) which will be added as part of the “5GMM capability IE” in CT1 specification TS 24.501. In version 19.0.0 of TS 24.501, there are currently only 33 bits that are spare in 5GMM capability IE and that can be used for any newly introduced capabilities. Also currently in 5GMM capability IE there are 10 bits defined only for 5G ProSe due to ProSe capabilities defined in Rel-17 and Rel-18 (i.e. due to the Rel-17 and Rel-18 capability requirements that can be found in clause 6.6.2 in TS 23.304). Hence given the new Rel-19 ProSe capability requirements, the capability bits in 5GMM capability IE needed only for ProSe would be in total around 19 bits which is quite huge for only one feature.

Hence CT1 would like to kindly ask SA2 to consider whether the Rel-19 multi-hop relay 5G ProSe capabilities can be associated with the existing pre-Rel-19 5G Prose capabilities, for example whether a UE supporting the capability of "5G ProSe Layer-3 UE-to-Network Relay" would also mean that UE can support "5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay" OR "5G ProSe Layer-3 Intermediate UE-to-Network Relay"…and so on.

Similarly, a similar issue exists for the new Rel-19 policies for the procedure of UE Policy Provisioning Request in UE Policy Container as defined in CR0468 (S2-2409393), where 5 new policies have been defined. CT1 would like to kindly ask SA2 to consider whether the Rel-19 multi-hop relay 5G ProSe Requested UE policies bits can be associated with the existing pre-Rel-19 5G Prose Requested UE policies bits.

Additionally, based on the statement in clause 6.5.14 of TS 23.304: "The Intermediate Relay act as DHCP proxy or relay the IP allocation message (e.g., Router Solicitation, Advertisement)", it is unclear for CT1 whether "DHCP proxy" refers to "DHCP relay agent" which is defined by specific IETF RFCs or else, since "DHCP proxy" is not a DHCP functionality defined by IETF.

In summary, CT1 kindly ask SA2 to provide answers to the following questions:

Q1- Can the Rel-19 multi-hop relay 5G ProSe capabilities be associated with the existing pre-Rel-19 5G Prose capabilities?

Q2- Can the Rel-19 multi-hop relay 5G ProSe Requested UE policies bits be associated with the existing pre-Rel-19 5G Prose Requested UE policies?

Q3- Does "DHCP proxy" refer to DHCP relay agent as defined in related IETF RFCs, e.g. RFC 3046?

2. Actions:

To SA2 group:

ACTION:  CT1 kindly asks SA2 to provide answers to the questions above and to perform any corresponding needed updates in their specifications, if any.

3. Date of Next CT1 Meetings:

CT1#152                           18th November- 22nd November 2024..............Orlando, US