SA WG2 Meeting #166	S2-2411321

# 14 - 18 November 2024, Orlando, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1269 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Terminology and high level description alignment of VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As outlined in S2-2411194, we have reached an agreement on the NWDAF initiated VFL training process procedures, this CR proposes to update the high-level description according to the agreement. Additionally, since the agreed NWDAF initiated training process includes terminology "label", this CR proposes to incorporate the agreed definition of "label" from the TR into the definitions clause. Furthermore, this CR introduces the agreed definitions of VFL and HFL from TR into the definition clause. In line with the agreement made in S2-2411193 to replace "FL" with "HFL," this CR addresses the alignment changes. Note that S2-2411193 has already implemented a small number of replacements from "FL" to "HFL." This CR keeps ‘FL capability’ naming as suggested by Nokia to avoid backward compatibility problem. This CR also proposes to delete the EN below: Editor's note: For an NWDAF impacts of the split into AnLF and MTLF are FFS. In corresponding of the deletion, a note is added: NOTE Y: To support VFL training and VFL inference, the NWDAF act as VFL server and/or VFL client shall contain both MTLF and AnLF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce VFL related terminology definitions. Replace FL to HFL among TS 23.288 in Rel-19. Refine clause 5.3 and clause 5.4 to align with agreed NWDAF-initiated VFL training procedure and adding 2 notes. One EN deletion and add a note to clarify the situation. Editorial modification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 5.3, 5.4, 6.2A.0, 6.2A.1, 6.2A.3, 6.2C, 6.2C.1, 6.2C.2.1, 6.2C.2.2, 6.2C.2.3, 7.10.1, 7.10.2, 7.10.3, 7.10.4, 7.11.1, 7.11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4]. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Analytics Accuracy Information: Represent a performance measure of an analytics ID provided by an NWDAF containing AnLF, which is composed of the number of correct predictions of the analytics ID out of all predictions and the corresponding number of samples.

ML Model Accuracy Information: Represent a performance measure of a ML Model provided by an NWDAF containing MTLF, which is composed of the number of correct predictions by the ML Model out of all predictions and the corresponding number of samples.

Analytics Feedback Information: Indicates that the consumer NF has taken action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data.

Vertical Federated Learning (VFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs).

Horizontal Federated Learning (HFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs).

Label: A label is the training objective in supervised machine learning.

* * * Next Change * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AI/ML	Artificial Intelligence/Machine Learning

DCAF	Data Collection Application Function

HFL	Horizontal Federated Learning

RE-NWDAF	Roaming Exchange Network Data Analytics Function

VFL	Vertical Federated Learning

* * * Next change* * *

5.3	Horizontal Federated Learning (FL) among multiple NWDAFs

Horizontal Federated learning among multiple NWDAFs is a machine learning technique in core network that trains an ML Model across multiple decentralized entities holding local data set, without exchanging/sharing local data set. This approach stands in contrast to centralized machine learning techniques where all the local datasets are uploaded to one server, thus allowing to address critical issues such as data privacy, data security, data access rights.

NOTE 1:	Horizontal Federated Learning is supported among multiple NWDAFs, which means the local data set in different HFL client NWDAFs have the same feature space for different samples (e.g. UE IDs).

For Horizontal Federated Learning supported by multiple NWDAFs containing MTLF, there is one NWDAF containing MTLF acting as HFL server (called HFL server NWDAF for short) and multiple NWDAFs containing MTLF acting as HFL client (called HFL client NWDAF for short), the main functionality includes:

HFL server NWDAF:

-	discovers and selects HFL client NWDAFs to participant in an HFL procedure

-	requests HFL client NWDAFs to do local model training and to report local model information.

-	generates global ML Model by aggregating local model information from HFL client NWDAFs.

-	sends the global ML Model back to HFL client NWDAFs to perform an additional training iteration if needed.

HFL client NWDAF:

-	locally trains ML Model as tasked by the HFL server NWDAF with the available local data set, which includes the data that may not be allowed to be shared with other HFL client NWDAFs due to e.g. data privacy, data security, data access rights.

-	reports the trained local ML Model information to the HFL server NWDAF.

-	receives the global ML Model from HFL server NWDAF and perform an additional training iteration if needed.

HFL server NWDAF or HFL client NWDAF register to NRF with their FL capability information as described in clause 5.2.

The NWDAF containing MTLF determines to train an ML Model either based on local configuration or when it receives a request from NWDAF containing AnLF. The NWDAF containing MTLF further determines whether the ML Model should be trained via HFL mechanism based on Analytic ID, Service Area/DNAI or when data can not be obtained directly from data producer NF (e.g. due to data privacy, data security). The NWDAF containing AnLF is not aware whether the ML Model is trained based on HFL or not.

If the NWDAF containing MTLF can act as an HFL server for the ML Model training, then HFL procedure is initiated by the NWDAF containing MTLF as HFL server NWDAF directly.

If the NWDAF containing MTLF determines to train an ML Model based on local configuration and the HFL mechanism is required, but the NWDAF containing MTLF can't act as an HFL server, the NWDAF containing MTLF should discover an HFL server NWDAF as described in clause 5.2 and request the HFL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The HFL server NWDAF may determine to initiate HFL procedure before providing the ML Model.

If the ML Model training is triggered by the request from NWDAF containing AnLF, the NWDAF containing MTLF determines the HFL mechanism is required but it can not act as an HFL server, the NWDAF containing MTLF should discover an HFL server NWDAF as described in clause 5.2 and request the HFL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The Notification Target Address and the Notification Correlation ID from the NWDAF containing AnLF is provided in the request message sent to the HFL server NWDAF. The HFL server NWDAF may determine to initiate HFL procedure before providing the ML Model. The HFL server NWDAF sends the ML Model information to the notification endpoint (e.g. the NWDAF containing AnLF) after the ML Model training success.

NOTE 2:	The security procedure on authorizating HFL server to initiate HFL procedure on the HFL client(s) is described in Annex X, clause X.9 of TS 33.501 [49]. The security procedure authorizing an MTLF to request ML Models on behalf of an AnLF to another MTLF (e.g., HFL server NWDAF) is described in Annex X, clause X.10 of TS 33.501 [49].

Before HFL procedure is initiated by HFL server NWDAF, appropriate HFL client NWDAFs should be discovered by HFL server NWDAF as described in clause 5.2.

When starting an HFL procedure, the HFL server NWDAF is to provide an initial model to each HFL client NWDAF, and then each HFL client NWDAF is to perform local model training using its local data set. The detailed procedure for HFL among Multiple NWDAFs is described in clause 6.2C.

* * * Next change* * *

5.4	Vertical Federated Learning (VFL)

Vertical Federated learning is a machine learning techinique without exchanging/sharing local data set, while maintaining some level of coordination amongst VFL participants, when training and inference are performed on local ML Models, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). Vertical Federated Learning may involve multiple NWDAFs and AF.

For Vertical Federated Learning, there may be one NWDAF or one AF acting as a VFL server and one or multiple NWDAF(s) and/or one or multiple AF(s) acting as VFL Client(s). Vertical Federated Learning is available among NWDAFs within a single PLMN or between an AF and NWDAF(s) in a single PLMN.

The main functionalities of VFL server and VFL client include:

VFL server:

-	An NWDAF acting as VFL server discovers and selects VFL client(s) (NWDAF(s) and/or AF(s)) to participate in a VFL training procedure as described in clause 6.2H.2.1.

Editor's note:	Support for AF acting as VFL server selecting multiple NWDAFs is FFS.

-	requests VFL clients to do local ML model training for an Analytic ID, it assigns VFL correlation ID, and it requests to report intermediate training results.

-	aggregates intermediate training results from VFL client(s) and computes gradient information or loss information for updating its own local ML Model and the ML Models of VFL clients during the VFL training process and sends the computed gradient information or loss information towards VFL clients involved in the joint VFL training process.

-	Evaluates and decides to terminate the VFL training process; sends VFL training termination message to VFL client.

-	Stores VFL correlation ID and latest information of locally trained model after training process.

-	It initiates the VFL inference process using VFL correlation ID that was stored in VFL training process.

-	It aggregates local inference result from VFL clients and generates the final VFL inference result.

-	It may send the final VFL inference result to the consumer when receiving analytics request from consumer.

NOTE X: Only one VFL server shall exist for each VFL process.

VFL client:

-	locally trains ML Model with the available local data set, which includes the data that may not be allowed to be shared with other VFL clients and VFL server due to e.g. data privacy, data security, data access rights.

-	computes the intermediate training results for their local ML Models involved in the VFL training and provide reports with the intermediate training results to the AF or NWDAF acting as VFL server.

-	Stores VFL correlation ID and latest information of locally trained model after training process.

-	performs inference based on the stored local model, local data, VFL correlation ID and provides inference results to VFL server.

NOTE Y: To support VFL training and VFL inference, the NWDAF act as VFL server and/or VFL client shall contain both MTLF and AnLF.

Editor's note:	Details regarding Sample alignment and features alignment functionality or whether the functionality needs to be specified are FFS.

Editor's note:	Accuracy monitoring in VFL is FFS.

* * * Next change* * *

6.2A.0	General

This clause presents the procedure for the ML Model provisioning.

An NWDAF containing AnLF may be locally configured with (a set of) IDs of NWDAFs containing MTLF and the Analytics ID(s) supported by each NWDAF containing MTLF to retrieve trained ML Models or may use the NWDAF discovery procedure specified in clause 5.2 for discovering NWDAFs containing MTLF. An NWDAF containing MTLF may determine that further training for an existing ML Model is needed when it receives the ML Model subscription or the ML Model request.

A NWDAF containing MTLF may retrieve trained ML Models from other NWDAF containing MTLF as described in clause 5.3. The NWDAF containing MTLF determines to train a ML Model either based on the request from NWDAF containing AnLF, or based on local configuration. The NWDAF containing MTLF further determines the HFL procedure is required but it can not act as an HFL server, therefore, the NWDAF containing MTLF should discover an HFL server NWDAF as described in clause 5.2 and request the ML Model provisioning from the HFL server NWDAF. How to protect the trained ML Model file from being used without authorization e.g. being forwarded by an NWDAF containing MTLF that has retrieved a model from another NWDAF containing MTLF is defined in Annex X, clause X.10 of TS 33.501  [49].

* * * Next change* * *

6.2A.1	ML Model Subscribe/Unsubscribe

The procedure in Figure 6.2A.1-1 is used by an NWDAF service consumer, i.e.:

-	an NWDAF containing AnLF to subscribe/unsubscribe at another NWDAF, i.e. an NWDAF containing MTLF;

-	or an NWDAF containing MTLF to subscribe/unsubscribe at another NWDAF containing MTLF, i.e. an HFL server NWDAF;

-	or an LMF to subscribe/unsubscribe at an NWDAF containing MTLF,

to be notified when ML Model Information related to the ML Model subscription becomes available, using Nnwdaf_MLModelProvision services as defined in clause 7.5. When the service consumer is an NWDAF containing AnLF, the ML Model Information is used by the NWDAF containing AnLF to derive analytics. When the service consumer is an LMF, how the ML Model Information is used by the LMF is described in TS 23.273 [39]. The service is also used by an NWDAF Service Consumer to modify existing ML Model Subscription(s). An NWDAF can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.1-1: ML Model for analytics subscribe/unsubscribe

1.	The NWDAF service consumer subscribes to, modifies, or cancels subscription for a (set of) trained ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation.

Editor's note:	When the service consumer is LMF, whether to use a specific analytics ID or a new indication to indicate to retrieve ML model for LMF based AI/ML positioning is FFS.

The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2A.2. The service consumer optionally indicates its support for multiple ML Models if available.

When a ML Model subscription is received, the NWDAF containing MTLF may:

-	determine whether existing trained ML Model(s) can be used for the subscription; or

-	determine whether triggering further training for the existing trained ML Models is needed for the subscription.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from NFs, (e.g. AMF/DCCF/ADRF), UE Application (via AF) or OAM as described in clause 6.2, to generate the ML Model.

If the service invocation is for a subscription modification or subscription cancelation, the NWDAF service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If the NWDAF service consumer subscribes to a (set of) trained ML Model(s) associated to a (set of) Analytics ID(s), the NWDAF containing MTLF notifies the NWDAF service consumer by invoking Nnwdaf_MLModelProvision_Notify service operation with:

-	For each Analytics ID requested by the service consumer, a set of pair(s) of unique ML Model identifier and the associated Information.

NOTE 1:	The structure and format of the ML Model identifier and its uniqueness are up to stage 3.

NOTE 2:	Parameters defined for Multiple ML Models are for Analytics accuracy enhancement.

The content of trained ML Model Information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

The NWDAF containing MTLF also invokes the Nnwdaf_MLModelProvision_Notify service operation to notify an available re-trained ML Model when the NWDAF containing MTLF determines that the previously provided trained ML Model required re-training at step 1.

When step 1 is for a subscription modification (i.e. including Subscription Correlation ID), the NWDAF containing MTLF may provide either a new trained ML Model different to the previously provided one, or a re-trained ML Model by invoking Nnwdaf_MLModelProvision_Notify service operation.

When the content includes ML Model provide indicator, the NWDAF service consumer (i.e. an NWDAF containing AnLF or an NWDAF containing MTLF) may triggers ML Model retrieval procedure in clause 6.2B.7 to retrieve ML Model(s) from ADRF.

* * * Next change* * *

6.2A.3	ML Model request

The procedure in Figure 6.2A.3-1 is used by an NWDAF service consumer, i.e.:

-	an NWDAF containing AnLF to request and get from another NWDAF, i.e. an NWDAF containing MTLF ML Model Information;

-	or an NWDAF containing MTLF to request and get from another NWDAF containing MTLF worked as HFL server ML Model Information;

-	or an LMF to request and get from NWDAF containing MTLF ML Model Information,

using Nnwdaf_MLModelInfo services as defined in clause 7.6. When the service consumer is an NWDAF containing AnLF, the ML Model is used by the NWDAF containing AnLF to derive analytics. When the service consumer is an LMF, the ML Model Information is used by the LMF as described in TS 23.273 [39]. An NWDAF can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.3-1: ML Model Request

1.	The NWDAF service consumer requests a (set of) ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking Nnwdaf_MLModelInfo_Request service operation. The parameters that can be provided by the NWDAF Service Consumer are listed in clause 6.2A.2. The service consumer optionally indicates its support for multiple ML Models if available.

Whe an ML Model Information request is received, the NWDAF containing MTLF may:

-	determine whether existing trained ML Model(s) can be used for the request; or

-	determine whether triggering further training for the existing trained ML Models is needed for the request.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from NFs, (e.g. AMF/DCCF/ADRF), UE Application (via AF) or OAM as described in clause 6.2, to generate the ML Model.

2.	The NWDAF containing MTLF responds to the NWDAF service consumer by invoking Nnwdaf_MLModelInfo_Request response service operation including:

-	a set of pair(s) of unique ML Model identifier, the ML Model Information for each Analytics ID that the NWDAF service consumer requests.

* * * Next change* * *

6.2C	Horizontal Federated Learning among Multiple NWDAFs

6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Horizontal Federated Learning (HFL) technique to train an ML Model.

* * * Next change* * *

6.2C.2.1	Registration and Discovery procedure for Horizontal Federated Learning

Figure 6.2C.2.1-1: Registration and Discovery procedure for Horizontal Federated Learning

Steps 1 to 3 are the NWDAF registration procedure.

1-3.	NWDAF containing MTLF as HFL Server NWDAF or HFL Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability type information (i.e. HFL server and/or HFL client) and Time interval supporting HFL as described in clause 5.2.

Steps 4 to 6 are the NWDAF Discovery procedure.

4-6.	NWDAF containing MTLF determines ML Model requires HFL based on operator policy (e.g. pre-configured list of ML Models), Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as HFL Server NWDAF, the MTLF first discovers and selects HFL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML Model required, Model filter information as defined in clause 6.2A.2, FL capability Type (i.e. HFL server), Time Period of Interest, Service Area.

Once the HFL Server NWDAF (the requested or the selected one) is determined, the HFL Server NWDAF discovers and selects other NWDAF(s) containing MTLF as HFL Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML Model required, FL capability Type (i.e. HFL client), Service Area, NF type(s) of data sources from which the HFL Client NWDAF is able to collect data for local ML Model training, Time Period of Interest, ML Model Interoperability Indicator.

7.	HFL Server NWDAF sends Horizontal Federated Learning preparation request to the FL Client NWDAF(s), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag, to check if the HFL Client NWDAF(s) can meet the ML Model training requirement (e.g. Analytics ID, ML Model Interoperability information, Data Availability requirement, HFL Availability time requirement (time span needed for the HFL process), etc.). Data Availability requirement includes a list of Event IDs of the local data for training, and may also include the dataset statistical properties, the time window of the data samples and the minimum number of data samples.

NOTE:	Horizontal Federated Learning preparation procedure (i.e. steps 7-9) can be skipped if the HFL Server NWDAF can decide that the HFL Client NWDAF(s) supports the HFL procedure to be performed, e.g. based on information acquired from previous HFL procedures or from the NRF, or based on local configuration.

8.	HFL Client NWDAF(s) checks if it can meet the ML Model training requirement and/or can successfully download the model if the model information is provided in the request and decides whether to join the Horizontal Federated Learning process based on operator policy (e.g. pre-configured list of ML Models) and/or implementation. Example criteria used by HFL Client NWDAF(s) may be based on its data availability and time availability, computation and communication capability and ML Model Interoperability information.

9.	FL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate to the HFL Server NWDAF whether it will join the HFL procedure and may include the reason in the response message if it cannot join the HFL process.

10.	FL Server NWDAF determines the HFL Client NWDAFs to be involved in the HFL procedures based on the information received in step 6 and other information received in step 9 (if available).

* * * Next change* * *

6.2C.2.2	General procedure for Horizontal Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.2-1: General procedure for Horizontal Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF or NWDAF containing MTLF) sends a subscription request to HFL server NWDAF to retrieve an ML Model, using Nnwdaf_MLModelProvision including Analytics ID, ML Model Monitoring information as defined in clause 7.5.2, desired ML Model metric (e.g. ML Model Accuracy).

NOTE 1:	The ML Model Accuracy threshold can be used to indicate the target ML Model Accuracy of the training process and the HFL server NWDAF may stop the training process when the ML Model Accuracy threshold is achieved during the training process.

If the consumer (i.e. the NWDAF containing AnLF or NWDAF containing MTLF) provides the Time when the ML Model is needed, the HFL Server NWDAF can take this information into account to decide the maximum response time for its FL Client NWDAF(s).

1.	HFL Server NWDAF selects NWDAF(s) containing MTLF (HFL Client NWDAF(s)) as described in clause 6.2C.2.1.

2.	HFL Server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF(s) containing MTLF (HFL Client NWDAF(s)), which participate in the Horizontal Federated learning to perform the local model training and determine the interim local ML Model information based on the input parameter in the request from HFL Server NWDAF. The request includes the desired ML Model metric and initial ML Model and also includes the maximum response time, the HFL Client NWDAF has to report the interim local ML Model information to the HFL Server NWDAF before the maximum response time elapses.

3.	[Optional] Each HFL Client NWDAF collects its local data by using the current mechanism in clause 6.2 if the Client NWDAF has not local data available already.

4.	During Horizontal Federated Learning training procedure, each HFL Client NWDAF further trains the ML Model provided by the HFL Server NWDAF based on its local data and reports the interim local ML Model information to the HFL Server NWDAF in Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response. The Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response may also include the Status report of HFL training that includes local ML Model metric value (and optionally the used metric) computed by the HFL Client NWDAF and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension of data, etc.) in the HFL Client NWDAF. The Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Response also includes the global ML Model metric value (and optionally the used metric) when the ML Model Accuracy Check Flag was included in the Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request (as described in step 7), the global ML Model metric value is calculated by the HFL Client NWDAF using the local training data as the testing dataset.

NOTE 2:	The parameters in characteristics of local training dataset are up to the implementation.

The local ML Model, which is sent from the HFL Client NWDAF(s) to the HFL Server NWDAF during the HFL training process, is the information needed by the HFL Server NWDAF to build the aggregated model.

If the HFL Client NWDAF is not able to complete the training of the interim local ML Model within the maximum response time provided by the HFL Server NWDAF, the HFL Client NWDAF shall send the Delay Event Notification that include the delay event indication, an optional cause code (e.g. local ML Model training failure, more time necessary for local ML Model training) and the expected time to complete the training if available to the HFL Server NWDAF before the maximum response time elapses.

4a.	[Optional]If HFL Server NWDAF receives notification/response that the HFL Client NWDAF is not able to complete the training within the maximum response time, the HFL Server NWDAF may send to the HFL Client NWDAF a new maximum response time in Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request, before which the HFL Client NWDAF has to report the interim local ML Model information to the HFL Server NWDAF. Otherwise, the HFL Server NWDAF may indicate HFL Client NWDAF to skip reporting for this iteration. HFL Server NWDAF includes the current iteration round ID in the message to indicate that the request is to modify the training parameters of the current iteration round.

Alternatively, the HFL Server NWDAF may inform the HFL Client NWDAF to cease the ML Model training by sending termination request and to report back the current local ML Model updates.

5.	The HFL Server NWDAF aggregates all the local ML Model information retrieved at step 4, to update the global ML Model. The HFL Server NWDAF may also compute the global ML Model metric value, e.g. based on the local ML Model metric value(s) provided by the HFL Client NWDAF(s) or by applying the global model on the validation dataset (if available). The HFL Server NWDAF may update the global ML Model each time a HFL Client NWDAF provides updated local ML Model information, or the HFL Server NWDAF may decide to wait for local ML Model information from all HFL Client NWDAFs before updating the global ML Model.

If the HFL Server NWDAF provides the maximum response time for the HFL Client NWDAF(s) to provide the interim local ML Model information in step 2, or the new maximum response time in step 4a, the HFL Server NWDAF decides either to wait for the HFL Client NWDAF(s) which have not yet provided their interim local ML Model within the new maximum response time or to aggregate only the retrieved local ML Model information instances to update global ML Model. The HFL Server NWDAF makes this decision, considering the notification/response from the HFL Client NWDAF or, if the notification is not received, based on local configuration.

6a.	[Optional] Based on the consumer request in step 0, the HFL Server NWDAF sends a Nnwdaf_MLModelProvision_Notify message to update the global ML Model metric value to the consumer periodically (e.g. a certain number of training rounds or every 10 min) or dynamically when some pre-determined status is achieved (e.g. the ML Model Accuracy threshold is achieved or training time expires).

6b.	[Optional] The consumer decides whether the current model can fulfil the requirement, e.g. global ML Model metric value is satisfactory for the consumer and determines to stop or continue the training process. The consumer re-invokes Nnwdaf_MLModelProvision_Subscribe service operation as used in step 0 to continue the training process or invokes Nnwdaf_MLModelProvision_Unsubscribe service operation to stop the training process.

6c.	[Optional] Based on the subscription request sent from the consumer in step 6b, the HFL Server NWDAF updates or terminates the current HFL training process.

If the HFL Server NWDAF received a request in step 6b to stop the Horizontal Federated Training process, steps 7 and 8 are skipped.

7.	If the HFL procedure continues, HFL Server NWDAF may determine HFL Client NWDAF as described in clause 6.2C.2.3 and sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request that includes the aggregated ML Model information to selected HFL Client NWDAF(s) for next round of Horizontal Federated Training. The request may also include the ML Model Accuracy Check Flag, that indicates the HFL Client NWDAF(s) to use the local training data as the testing dataset to calculate the Model Accuracy of the global ML Model provided by the HFL Server NWDAF.

8.	Each HFL Client NWDAF updates its own ML Model based on the aggregated ML Model information distributed by the HFL Server NWDAF at step 7.

NOTE 3:	The steps 3-8 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

When the Horizontal Federated Training procedure is complete, the HFL Server NWDAF requests the HFL client NWDAF(s) to terminate the HFL procedure by invoking Nnwdaf_MLModelTraining_Unsubscribe service with a cause code that the HFL process has finished and optionally with the final aggregated ML Model information. Then the HFL client NWDAF(s) terminate the local model training and if the final aggregated ML Model information is received from the HFL server NWDAF, the HFL client NWDAF(s) can store it for further use.

9.	After the training process is complete, the HFL Server NWDAF may send Nnwdaf_MLModelProvision_Notify that includes the globally optimal ML Model information to the consumer.

* * * Next change* * *

6.2C.2.3	Procedures for Maintaining Horizontal Federated Learning Processes

This clause specifies how to maintain a Horizontal Federation Learning process in HFL execution phase, including HFL Server NWDAF triggers reselection, addition, or removal of HFL Client NWDAF(s), discovers new HFL Client NWDAF(s) via NRF and HFL Client NWDAF(s) joins or leaves Horizontal Federated Learning process dynamically.

In Horizontal Federated Learning execution phase, HFL Server NWDAF monitors the status changes of HFL Client NWDAF(s) and may reselects HFL Client NWDAF(s) based on the updated status, availability and/or capability, etc.

NOTE 1:	HFL Server NWDAF checks if there is a need to carry on the HFL execution phase and then reselects HFL members for the next iteration if needed.

Figure 6.2C.2.3-1: Procedure of HFL Server NWDAF reselects HFL Client NWDAF(s), HFL Client NWDAF(s) Join or Leave Horizontal Federated Learning Process Dynamically in Horizontal Federated Learning execution phase

The procedure for HFL Server NWDAF reselecting HFL Client NWDAF(s), HFL Client NWDAF(s) joining or leaving Horizontal Federated Learning process dynamically is as follows:

1a.	HFL Server NWDAF may get the updated status of current HFL Client NWDAF(s) via NRF by using Nnrf_NFManagement service (as in clause 5.2.7.2 of TS 23.502 [3]) in the Horizontal Federated Learning execution phase.

HFL Server NWDAF may subscribe to NRF for notifications of status changes of the current NWDAF(s) (HFL Client NWDAFs 1…N) by invoking an Nnrf_NFManagement_NFStatusSubscribe service operation. NRF notifies the HFL Server NWDAF the status changes of the current HFL Client NWDAF(s) by invoking Nnrf_NFManagement_NFStatusNotify service operation(s).

The status of a current HFL Client NWDAF could be availability changes, capability changes (e.g. it will not support HFL anymore, etc.).

1b.	The current HFL Client NWDAF(s) may inform HFL Server NWDAF that it is leaving the Horizontal Federated Learning process by invoking Nnwdaf_MLModelTraining_Notify service operation with Termination Request and cause code (reason for leaving, e.g. high NF load, time availability changes).

1c.	HFL Server NWDAF may get the information of the new HFL Client NWDAF(s) dynamically via NRF by subscribing to the event that a new HFL Client NWDAF registers (Nnrf_NFManagement_NFStatusSubscribe service as in clause 5.2.7.2 of TS 23.502 [3]).

1d.	NWDAF may subscribe for NF load analytics of the HFL Client NWDAF(s).

1e.	HFL Client NWDAF(s) may send Status report of HFL training and Global ML Model Accuracy Information by invoking Nnwdaf_MLModelTraining_Notify service.

2.	HFL Server NWDAF checks HFL Client NWDAF(s) status based on the received information and may determine whether reselection of HFL Client NWDAF(s) for the next round(s) of Horizontal Federated Learning is needed based on the received information from step 1.

NOTE 2:	Several examples of the factors that the HFL Server NWDAF can consider to reselect the HFL Client NWDAF(s) are updated status of HFL Client NWDAF reported by NRF is different than the criteria were initially used for selecting the client; characteristics of local training dataset is different than global validation dataset owned by HFL Server NWDAF and/or the metric value of the global model calculated using the local training dataset is much different from that calculated by other HFL Client NWDAFs; the metric value of the global model calculated using the local training dataset is lower than the metric value calculated using the global validation dataset owned by HFL Server NWDAF; the metric value of the global model calculated using the local training dataset is lower than ML Model metric value received in Nnwdaf_MLModelMonitor_Notify when HFL Server NWDAF using AnLF-assisted MTLF ML Models Accuracy Monitoring; the load of the HFL Client NWDAF (from the NF load Analytics or from the HFL Client NWDAF directly) is high; the HFL Server NWDAF receives leave request from the HFL Client NWDAF; the HFL Client NWDAF did not report the status of HFL Training within the maximum response time.

3.	[If re-selection is needed as judged in step 2] If step 1c is not performed, HFL Server NWDAF may discover new candidate FL Client NWDAF(s) via NRF by using Nnrf_NFDiscovery services as in clause 5.2.7.3 of TS 23.502 [3]. HFL Server NWDAF reselects HFL Client NWDAF(s) from the current HFL Client NWDAF(s) and the new candidate HFL Client NWDAF(s) (found in steps 1c or 3). For the new candidate HFL Client NWDAF(s), the interaction between HFL Server NWDAF and HFL Client NWDAF(s) is same as the selection procedure described in clause 6.2C.2.1. The adding / deleting HFL Client NWDAF(s) may happen at the end of each iteration.

4.	HFL Server NWDAF sends termination request by invoking Nnwdaf_MLModelTraining_Unsubscribe service operation or Nnwdaf_MLModelTrainingInfo_Request service operation with Termination Request to the HFL Client NWDAF(s), optionally indicating the reason, e.g. HFL Client NWDAF is unselected by the HFL Server NWDAF for the HFL process, or the HFL process is suspended, etc. And HFL server may also send the updated global ML Model information to the unselected HFL client NWDAF. HFL Client NWDAF(s) terminates operations for the Horizontal Federated Learning process if receive termination request from the HFL Server NWDAF and may perform further action to be qualified in participation of HFL training in the next cycles.

NOTE 3:	In the case of high load, the HFL Client NWDAF can e.g. decline new service request. In the case of low accuracy, the HFL Client NWDAF can gather new local training data.

* * * Next change* * *

7.10.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe/notify/modify for ML Model training.

NOTE:	In this release of the specification, the service provider and consumer are limited to NWDAF containing MTLF.

When used for Horizontal Federated Learning, this service enables HFL server NWDAF to enable Horizontal Federated Learning while providing global ML Model information to HFL Client NWDAF and getting local ML Model information and status report of HFL training as defined in clause 6.2C.2.3 from the HFL Client NWDAF.

This service may also be used by the consumer (i.e. HFL Server NWDAF) to check if the service provider (i.e. HFL Client NWDAF) can meet the ML Model training requirement as described in clause 6.2F.1.

This service may also be used by the consumer (i.e. HFL Server NWDAF) to request the service provider (i.e. HFL Client NWDAF) to calculate and provide Model Accuracy of the global ML Model as described in clause 6.2F.1.

* * * Next change* * *

7.10.2	Nnwdaf_MLModelTraining_Subscribe service operation

Service operation name: Nnwdaf_MLModelTraining_Subscribe

Description: Subscribes to NWDAF ML Model training with specific parameters.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	ML Model Interoperability information;

-	Notification Target Address (+ Notification Correlation ID).

Inputs, Optional:

-	ML Model identifier: identifies the provided ML Model.

-	ML Model Information (as defined in clause 6.2A.2);

-	ML Model file;

-	Subscription Correlation ID (in the case of modification of the ML Model Training subscription);

-	ML Training Information, i.e. data availability requirement, time availability requirement.

-	ML Preparation Flag;

-	ML Model Accuracy Check Flag;

-	ML Correlation ID;

-	Training Filter Information;

-	Target of Training Reporting;

-	Training Reporting Information as defined in clause 6.2F.2;

-	Use case context;

-	Iteration round ID;

-	Expiry time.

-	Indication of skipping the current HFL round.

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the ML training requirements, ML training is not complete, NWDAF overload, not available for the HFL process anymore, etc.).

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: ML Correlation ID (e.g. confirm of the subscription for this HFL process).

* * * Next change* * *

7.10.3	Nnwdaf_MLModelTraining_Unsubscribe service operation

Service operation name: Nnwdaf_MLModelTraining_Unsubscribe

Description: Terminate NWDAF ML Model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code (e.g. HFL Client NWDAF is unselected by the HFL Server NWDAF for the HFL process, or the HFL process is suspended or finished, etc.). Final aggregated ML Model information (if HFL has finished) or updated aggregated ML Model information (if HFL is suspended).

* * * Next change* * *

7.10.4	Nnwdaf_MLModelTraining_Notify service operation

Service operation name: Nnwdaf_MLModelTraining_Notify

Description: NWDAF notifies the consumer instance of the trained ML Model that has subscribed to the specific NWDAF service. The NWDAF can also use this service to indicate to consumer it will terminate the ML Model training.

Inputs, Required:

-	Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during ML Model training.

Inputs, Optional:

-	Set of the tuple (Analytics ID, ML Model Information as defined in clause 6.2F.2;

-	ML Correlation ID, when for Horizontal Federated Learning;

-	Corresponding Use case context;

-	Termination Request: this parameter indicates that NWDAF requests to terminate the ML Model training, i.e. NWDAF will not provide further notifications related to this request, with cause code (e.g. NWDAF overload, not available for the HFL process anymore, etc.);

-	ML Model identifier: this parameter identifies the provisioned ML Model;

-	Global ML Model Accuracy information: The model accuracy of the global ML Model, which is calculate by the HFL Client NWDAF using the local training data as the testing dataset;

-	Status report of HFL training: local ML Model accuracy information and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.), which are generated by the HFL Client NWDAF during HFL procedure;

-	Delay Event Notification: as defined in clause 6.2F.2;

-	Iteration round ID.

NOTE:	The detail reasons in the cause code are up to stage 3.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * Next change* * *

7.11.1	General

Service Description: This service enables the consumer to request for the information about ML Model training based on the ML Model file or ML Model information as described in clause 6.2F.2 provided by the consumer.

NOTE:	In this release of the specification, the service provider and consumer are limited to NWDAF containing MTLF.

When used for Federated Learning, this service enables HFL server NWDAF to enable Horizontal Federated Learning while providing global ML Model information to HFL Client NWDAF and getting local ML Model information from the HFL Client NWDAF.

* * * Next change* * *

7.11.2	Nnwdaf_MLModelTrainingInfo_Request service operation

Service operation name: Nnwdaf_MLModelTrainingInfo_Request

Description: Request information about NWDAF ML Model training with specific parameters.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2.

-	ML Model Interoperability information.

Inputs, Optional:

-	ML Model identifier: identifies the provided ML Model.

-	ML Model Information (as defined in clause 6.2A.2).

-	ML Model file.

-	ML Training Information (i.e. data availability requirement, time availability requirement).

-	Training Reporting Information as defined in clause 6.2F.2.

-	ML Preparation Flag.

-	ML Model Accuracy Check Flag.

-	ML Correlation ID.

-	Termination Request, when terminating the Horizontal Federated Learning identified by the ML Correlation ID and optionally indicating the reason, e.g. HFL Client NWDAF is unselected by the HFL Server NWDAF for the HFL process, or the HFL process is suspended, etc.

-	Training Filter Information.

-	Target of Training Reporting.

-	Use case context.

-	Indication of skipping the current HFL round.

Outputs Required: When the request is accepted: Operation execution result indication. When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the ML training requirements, ML training is not complete, NWDAF overload, not available for the HFL process anymore, etc.).

NOTE:	The detail reasons in the cause code are up to stage 3.

Outputs, Optional:

-	ML Model identifier.

-	Set of the tuple (Analytics ID, ML Model Information (as defined in clause 6.2A.2)).

-	ML Correlation ID, when for Federated Learning.

-	Corresponding Use case context.

-	Global ML Model Accuracy information: The model accuracy of the global ML Model, which is calculate by the HFL Client NWDAF using the local training data as the testing dataset.

-	Status report of HFL training: local ML Model Accuracy Information and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension of data, etc.), which are generated by the HFL Client NWDAF during HFL procedure.

-	Delay Event Notification as defined in clause 6.2F.2.

* * * End of changes* * *