SA WG2 Meeting #S2-166	S2-2411371

18 - 22 November, 2024, Orlando, USA

Source:	Nokia

Title:	KI#2: VFL procedures with AF as server

Document for:	Approval

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

This contribution discusses VFL procedures with AF as server. Related changes are proposed in S2-2411372.

# Discussion

The following EN should be resolved:

Editor's note:	Support for AF acting as VFL server selecting multiple NWDAFs is FFS.

Editor's note	Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable.

There were also some specific key issue 2 conclusions for AF as VFL server in TR 23.700-84:

## 8.2	Conclusions for KI#2: 5GC Support for Vertical Federated Learning

P#2.1:  VFL related new functionalities include:

…

P#2.1.4 VFL process is associated with internal AF process (i.e. VFL training of models for internal AF process, VFL inference for internal AF process) when VFL server is AF.

…

P#2.2:  For registration and discovery of VFL entities:

…

P#2.2.3 The AF (via NEF in case of untrusted AF) as VFL server will select candidate NWDAF(s) as VFL client(s) from NRF for VFL training process.

…

P#2.3:  For sample alignment for VFL:

…

P#2.3.3: For AF acting as VFL Server and NWDAFs acting as VFL clients, AF performs sample alignment, selects samples to be used in the training process and generates the interclause of samples.

…

NOTE 4:	How NEF assists sample alignment in P#2.3.3 will be determined in normative phase.

NOTE 5:	For P#2.3.2 and P#2.3.3, whether and how NEF supports to be involved in generating the intersection of candidate samples is to be discussed in normative phase.

P#2.4:  For VFL training process:

P#2.4.1: Either the NWDAF or the AF can act as VFL server and initiate VFL training process with the VFL client(s).

P#2.4.2: If an untrusted AF is involved in the VFL training process, their interactions with the NWDAF(s) are via NEF. When the NWDAF acts as VFL Server, the NWDAF can receive labels from an AF.

…

P#2.5:  For VFL inference process:

…

P#2.5.2: For the AF acting as a VFL server, the AF acting as VFL server can also start an inference process with corresponding NWDAF acting as VFL client(s). The AF may be triggered to start the inference by a 5GC consumer (i.e. NWDAF containing AnLF). Interactions are via NEF if AF is untrusted.

…

NOTE 11:	How multiple NWDAFs are involved in VFL when AF is acting as VFL server is defined in normative phase.

Issue 3: How is sample alignment done when AF is VFL server?

It is conceivable to apply “normal” sample alignment procedures where the AF as server sends a list of candidate samples to each client, and each client replies with a reduced list, and the AF as server then calculates the intersection.

However, some caveats need to be observed: Mapping of external and Internal sample identifications, e.g. SUPI and GPSI as UE ID, or different description of an area. This is functionality that has already been assigned to the NEF for many APIs.

For UEs as samples, an important consideration is availability of UEs (possibly in an are of interest). The existing Nnef_MemberUESelectionAssistance service may enable an AF to select suitable UEs without interactions with NWDAF acting as VFL clients.

Proposal 5: The NEF supports sample alignment by proxying related messages and modifying the identifiers/description of samples between an external and internal format where required (e.g. Transforming GPSIs into SUPIs, and modifying description of an area.

Proposal 6: For UEs as samples, the AF can reuse the Nnef_MemberUESelectionAssistance service to check the availability of UEs.

Issue 4: Triggering of Inference procedure when AF is VFL server?

The NWDAF as server is triggered by requests for Analytics to perform a VFL inference procedure.

However, for an AF as VFL server there are no standardized procedures to request for analytics. The AF may however receive unstandardized external triggers or internal triggers to perform an inference procedure.

In addition, the following was concluded:

P#2.5.2: For the AF acting as a VFL server, the AF acting as VFL server can also start an inference process with corresponding NWDAF acting as VFL client(s). The AF may be triggered to start the inference by a 5GC consumer (i.e. NWDAF containing AnLF). Interactions are via NEF if AF is untrusted.

Thus, a procedure for an 5GC consumer (i.e. NWDAF containing AnLF) to trigger an inference procedure (and likely receive related results) needs to be discussed.

Due to the use case description in TR 23.700-84, it can be assumed that the network request would relate to a demand to obtain QoE estimated by the AF-

TS 23.288 already contains a data collection procedure from an untrusted AF based on the Naf_EventExposure service.

The defined events include “Service Experience information, as defined in clause 6.4.2 of TS 23.288 [50]”. Either a new event could be defined or this event could be reused for the purpose of triggering a VFL operation to derive QoE.

Proposal 7:  The AF as VFL server may be triggered by internal logic or the subscription to an event received via the Naf_EventExposure service from an NWDAF to perform a VFL inference procedure

Issue 5: Document procedures when AF is VFL server separately?

The issues discussed above demonstrate that there will be significant differences compared to the case where the NWDAF is a server. In particular, the role of the NEF and services at the NEF should be described.

Proposal 8: Document call flows when AF is VFL server separately from call flows when NWDAF is VFL server.

Summary of Proposals

Proposal 5: The NEF supports sample alignment by proxying related messages and modifying the identifiers/description of samples between an external and internal format where required (e.g. Transforming GPSIs into SUPIs, and modifying description of an area.

Proposal 6: For UEs as samples, the AF can reuse the Nnef_MemberUESelectionAssistance service to check the availability of UEs.

Proposal 7:  The AF as VFL server may be triggered by internal logic or the subscription to an event received via the Naf_EventExposure service from an NWDAF to perform a VFL inference procedure

Proposal 8: Document call flows when AF is VFL server separately from call flows when NWDAF is VFL server.

Annex: From TS 23.288: Data Collection from AF

#### 6.2.2.3	Procedure for Data Collection from AF via NEF

The procedure in Figure 6.2.2.3-1 is used by NWDAF to collect information from AFs via the NEF.

NOTE 1:	In this release, AF registers its available data to NWDAF via OAM configuration at NEF.

The AF collectable data information includes: AF identification, AF service identification (e.g. endpoint information of Naf_EventExposure), available data to be collected per application (e.g. identified by Event ID(s)).

Figure 6.2.2.3-1: Data Collection from AF via NEF

1a.	After the registration of AF available data at the NEF, NEF generates an event exposure with new EventID to be associated with available data to be collected from AF. NEF invokes Nnrf_NFManagement_NFUpdate_request service operation to update its registration information (i.e. NEF Profile) including the generated Event IDs and associated AF identification, Application ID(s) (i.e. internal application ID or Application ID known in the core network).

1b.	NRF stores the received NEF registration information including available data to be collected from AF.

1c.	NRF sends Nnrf_NFManagement_NFUpdate_response message to NEF.

1d.	When NWDAF needs to discovery the available data from AFs and the appropriated NEF to collect this data, NWDAF invokes Nnrf_NFDiscovery_Request_request service operation using as parameter the NEF NF Type and optionally a list of Event ID(s), AF identification and application ID.

1e.	NRF matches the requested query for available data in AFs with the registered NEF Profiles and sends this information via Nnrf_NFDiscovery_Request_response message to NWDAF.

NOTE 2:	After the registration and discovery procedure described in step 1, NWDAF identifies the available data per AF per application and the proper NEF to collect such data.

2.	The NWDAF subscribes to or cancels subscription to data in AF via NEF by invoking the Nnef_EventExposure_Subscribe or Nnef_EventExposure_Unsubscribe service operation. If the event subscription is authorized by the NEF, the NEF records the association of the event trigger and the NWDAF identity.

NOTE 3:	User consent for retrieving user data in AF via NEF is not specified in this Release.

3.	Based on the request from the NWDAF, the NEF subscribes to or cancels subscription to data in AF by invoking the Naf_EventExposure_Subscribe/ Naf_EventExposure_Unsubscribe service operation.

4.	If the NEF subscribes to data in AF, the AF notifies the NEF with the data by invoking Naf_EventExposure_Notify service operation according to Event Reporting Information in the subscription.

5.	If the NEF receives the notification from the AF, the NEF notifies the NWDAF with the data by invoking Nnef_EventExposure_Notify service operation.

When the Reporting type is provided at step 2, the NWDAF determines that the events are disappeared, if the same events are included in the notification compared to the previous notification. Otherwise, NWDAF determines the events are newly appeared or changed. Also, the NWDAF restores the events that are not included in the notification, but included in the previous notification.

If the Granularity of dynamics is applied to the subscription, the NWDAF shall infer the events in the AF from the events in the previous notification and the applied Granularity of dynamics.

From TS 23.288

#### 5.2.19.2	Naf_EventExposure service

##### 5.2.19.2.1	General

Service description: This service enables consumer NF to subscribe and get notified of events.

The events can be subscribed by a NF consumer are described in TS 23.288 [50].

The following service operations are defined for the Naf_EventExposure service:

-	Naf_EventExposure_Subscribe.

-	Naf_EventExposure_Unsubscribe.

-	Naf_EventExposure_Notify.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	Service Experience information, as defined in clause 6.4.2 of TS 23.288 [50].

-	Performance Data information, as defined in clauses 6.4.2 and clause 6.14.2 of TS 23.288 [50].

-	Collective Behaviour information, as defined in clause 6.5.2 of TS 23.288 [50].

-	UE Mobility information, as defined in clause 6.7.2.2 of TS 23.288 [50].

-	UE Communication information, as defined in clause 6.7.3.2 of TS 23.288 [50].

-	Exceptions information, as defined in clause 6.7.5.2 of TS 23.288 [50].

-	User Data Congestion information, as defined in clause 6.8.2 of TS 23.288 [50].

-	UE Data volume Dispersion information, as defined in clause 6.10.2 of TS 23.288 [50].

-	DN Performance information, as defined in clause 6.14.2 of TS 23.288 [50].

-	Global Navigation Satellite System (GNSS) UE location as defined in clause 6.17.2 of TS 23.288 [50].

-	E2E data volume transfer time information, as defined in clause 6.18.2 of TS 23.288 [50].

-	Relative Proximity information, as defined in clause 6.19.2 of TS 23.288 [50].

Event Filters are used to specify the conditions to match for notifying the event (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides some examples on how the conditions to match for event reporting can be specified for various Event IDs for AF exposure.

Table 5.2.19.2.1-1: Example of Event Filters for AF exposure events

| Event ID for AF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Exceptions information | <Parameter Type = Exception ID, Value = Exception ID1> |
| Service Experience information | <Parameter Type = TAI, Value = TAI1> |
| Service Experience information | <Parameter Type = geographical area, Value = civic address1 or shape1> |
| Collective Behaviour information | <Parameter Type = collective attribute, Value = collective value1> |
| Collective Behaviour information | <Parameter Type = data processing type, Value = data processing type1> |

Annex: From TS 23.502: Nnef_MemberUESelectionAssistance service

### 4.15.13	Assistance for Member UE selection

#### 4.15.13.0	General

Assistance for Member UE selection allows the AF to request the 5GC to filter a list of AF provided target member UEs according to AF provided criteria (Member UE filtering criteria) and to provide one or more list(s) of candidate UEs (and additional information) as response to the AF.

With the initial subscription request, the AF provides a list of target member UEs in the form of a list of GPSIs or a list of UE IP addresses and at least one Member UE filtering criterion as part of the service operation input parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve from 5GC NFs the information for the UEs in the list of target member UEs. Before sending the list(s) of candidate UEs to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the Member UE filtering criteria provided by the AF. The NEF finally provides the one or more list(s) of candidate UEs and possibly additional information to the AF together with a Subscription Correlation ID.

AF may subsequently update the parameters for the Member UE filtering criteria, the Member UE filtering criteria, or the Member UE filtering criteria values. In all cases, AF includes the Subscription Correlation ID in the request, but does not include the list of target member UEs. AF may request to update the parameters (e.g. expiry time, time window for selecting the candidate UEs) for the subscribed Member UE filtering criteria. AF may also request to update the Member UE filtering criteria by adding new Member UE filtering criteria or removing part of the subscribed Member UE filtering criteria. The AF may also request to update the Member UE filtering criteria values.

The Member UE selection assistance capability can be used to assist the AF to select the list of member UEs to support application service (e.g. FL operation) as described in clause 5.46.2 of TS 23.501 [2].

Additionally, AF may leverage the 5GC network exposure for Member UE selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how the AF leverages the 5GC network exposure for Member UE selection is described in (informative) Annex I.

AF may also delete the previous subscription for Member UE selection assistance as described in clause 4.15.13.7.

#### 4.15.13.1	Member UE selection assistance subscribe and update procedure

This clause describes the Member UE selection assistance subscribe and update procedure that is generally applicable independently of the Member UE filtering criteria sent by the AF.

Figure 4.15.13.1-1: Member UE selection assistance subscribe and update procedure

1.	AF subscribes the Member UE selection assistance by sending a Nnef_MemberUESelectionAssistance_subscribe request including a list of target member UEs, one or more member UE filtering criteria as listed in Table 4.15.13.2-1 and optionally, time window(s). Subsequently, the AF may only update the Member UE filtering criteria of the subscription as described in clause 4.15.13.0 by invoking Nnef_MemberUESelectionAssistance_subscribe and providing a Subscription Correlation ID, i.e. the AF does not provide the list of target member UEs again.

NOTE:	The time window(s) for selecting the candidate UE(s) is used by the NEF when subscribing/requesting to NWDAF. The NEF maps the time window(s) for selecting the candidate UE(s) to the Analytics target period, which should be included in the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations.

2a.	[CONDITIONAL] If the AF request does not contain a Subscription Correlation ID, the NEF verifies the authorization of the AF request and identifies which information needs to be collected for each UE in the list of target member UEs and executes the corresponding service operations based on the Member UE filtering criteria provided by the AF, e.g. events, analytics ID(s), notifications, etc.

2b.	[CONDITIONAL] If the AF request contains a Subscription Correlation ID, the NEF correlates the Nnef_MemberUESelectionAssistance_Subscribe request to an existing subscription according to the Subscription Correlation ID. The NEF updates the Member UE filtering criteria and/or their parameters (as described in clause 4.15.13.0) for the list of target member UEs received in initial subscription (i.e. no Subscription Correlation ID in the subscription request).

3.	NEF interacts with different 5GC network functions to collect the required information for each UE in the list of target member UEs. The set of interactions between the NEF and the 5GC NFs depend on the Member UE filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

4.	Based on the collected information from other 5GC NFs, the NEF consolidates all the information to derive the list(s) of candidate UEs which fulfil the Member UE filtering criteria in the AF request. The NEF may derive recommended time window(s), considering the validity period(s) of the analytics used for Member UE selection criteria. The recommended time window(s) may be a subset of the time window(s) received from the AF. In different recommended time windows, the list of candidate UEs which fulfil the Member UE filtering criteria may be different.

5.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UEs and possibly additional information. See clause 5.2.6.32.4 for details.

#### 4.15.13.2	Member UE Filtering Criteria for 5GS assistance to Member UE selection

Table 4.15.13.2-1 provides a summary of the Member UE filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of Member UE filtering criteria

| Member UE filtering criteria | Description of filtering criteria for member UEs selected by NEF | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the member UEs match or exceed the QoS of the filtering criteria. | NF service: Nsmf_EventExposure or Nudm_EvenExposure, Filter: target=SUPI, traffic descriptor (e.g. Application ID), DNN/S-NSSAI Event ID: QoS Monitoring | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the member UEs for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session). | NF service: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI, Event ID: Change of Access Type and/or Change of RAT Type | 5.2.8.3 |
| End-to-end data volume transfer time | Indicate the target end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE and AF, e.g. the average and variance of End-to-end data volume transfer time. | NF service: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Target = GPSI(s) or SUPI(s) Analytics ID: E2E data volume transfer time | 4.15.13.6 |
| UE current location | Indicate the certain area that the member UEs are currently located in. | NF service: Namf_EventExposure Filter: a list of GPSI(s) or SUPI(s) Event ID: Location Report | 4.15.13.4 |
| UE historical location | Indicate the certain area that the member UEs appeared in a historical period of time. | NF service: Nnwdaf_AnalyticsSubscription / Nnwdaf_AnalyticsInfo Filter: Visited AoI = Target AOI, target period = historical nomadic period Analytics ID = UE mobility | 4.15.13.4 |
| UE direction | Indicate the member UEs should include different moving directions. | NF service: Nnwdaf_AnalyticsSubscription / Nnwdaf_AnalyticsInfo Filter: UE Direction Analytics ID= UE Mobility | 4.15.13.4 |
| UE separation distance (NOTE 1) | Indicate the member UEs should comply with a minimum separation distance between each other. | NF service: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Proximity Attributes Analytics ID: Relative Proximity | 4.15.13.4 |
| Service Experience | Indicates member UEs fulfilling certain Service Experience criteria e.g. MOS value. | NF service: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: S-NSSAI, DNN, Application ID, DNAI, AoI, Service Experience Contribution weight, reporting threshold (NOTE 2), Service Experience Type (NOTE 3) Analytics ID=Service Experience | 4.15.13.5 |
| DNN | Indicate the DNN of the member UEs for the PDU Session used by the application. | NF service: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), Event ID: QFI allocation | 5.2.8.3 |
| NOTE 1: This criterion should only be applied when the number of UEs is in the range of 10's or less. NOTE 2: The Service Experience Contribution Weights signal the relative importance of each UE's Service Experience value (i.e. MOS), as defined in TS 23.288 [50]. For example, it might be that the service experience of a UE in relation to other UEs may not be as important e.g. because the data provided by such UE is not as critical to the service. NOTE 3: Indicates the type of service experience analytics, e.g. AI/ML traffic where a customized MoS apply. |  |  |  |

#### 5.2.6.32	Nnef_MemberUESelectionAssistance service

##### 5.2.6.32.1	General

Service description: The service can be used by the consumer to subscribe or unsubscribe to the Member UE selection assistance information, or to receive notifications on the Member UE selection assistance information, from the NEF. See Table 4.15.13.2-1 for the Member UE filtering criteria.

##### 5.2.6.32.2	Nnef_MemberUESelectionAssistance_Subscribe service operation

Service operation name: Nnef_MemberUESelectionAssistance_Subscribe

Description: The NF consumer subscribes to receive the Member UE selection assistance information, or the subscription is updated if the same subscription is already existing in NEF.

Inputs, Required: At least one Member UE filtering criterion shown in Table 4.15.13.2-1.

Inputs, Conditional Required:

If no Subscription Correlation ID is provided in the subscription, AF Identifier, Notification Target Address (+ Notification Correlation ID), a list of target member UEs in the form of a list of GPSIs or a list of UE IP addresses) is required.

Inputs, Optional: Application ID, Subscription Correlation ID (in the case of update of the existing subscription), Expiry time, time window(s) for selecting the candidate UEs, specific parameters depending on the Member UE filtering criteria as shown in Table 4.15.13.2-1, Periodicity (the periodicity of member update), maximum number of candidate UEs.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

##### 5.2.6.32.3	Nnef_MemberUESelectionAssistance_Unsubscribe service operation

Service operation name: Nnef_MemberUESelectionAssistance_Unsubscribe

Description: The NF consumer deletes a subscription for Member UE selection assistance information that is already created in NEF.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

##### 5.2.6.32.4	Nnef_MemberUESelectionAssistance_Notify service operation

Service operation name: Nnef_MemberUESelectionAssistance_Notify

Description: NEF reports the Member UE selection assistance information to the consumer that has previously subscribed.

Inputs, Required: Notification Correlation Information.

Inputs, Conditional Required:

At least one of the following inputs is required:

-	One or more list(s) of candidate UEs in the form of a list of GPSIs or a list of UE IP addresses.

Inputs, Optional: Recommended time window for performing the application operation per list of candidate UEs as described in clause 4.15.13.1, specific values of the parameters that NEF gathered for the Member UE filtering criteria per candidate UE, a number for each filtering criterion that indicates the amount of UEs in the initial list which do not meet the criterion.

NOTE:	This number can be an indication for AF to revise one or multiple of the corresponding filtering criteria.

Outputs, Required: Operation execution result indication.