3GPP TSG-SA2 Meeting #166	S2-2411381

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024         (revision of S2-2411192

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1208 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the general inference procedure for vertical federated learning to resolve ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [OPPO, ZTE, Samsung,] Nokia [, Lenovo, Toyota, ICS, Huawei, HiSilicon, Ericsson] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Editor's note: How the VFL server determines the VFL clients and whether the VFL server may not select all VFL clients is FFS. is removed since clause 6.2H.2.1 describes the VFL discovery. The UE ID(s) is added in the request send from the VFL server to the VFL client to indicate the VFL client which UE will be used as the sample to let VFL client collect the data and perform the VFL inference. The Editor's note: Whether the VFL model correlation ID is needed or using the model ID is FFS. is removed. The local model in each client can be different. Model ID is designed to identiy the sepcfic model. The model ID is same means the model should have the same topology. However, in the VFL case, the topology for each client local model can be different. It is not suitable to use the same model ID for every client. The VFL model correlation ID is perferred to indicate those local model in each client should used for the same VFL task e.g. for the same analytcis ID. The VFL model correlation ID is added when the VFL client send the VFL inference result to the VFL server. VFL model correlation ID is used to let VFL server know which inference result from which client need to be combined to generate the VFL inference result. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Several ENs are resolved for the VFL inference procedure. Parameter from Analytics subscription such as analytics target and analytics filer is added in the request send from the VFL server to the VFL client and in related replies. The VFL model correlation ID is added when then VFL client send the VFL inference result to the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not perform VFL inference sucessully if some open issue can not be resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G, 6.2H.1, 6.2H.2.4(new), 6.2H.2.4.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CRs shall be implemented in the following order: CR 1198, CR 1246, CR1208, CR1134 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev4: Inference procedures with untrusted AF as server are added. Inference procedures with NWDAF as server are amended to show NEF and AF procedures clearly for trusted and untrusted AF as VFL client and to enable an easy cross-reference from flow for Inference procedures with untrusted AF as server. Suggested procedure names are added. For the inference request the following parameters are added: Analytics ID, Target of Analytics Reporting, e.g the UE IDs, analytics Filters, possible Metadata request, Those parameters can be received by the VFL server as part of an analytics request and will need to be forwarded to the clients. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.2G	Void

### 6.2G.1	Void

### 6.2G.2	Void

#### 6.2G.2.1	Void

# * * * * Next change * * * *

### 6.2H.1	General

This clause specifies how NWDAF as a VFL server can register and also discover other NWDAFs and/or AFs via NRF

Both the VFL server and VFL client store the VFL model after finishing the VFL training process and use the same VFL local model to perform the VFL inference later based on the VFL correlation ID. The differences between the VFL training and inference are that for the inference there is no check of the labels, nor any server intermediate results are sent and as such only client intermediate results are sent to the server.

Editor's note:	This clause needs to be update to include the description of the subclauses for NWDAF or AF as the server case.

# * * * * Next change * * * *

#### 6.2H.2.4	Inference procedure for vertical federated learning

#### 6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Editor's note:	If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS.

0.	For an NWDAF as a VFL server, the analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g., UE IDs, Analytics Reporting Information=Analytics target period), analytics Filters, possible Metadata request) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF determines the VFL Server for the requested analtyics, sends a subscription request to VFL server NWDAF using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g., UE IDs.

Editor's note:	Whether the case that the AnLF can not generathe the analytics output but can determines the VFL server for the requested analytics is exist is FFS. Whether and how such NWDAF is capable to determine the NWDAF as VFL server is FFS.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. VFL Server selects clients(s) using information stored in the VFL training process. The server may select some or no clients, e.g., depending on their contribution to the training result and the current status of the client.

VFL server sends a VFL Inference request/subscription to the VFL clients including Analytivs ID, Target of Analytics Reporting, e.g UE IDs, analytics Filters, possible Metadata request, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

Editor's note:	It is FFS whether additional parameters are needed to send from the VFL server to VFL client.

Editor's note:	It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying.

2a.	For each NWDAF VFL client, the VFL Server NWDAF sends an Nwdaf_VflInference_Subscripe or Nwdaf_VflInference_Request to the VFLclient.

2b	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VflInference_Subscripe or Naf_VflInference_Request to the VFLclient.

2c	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VflInference_Afclient_Subscripe or Naf_VflInference_Afclient_Request to the NEF serving the AF.

2d	For each untrusted AF VFL client, the NEF converts any internal identifiers to external identifiers and sends an Naf_VflInference_Subscripe or Naf_VflInference_Request to the untrusted AF VFLclient.

Editor's note:	Which service is used between the VFL server and VFL client is FFS.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	VFL Client sends the client intermediate local inference results and if requested analytics metadata  to the VFL server.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

Editor's note:	It is FFS additional parameters are needed to be sent from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nwdaf_VflInference_Notify or Nwdaf_VflInference_Request response to the VFL Server NWDAF.

5b	Each trusted AF VFL client sends a Naf_VflInference_Notify or Naf_VflInference_Request response to the VFL Server NWDAF.

5c	Each untrusted AF VFL client sends a Naf_VflInference_Notify or Naf_VflInference_Request response to the NEF.

5d	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VflInference_Afclient_Notify or Naf_VflInference_Afclient_Request response to the NWDAF VFL server.

6.	The VFL server may collects its local data and generate the intermediate local inference results. The VFL Server combines all the intermediate local inference results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate results when generates the combined inference output. The VFL server also combines received analytics metadata.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

7.	If the NWDAF is the VFL server, the VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

##### 6.2H.2.4.2	Inference procedure for vertical federated learning with untrusted AF as VFL server

Figure 6.2H.2.4.2-1: inference procedure for vertical federated learning with AF as VFL server.

1.	The inference procedure may be triggered by internal service logic of the AF acting as VFL server, or by a request or subscription for information from a 5GC entity, for instance for service experience information.

1a.	For an untrusted AF, the 5GC entity sends an NefEventExposure Subscribe to the NEF serving the untrusted AF

1b.	For an unstrusted AF, fhe NEF forwards the subscription to the AF.

Editor's note:	Whether and how a 5GC NF can send a request to trigger the VFL server AF to perform inference is FFS. Whether the AF event exposure service can be reused is FFS

2.	Same as step 2 of Figure 6.2H.2.4.1-1. However, sub steps in that figure are not applicable.

2a.	The Untrusted AF acting as VFL server sends for each NWDAF VFL client an Nnef_VflNwdatClient_InferenceSubscription or Nnef_VflNwdafClient:Request. An untrusted AF includes the temporary NWDAF ID obtained in the discovery procedure (see Clause 6.2H.2.1.1) and sends the request to the NEF.

2b For each NWDAF VFL client, the NEF converts any received external identifiers to internal identifiers and sends an Nnwdaf_VflInference_Subscribe or Nnwdaf_VflInference_Request to the NWDAF VFL client indicated by the received temporary NWDAF ID.

3.	Same as step 3 of Figure 6.2H.2.4.1-1.

4.	Same as step 4 of Figure 6.2H.2.4.1-1

5.	Same as step 5 of Figure 6.2H.2.4.1-1. However, sub steps in that figure are not applicable.

5a 	Each NWDAF VFL client sends a Nnwdaf_VflInference_Notify or Nnwdaf_VflInference_Request response to the NEF,.

5b. For each NWDAF VFL client, the NEF converts any received internal identifiers to external identifiers and sends to the Untrusted AF VFL server an Nnef_VflNwdatClient_InferenceNotify or Nnef_VflNwdafClient:Request response including the temporary NWDAF ID.

6.	Same as step 6 of Figure 6.2H.2.4.1-1.

7.	If the inference procedure was triggered by a request or subscription for information from a 5GC entity, for instance for service experience information, the AF provides the requested notification.

7a.	The AF sends an Naf_EventExposure Notify to the AF

7b.	The NEF sends an Nnef_EventExposure Notify to the 5GC entity.

# * * * * End of changes * * * *