3GPP TSG SA WG2#166	S2-2411454

# Orlando, USA, 18th -22nd, Nov. 2024		(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0268 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Clarification on EAS Discovery using IP replacement to support Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on TR conclusion for KI#1, the Local Offloading Policy is sent to I-SMF. The I-SMF can configure the UL CL and L-PSA UPF based on the Local Offloading Policy. Existing description for HR-SBO can be reused to support the EAS discovery procedure with EASDF/Local DNS Server using IP replacement mechanism for supporting Local Offloading Management with some clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description on how to support EAS discovery procedure with EASDF/Local DNS Server using IP replacement mechanism for Local Offloading Management. |  |  |  |  |  |  |  |  |
| ` |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EAS discovery procedure with EASDF/Local DNS Server using IP replacement mechanism for supporting Local Offloading Management is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

6.7.2.5	EAS discovery procedure with (V-)EASDF/Local DNS Server using IP replacement mechanism for supporting HR-SBO or Local Offloading Management

When the IP replacement mechanism is used for EAS discovery in Local Offloading Management scenario, for the description in below, the H-SMF is replaced by SMF, the V-SMF is replaced by I-SMF, the VPLMN Specific Offloading Information is replaced by Local Offloading Policy, the V-EASDF is replaced by EASDF, the UPF in VPLMN is replaced by L-PSA UPF.

Based on the operator's configuration and local regulations, the IP replacement mechanism may be used for EAS discovery supporting HR-SBO:

-	The H-SMF sends DNS server address provided by the HPLMN included in PCO to UE via V-SMF during PDU Session Establishment/Modification procedure. The DNS query related to the edge computing (corresponding to FQDNs) can be routed to V-EASDF/Local DNS server in the VPLMN using IP replacement mechanism.

NOTE 1:	This EAS discovery procedure requests modification of IP address of DNS messages. Whether this is allowed or not is subject to local regulations. As this procedure for EAS discovery requires an UPF to detect target FQDN in DNS message, it cannot apply when DNS security applies e.g. it does not apply to usage of DoH or DoT.

NOTE 2:	This clause only applies when FQDN range is included in the VPLMN Specific Offloading Information. If Local DNS Server is used for the EAS discovery (i.e. not involving V-EASDF), it is assumed that IP address range is also provided in the VPLMN Specific Offloading Information.

To support IP replacement mechanism, in this clause the SMF shall instruct the UPF to use the FAR that contains Information elements of "IP Address and Port Number Replacement".

Figure 6.7.2.5-1: EAS discovery procedure with V-EASDF/Local DNS Server using IP replacement mechanism for supporting HR-SBO

NOTE 3:	This clause assumes the V-SMF has received the HR-SBO allowed indication from the AMF and supports IP replacement mechanism for HR-SBO, it also assumes the HPLMN authorizes HR-SBO in the VPLMN.

0.	The Registration procedure is described in step 1 of clause 6.7.2.2.

1.	The HR-SBO PDU Session Establishment is described in the step 2 of clause 6.7.2.2 with the following differences:

-	After step 3 in clause 4.3.2.2.2 of TS 23.502 [3], the V-SMF selects a UPF in VPLMN supporting UL CL/BP and PSA functionalities based on UE location information.

NOTE 4:	Based on the deployment of VPLMN, the selected UPF in VPLMN can support both UL CL and PSA functionalities in case that the UL CL UPF and PSA UPF are co-located. This UPF in VPLMN can be the V-UPF selected in the step 4 of clause 4.3.2.2.2 of TS 23.502 [3].

-	The V-SMF sends the request for establishment of the PDU session supporting HR-SBO in VPLMN without the V-EASDF/Local DNS server IP address in the step 6 of clause 4.3.2.2.2 of TS 23.502 [3].

-	If the Nsmf_PDUSession_Create Request received by the H-SMF does not include the V-EASDF/Local DNS server IP address, the H-SMF constructs PCO with DNS server address field set to DNS server address provided by the HPLMN and sends the PCO to UE via V-SMF in the step 13 of clause 4.3.2.2.2 of TS 23.502 [3].

NOTE 5:	The V-SMF can select the V-EASDF and create the DNS context in the V-EASDF or select the Local DNS server before sending Nsmf_PDUSession_Create Request or after having received Nsmf_PDUSession_Create response.

2.	The V-SMF configures the UPF in VPLMN as described in step 3 of clause 6.7.2.2 with the following differences:

-	Based on the FQDN(s) received from the VPLMN Specific Offloading Information, the V-SMF indicates the UPF in VPLMN to route DNS queries for the FQDN (range) query to V-EASDF/Local DNS server. In the case of UL CL, the V-SMF configures the UL CL UPF in VPLMN to route DNS queries for the FQDN (range) query to V-EASDF/Local DNS Server. The V-SMF configures the UPF in VPLMN with IP replacement information (i.e. DNS server IP address and port number of HPLMN, V-EASDF/Local DNS server IP address and port number). In uplink direction, UPF in VPLMN replaces the destination address of the DNS query targeting an FQDN eligible for HR-SBO related offload from DNS server IP address of HPLMN to V-EASDF/Local DNS server IP address; In downlink direction, UPF in VPLMN replaces the source address of the DNS response targeting an FQDN eligible for HR-SBO related offload from V-EASDF/Local DNS server IP address to DNS server IP address of HPLMN.

If V-EASDF is used, the V-SMF also configures the V-EASDF as described in step 3 of clause 6.7.2.2.

NOTE 6:	For the DNS query requiring DNS resolution in the HPLMN, the DNS resolution path is same as the normal path in the HR PDU Session.

3.	UE sends DNS query to DNS server of HPLMN.

4a.	If the DNS query does not match the FQDN range eligible for HR-SBO related offload, UPF in VPLMN delivers the DNS query via H-PSA through N9 and H-PSA delivers the DNS query to the DNS server of HPLMN.

4b.	If the DNS query matches the FQDN range eligible for HR-SBO related offload, the UPF in VPLMN delivers the DNS query to V-EASDF/Local DNS server using IP replacement mechanism. The following EAS discovery procedure is based on step 4b.

5.	The EAS discovery procedure described in steps 8-18 of clause 6.2.3.2.2 or steps 4-6 of clause 6.2.3.2.3 applies with the following differences:

-	This EAS discovery procedure with IP replacement is implemented in the VPLMN.

-	In step 16 of clause 6.2.3.2.2, the V-SMF may perform insertion or change of UL CL/BP and local PSA in VPLMN as described in the step 3 of clause 6.7.2.3.

6.	The V-EASDF/Local DNS server sends the DNS response including FQDN to the UPF in VPLMN. The UPF in VPLMN replaces the source address from V-EASDF/Local DNS server to DNS server of HPLMN in the DNS response based on the V-SMF instructions and sends this DNS response to the UE directly or via UL CL/BP of VPLMN if existing in this PDU Session.

# * * * * End of changes * * * *