3GPP SA WG2 Meeting #166	S2-2411519

# Orlando, FL,USA, 18-22 November 2024								                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1506 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures of IMS Avatar communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions of KI#8 on " supporting IMS Avatar communication " were agreed in clause 8.8 of TR 23.700-77. This CR implements the approved conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Avatar procedures for both UE centric and network centric based modes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS Avatar communication is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X.3 (new), AC.X.3.1 (new), AC.X.3.1.1 (new), AC.X.3.1.2 (new), AC.X.3.2 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | AC.X hereby refers to the corresponding new clause specified in CR #1480. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES (All new)<<<<

## AC.X.3	Procedures

### AC.X.3.1 UE Centric Avatar Communication

#### AC.X.3.1.1	Sending UE based rendering

Figure AC.X.3.1.1-1: Establishment of Sending UE rendered Avatar communication session

Figure AC.X.3.1.1-1 depicts a typical call flow to establish a Sending UE (UE-A) rendered Avatar communication. The main steps in the call flow are as follows:

0.	UE-A and UE-B establish an IMS session that includes audio, video, and bootstrap data channels.

1.	The user of UE-A decides to initiate Avatar communications and triggers the application for Avatar communication.

2.	UE-A upgrades the IMS session for Avatar communication. UE-A initiates a re-INVITE adding media descriptors required for Avatar communication, e.g. RTP and/or application data channels.

3.	The user of UE-A selects the Avatar IDs that associated with specific digital assets (including Base Avatar and personalized accessories). The list of available Avatar IDs may be pre-configured in the UE-A, downloaded via bootstrap data channel, or downloaded via application data channel as defined in clause AC.X.1.

4.	UE-A retrieves the required digital assets from BAR based on the selected Avatar IDs. If UE-A has already downloaded the required digital assets, step 4 is skipped.

5.	UE-A uses its input data, e.g., the camera feeds and the user voice to generate the animation data, then animates the Avatar Representation and renders based on the scene description.

#### 6.	UE-A sends the animated Avatar media to UE-B via RTP.

#### AC. X.3.1.2	Receiving UE based rendering

Figure AC.X.3.1.2-1: Establishment of Receiving UE rendered Avatar communication session

Figure AC.X.3.1.2-1 depicts a typical call flow to establish a Receiving UE (UE-B) rendered Avatar communication. The main steps in the call flow are as follows:

0.	UE-A and UE-B establish an IMS session that includes audio, video, and bootstrap data channels.

1.	The user of UE-A decides to initiate Avatar communications and triggers the application for Avatar communication.

2. The application data channels for avatar communication are established, including P2P and/or P2A application data channels.

3.	The user of UE-A selects the Avatar IDs that associated with specific digital assets (including Base Avatar and personalized accessories) for the Avatar communication. The list of available Avatar IDs may be pre-configured in UE-A, downloaded via bootstrap data channel, or downloaed via application data channel

4.	Considering the local media capability and/or power status, UE-A determines to triggers receiving UE based rendering. UE-A initiates media negotiation with DC AS to perform receiving UE rendered Avatar communication. The DC AS further negotiates the requested media with UE-B and the DCSF.

5.	Based on the network policy of Avatar rendering (e.g., for 3D Avatar Communication), UE-B may retrieve via MF the scene description generated by the DC AS.

6.	The MF retrieves the required digital assets from BAR based the received Avatar IDs selected by UE-A.

7.	The IMS session may be updated to establish the required application data channel used to transmit animation data and /or RTP media used to transimt video streams. The MF is instructed by IMS AS on media handling.

8a.	In this option, UE-A uses its input data, e.g., the camera feeds and the user voice to generate the animation data, then sends the animation data stream to MF via the established data channel.

8b.	In this option, UE-A sends the media streams that are used to generate the animation data to the MF. These streams may include video streams from the user’s cameras and/or the user’s captured audio streams. The MF generates the animation data from the received media streams.

NOTE:	Whether alternative 8a or 8b is performed is determined by media negotiation with DC AS and instructed by scene description.

9.	 The MF then sends the animation data, which it received from UE-A or generated by itself, to UE-B.

10.	UE-B animates UE-A’s Avatar based on the downloaded digital assets and the received animation data. If UE-B receives the scene description, the animated Avatar may be further rendered in the scene environment.

11.	Based on local UE and network policy, UE-B may send back the rendered Avatar media to UE-A.

### AC.X.3.2 Network Centric Avatar Communication

Figure AC.X.3.2-1: Establishment of Network Centric Avatar communication session

Figure AC.X.3.2-1 depicts a typical call flow to establish a network rendered Avatar communication. The main steps in the call flow are as follows:

0.	UE-A and UE-B establish an IMS session that includes audio, video, and bootstrap data channels.

1.	The user of UE-A decides to initiate Avatar communications and triggers the Avatar application.

2. The application data channels for Avatar communications are established, including P2P and/or P2A applicationdata channels.

3.	The user of UE-A selects the Avatar IDs that associated with specific digital assets (including Base Avatar and personalized accessories) for the Avatar communication. The list of available Avatar IDs may be pre-configured in UE-A, downloaded via bootstrap data channel, or downloaed via application data channel

4.	Considering the local media capability and/or power status, UE-A determines to triggers network centricrendering. UE-A initiates media negotiation with the DC AS to perform network rendered Avatar communication. The DC AS further negotiates the requested media with UE-B and the DCSF.

5.	Based on the network policy of Avatar rendering (e.g., for 3D Avatar Communication), the MF may retrieve the scene description generated by the DC AS.

6.	UE-B retrieves the required digital assets from BAR via MF based the received Avatar IDs selected by UE-A.

7.	The IMS session may be updated to establish the required application data channel used to transmit animation data and /or RTP media used to transimt video streams. The MF is instructed by IMS AS on media handling.

8a.	In this option, UE-A uses its input data, e.g., the camera feeds and the user voice to generate the animation data, then sends the animation data stream to MF via the established data channel.

8b.	In this option, UE-A sends the media streams that are used to generate the animation data to the MF. These streams may include video streams from the user’s cameras and/or the user’s captured audio streams. The MF generates the animation data from the received media streams.

NOTE:	Whether alternative 8a or 8b is performed is determined by media negotiation with DC AS and instructed by scene description.

9.	The MF animates UE-A’s Avatar based on the downloaded Avatar digital assets and the animation data The MF may further render the Avatar media according to the scene description.

10.	The MF sends the animated Avatar media to UE-B via RTP.

11.	Based on local UE and network policy, The MF may send back the rendered Avatar media to UE-A.

# >>>>END OF CHANGES<<<<