SA WG2 Meeting #S2-166                                            S2-2411711

Orlando, USA, 2024-11-18 – 2024-11-22                       (revision of S2-241xxxx)

Source:	Ericsson

Title:	Discussion on the energy related information in NF Profile for KI#3

Document for:	Discussion

Agenda Item:	19.4.2

Work Item / Release:	EnergySys / Rel-19

Abstract of the contribution: In the solution for KI3, A lot of CRs propose extending the NF profile with information elements of energy related information to support energy aware NF Selection. Various of IEs are proposed in those CRs without clear description on how each of those information elements shall be used by a NF consumer. Some information elements are overlapping. This document wants to have a discussion on the usage of each information element and where it should be present it into the NF profile.

# 1. Discussion

The CR in S2-2411077 below has proposed 2 IEs in section 1.1 and in earlier revision many other IEs are also proposed, see section 1.2.

## 1.1 IEs in the final S2-2411077 version

Table 5.X.2-1: Energy related information

| Information | Description | Category |
| --- | --- | --- |
| Energy Efficiency | Ratio of the NF service KPI (e.g. forwarded data volume of UPF) over the energy consumed in the NF. | Optional |
| Renewable Energy | Ratio of the renewable energy to the total energy (see ISO/IEC 30134-3:2016 [y]) | Optional |

Energy Efficiency: May be OK with clarifications on usage and relation to other NF profile parameters like priority and Load.

In current proposals it’s unclear whether this pertains to current energy efficiency (almost real-time), predicted energy efficiency, or normalized energy efficiency over an extended period. A semi-static approach with configured levels should also be considered as an alternative.

If this is based on the energy performance KPI defined by SA5, all NF instances or NF service instances within an NF set or NF service set should use the same NF service KPI. If all NF instances utilize the same hardware and settings, NF consumers might attempt to direct traffic to more heavily loaded servers. Therefore, NF consumers may also need to consider the load on NF producers as an additional factor.

The Energy Efficiency could be acceptable IE to be added into the NF profile with additional work on the following points:

- The value of these metrics and how NF consumers should use them for selecting an NF.

- How should the NF load be considered and what is the relation to the priority value in the NF profile.

- Operator configured or measured Energy Efficiency

  - A (semi-)static configuration approach only requires agreement on some enumeration of levels

  - For measurement approach it’s recommended to issue a LS to SA5 to define the Energy Efficiency KPI for each NF type considering factors like:

    - Current, predicted or normalized Energy Efficiency?

    - The measurement interval for the IEs

Renewable Energy: Not clear if it is available to the operator (request for comment from operators?) and how accurate it is

ISO/IEC 30134-3:2016 does not seem to be publicly available for free and should therefore not be used as a reference.

It is unclear if operators will be able be retrieve this information from their energy providers in an easily consumable format. Feedback from operators and/or SA5 is needed before we add this type of information to the NF profile.

Additionally, the accuracy and usefulness of this information should be considered. If the operator can obtain the information, the energy company might provide information according to the energy subscription rather than the actual energy used.

## 1.2 IEs in the historical CR versions

| Information | Description | Category |
| --- | --- | --- |
| Energy Saving States | The state of energy saving feature. E.g., energy saving activated, energy saving deactivated, or notEnergySaving state, energySaving state as specified in TS 28.310 [x]. | Optional |
| Energy Consumption | The amount of energy consumed by the NF | Optional |
| Energy Consumption Restriction Level | The level that restricts the energy consumption allowed for the NF instance uses over time window. The restrict level may be a certain amount of energy consumption, or a range of amount of energy consumption, or low, middle, high. | Optional |
| Energy Efficiency Level | The level of energy efficiency of the NF (e.g. Low, Medium, High, or 1, 2, 3, 4, 5). Energy Efficiency level may be mapped with the NF load information. | Optional |
| Carbon emission factor | Indicating amount of carbon emissions relative to an amount of resource consumption (NOTE 1) (NOTE 2) (NOTE 9) | Optional |
| -Schedule of energy states | The schedule of the energy states, e.g. the time of entering the states regarding energy saving. (NOTE 5) (NOTE 7) | Optional |
| Schedule of power source characteristics | The schedule of the power source characteristics. These characteristics include the Renewable Energy Ratio and the Carbon emission factor. | Optonal |
| Energy saving state transition execution indication | Indicating whether the NF is subject to energy related execution (NOTE 3). (e.g., a UPF that is scheduled to be taken out of service to save energy in energy saving state). | Optional |
| NOTE 1: Resource consumption is defined in TS 28.554 [z] for a VNF. The indication refers to a relative value. The granularity of such indication and how to calculate is to be defined by SA WG5, e.g. it could be of three levels: Low, Medium, High. NOTE 2: Carbon emission factor can also indicate relative carbon efficiency level of the NF, e.g. an indication with Low, Medium, High, or a level value with 1, 2, 3, 4, 5. KPI can be further adjusted/defined based on related study in SA WG5. NOTE 3: Energy related execution refers to switch the NF to energySaving state for energy saving. NOTE 4: The Renewable Energy Ratio can also denote the type of energy type of the NF, e.g. renewable energy, non-renewable energy. NOTE 5: The NF service consumer may select at NF based on the time-based received information from NRF. NOTE 7: Energy states refers to state of a cell, a network element and/or a network function with respect to energy, e.g. (not) energy saving states, which are defined in TS 28.310. NOTE 8: See ISO/IEC 30134-3:2016 [y]. NOTE 9: kilograms of equivalent carbon dioxide emitted per kWh (kg of CO2eq/kWh). (see ETSI GS OEU 020 [z]) |  |  |

Energy Saving States: Not clear.

It is unclear how to transition an NF into this state. While the RAN can have energy-saving features like a minor sleep mode, 5GC NFs do not have a well-defined set of similar features. It seems that the operator may need to "lock" the NF, requiring NF service consumers to stop sending new traffic to that NF provider. The NF provider would also need to redirect traffic to other NF providers. This approach seems to mix the concepts of "shutting down" and "locked" within one IE (refer to ITU-T X.731 Administrative state). Shutting down a VNF defined by ETSI MANO does not directly save energy. Additional measures, such as shutting down the server or putting it into sleep mode, are needed to free up hardware resources from the underlying infrastructure layer.

If an NF is shut down, it will deregister itself. Alternatively, is this a state where the NF is in the process of "shutting down" or "shut down" but remains registered in the NRF?

Coordinating with SA5 would be required to understand the energy saving state in relation to the life cycle management of a NF before adding the parameter in the NF profile.

Energy saving state transition execution indication: not clear.

This sounds to be a “shutting-down” state, refer to ITU-T X.731 and relating to the Energy Saving States (locked).

Schedule of energy states: Not OK

No benefit has been identified for providing time windows for energy states compared to simply changing the energy-saving state when an NF enters this condition.

Energy Consumption: either Energy Efficiency or this one with NF Service KPI

An absolute value is meaningless unless we also provide an additional IE for the NF Service KPI. If the NF Service KPI is provided, it can also be used to calculate a relative value by incorporating other IEs, such as the carbon emission factor (e.g., Carbon emission / NF Service KPI). This could be explored in future releases.

Energy Consumption Restriction Level: Not OK

An absolute value is not useful for NF selection to improve the Energy Efficiency KPI unless we also provide an additional IE for the NF Service KPI. It is unclear:

- what we aim to achieve by using this in the selection process—are we trying to reduce energy consumption for an individual NF?

- what actions the NF consumer is expected to take.

- how the level should be set

Carbon emission factor: Not clear if it is available to the operator (request for comment from operators) and how accurate it is.

An absolute value is not useful for NF selection aimed at reducing total CO2e emissions. In addition to energy consumption and the carbon emission factor, a Service KPI is also needed. This can be helpful if CO2e emissions vary across different sites (e.g., due to differences in cooling-related energy consumption and energy source mixes), even when the energy consumption of NF instances at those sites is similar. However, there are similar concerns regarding the availability and accuracy of this information as there are with renewable energy data. It also needs to be clarified whether this is an average value of different energy sources, e.g. wind power, solar power, and fossil power.

Schedule of power source characteristics: Not OK

Is the schedule the time window for different renewable energy source Co2e emission factor.

No benefit has been identified for providing time windows for energy states compared to simply changing the Renewable Energy and Carbon emission factor the value for them is changed in specific time window. This has the same effect as providing the schedule.

# 2. Proposal

The Energy Efficiency could be acceptable IE to be added into the NF profile with additional work on the following points:

- The value of these metrics and how NF consumers should use them for selecting an NF.

- How should the NF load be considered and what is the relation to the priority value in the NF profile.

- Operator configured or measured Energy Efficiency

  - A (semi-)static configuration approach only requires agreement on some enumeration of levels

  - For measurement approach it’s recommended to issue a LS to SA5 to define the Energy Efficiency KPI for each NF type considering factors like:

    - Current, predicted, or normalized Energy Efficiency?

    - The measurement interval for the IEs

Clarification and coordination with SA5 is needed regarding the behavior of an NF in the Energy Saving State before including the IEs related to the Energy Saving State in the NF profile.

The availability and accuracy of the Carbon Emission Factor and Renewable Energy Ratio need to be investigated before including them into the NF profile.