3GPP TSG SA WG2 Meeting #166	S2-2411717

Orlando, US, 2024‑11-18 - 2024‑11-22	(revision of S2-220xxxx)

Source:	Apple

Title:	UE Data Collection in 5GC for RAN AIML operation

Document for:	Discussion

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract: This discussion paper analyses the status of UE data collection discussion in SA2 and proposes a way forward.

# 1. Discussion

The LS S2-2411455 (RP-242389) from RAN asked SA2 to provide views on Solution 1b, Solution 2 and Solution 3.

In the LS, RAN highlighted their agreements:

RAN has agreed to the following requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is futureproof and extendable. FFS/study if and how to handle non-standardized data (i.e. partial visibility). FFS controllability on data collection Standardized Solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

RAN made the following request towards SA2

RAN2 could not reach consensus without input from SA groups on feasibility of MNO full controllability and full visibility (Note 5) for option 1b and couldn’t reach consensus on the feasibility of UP solution for option 2 and option 3 (Note 7).

….

The standardized solutions for data collection (if standardized) need to meet the requirements above. TSG RAN kindly asks TSG SA to initiate the work and at least for SA2 to provide inputs on solutions 1b, 2, and 3 by RAN#106.

This discussion paper analyses the status of SA2 discussions and proposes a way forward.

## 1.1 Discussion in SA2#165

The input contributions in SA2#165 demonstrated varying interpretation of requirements from RAN2. The terminology ‘full controllability’ and ‘full visibility’ caused confusion among other unclear aspects from these requirements. SA2 agreed on an LS to RAN2 to clarify some of the aspects from RAN2 agreements in SA2#165 (S2-2411191).

It is expected that SA2 will receive an early reply from RAN2 during the SA2#166. This discussion paper analyses the current state of discussion to propose a way forward in SA2.

## 1.2 Applicability of Option 1b

Option 1b is defined as an application layer data collection scheme. For data collection from the UE, it uses an Application Function (DCAF) that collects data from UE applications. The data traffic is over the 5G user plane.

However, only a customised deployment of option 1b can be considered for evaluation against RAN2 requirements of controllability and visibility for MNO. The DCAF has to be deployed in MNO domain. The limitations imposed by such a deployment (various DCAF instances per UE vendor) and whether any modifications are required (e.g., to make this architecture work with different vendors) requires more study in SA2.

## 1.3 User plane approach for Option 2

In Option 2, a Network Function in the 5G CN is responsible for collection of data from the UE and exposure of data outside of MNO domain. The request from RAN2 is to provide feasibility analysis of a user plane architecture in this option. The main aspect to analyse is which entity is performing this data collection from the UE. There is no NF with this functionality today. A feasibility analysis becomes impossible when we have to evaluate various candidate NFs that can be made responsible for UE data collection.

## 1.4 Mix of control plane and user plane

Discussions in SA2#165 also proposed approaches which combines control plane and user plane. One of the possibilities is to provide measurement configuration through control plane, collect data using user plane and further expose data using service based interfaces. Such an approach offers flexibility, but that also needs time to study and evaluate against complexity.

## 1.5 Provisioning data collection requirements from external AFs

The purpose of data collection is to train UE side ML models. This training is to be performed by an entity external to 3GPP network. The collected data is useful for model training if it is customised according to the requirements from the external entities responsible for model training. SA2 need to study how to receive requirements from AF (entity responsible for performing training), how to reflect these requirements to configuration, data collection and reporting.

## 1.6 Option 3

Option 3 performs data collection using an entity located in the OAM domain. This requires coordination with SA5 to identify the suitable entity that is talking with NG-RAN and the UE to configure measurements and collect data from the UE. SA2 have not discussed the details.

## 1.7 Other aspects

It is likely that even with additional inputs from RAN2, some of the questions posed by SA2 to RAN2 may require additional discussion. These include conditions that may affect full controllability by MNO, impact on UE’s normal operations and roaming considerations required for UE data collection. The topics highlight the need to align across WGs to define requirements that could have a system level impact.

## 1.8 Proposals

Considering the status of discussion as described above, it is clear that SA2 needs to go into more detailed analysis of options before providing a feasibility analysis that RAN has requested.

Proposal 1: SA2 sends a reply LS to SA#106 indicating the current status of discussions and requesting dedicated study TUs to address the RAN request.

# 2. Conclusions

It is proposed to send an LS indicating that more TUs are required for studying these options before providing a feasibility report.

Proposal 1: SA2 sends a reply LS to SA#106 indicating the current status of discussions and requesting dedicated study TUs to address the RAN request. A draft is provided in S2-24xxyyy

# 3. References

1. S2-2409600 - LS from TSG RAN: LS on AIML data collection

2. S2-2411191 – LS to RAN2