3GPP TSG-SA2 Meeting #166	S2-2411741

# Orlando, US, November 18 - 22, 2024											   (revision of S2-241XXXX)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5147 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: NEF Service operation to discover VFL client |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Untrusted AF as VFL server to discover the VFL client NWDAF, a new NEF service operation is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New Nnef_NFDiscovery service operation is defined |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support the discovery of VFL client NWDAF when untrusted AF as VFL server |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.X (new), 5.2.6.X.1 (new), 5.2.6.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.2.6.X	Nnef_NFDiscovery service

5.2.6.X.1	General

Service description: This service enables one NF consumer to discover a set of NF instances with specific NF service or a target NF type.

5.2.6.X.2	Nnef_NFDiscovery_Request service operation

Service operation name: Nnef_NFDiscovery_Request

Description: provides the temporary ID of the expected NF instance(s) to the NF service consumer.

Inputs, Required: one or more target NF service Name(s), NF type of the target NF (i.e. NWDAF), NF type of the NF service consumer (i.e. AF).

If the NF service consumer intends to discover an NF service producer providing all the standardized services, it provides a wildcard NF service name.

Inputs, Optional:

-	If the target NF is NWDAF, the request may include:

-	Analytics ID(s) (possibly per service).

-	Required VFL capability type (i.e. VFL client, if available) and Time period of interest when the target is an NWDAF with VFL capability. When the target is an NWDAF with VFL client capability, and NF type(s) where data can be collected as input for local model training may be included.

Details about NWDAF discovery and selection are described in clause 6.3.13 of TS 23.501 [2].

Outputs, Required:

-	NF type

-	Temporary ID of the NF instance

Outputs, Optional: Per NF instance, other information in the NF profile listed in clause 6.2.6 of TS 23.501 [2] related to the NF instance, such as:

-	If the target NF is NWDAF, it may include:

-	If the target NF is NWDAF with VFL capability, it may also include VFL capability information per analytics ID containing VFL capability type (i.e. VFL client, if available) and Time interval supporting VFL, if available (see clause 5.2 of TS 23.288 [50]).

Details about NWDAF specific information are described in clause 6.3.13 of TS 23.501 [2].

See clause 6.2H.2.1.2 of TS 23.288 [50] for details on the usage of this service operation.

Editor's Note:	Further updates to Nnef_NFDiscovery_Request Service is FFS.

* * * * End of Changes * * * *