SA WG2 Meeting #166	S2-2411771

# Orlando, USA, 18 – 22 November 2024                         (revision of S2-240xxxx)

Source: 	CATT

Title: 	KI#1 Conclusions on Reader selection

Document for: 	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to conclude RAN Reader selection for Topology 1 architecture and UE Reader selection for Topology 2 architecture.

1	Discussion

This paper discusses the RAN/UE Reader selection based on the solutions of TR 23.700-13 and makes conclusion proposals.

The solution proposals for RAN Reader selection (for Topology 1 architecture) or UE Reader selection (for Topology 2 architecture) can be summarized in the following table.

| Solution# | RAN Reader selection | UE Reader selection |
| --- | --- | --- |
| #3 | AF provides a list of reader IDs to limit the inventory to specific Readers to AIoT controller. The AIoT Controller determines the list of Readers to use for sending the Command taking the Reader IDs (if provided by the AF) and any stored information about the last known Reader for specific Device IDs into account. The details of determining candidate Readers are up to AIoT Controller implementation. |  |
| #4 | AIoT function discovers and selects reader (i.e. UE reader(s) and/or RAN reader(s)) to perform Inventory Operation according to the Inventory Operation information. e.g. using the target area for the operation to discovery the readers. If the target area for the operation matches the AIoT service area supported by the readers, those readers can be selected for execute the inventory operation. |  |
| #8 | The AIOTF discovers A-RANs based on internal area information. The NEF perform the area translation to translate external area information to the internal area information. |  |
| #10, #18, #31 | The AMF/Ambient IoT NF selects the NG-RAN reader based on the TA list. The NEF Converting location information to TA List information. | If UE GPSI isn't included in the message, the AMF/Ambient IoT NF selects the fixed-type UE readers based on the TA, Location list, and AF ID. Alternatively, if UE GPSI is carried in the message, the AMF/Ambient IoT NF selects the designated UE reader based on UE GPSI. |
| #11 |  | The AIOTF collects necessary information from relevant NFs, for I-node selection, based on the requested location info (candidate locations or the SUPI of the intermediate node). The gNB may respond to the AIOTF via AMF with the final list of selected I-nodes. In case the gNB does not selection any I-node, the response may indicate there is no available UE. |
| #17 | The AIoTMF selects the Reader(s) and corresponding Ambient IoT capable RANs, taking the device information (e.g. AF requested device area information) and information obtained from the UDM/AAA (e.g. allowed service area) into account. The AIoTMF may also use the last serving Reader in the subscription-like information to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device. |  |
| #22 |  | The AF determines Intermediate UEs based on the location information and reader capability information reported by the Intermediate UEs in step 1, as well as the area AF intended to perform inventory. |
| #23 |  | The AIoTF selects the appropriate intermediate node UEs that will be engaged in the inventory procedure. The inventory frequency of the intermediate UE should overlap with the inventory frequency supported by the AIoT device. |
| #26 | Based on the Target Set ID, the 5GC determines the reader(s) and the reader type(s) to perform the service operation, including checking whether the reader has any constraints that must be satisfied. |  |
| #30 |  | The AIOTF generates the candidate UE list for the Intermediate UEs to be used, based on the location information for the Intermediate UEs, as well as area information provided by the AF. |

1) RAN Reader selection for Topology 1 architecture

For RAN Reader selection, it is observed from the above summaries that all the solutions propose that CN selects the RAN Reader. Since it is concluded a new AIoTF is introduced to support AIoT Services, it is proposed that AIoTF selects the RAN Reader.

Proposal 1: AIoTF selects the RAN Reader.

Regarding on the information used for RAN Reader selection, it is observed that the common way is the AIoTF selects the RAN Reader based on the AF provided service area information.

Proposal 2: AIoTF may select the RAN Reader based on the AF provided service area information.

In order for AIoTF to perform RAN Reader selection, each RAN Reader coverage information is required in the AIoTF. Considering the RAN Readers deployment is static, it proposed to use OAM to configure RAN Reader coverage information in the AIoTF. How to perform mapping between AF provided service area information and RAN Reader coverage information depends on implementation.

Proposal 3: AIoTF may determine RAN Reader coverage information based on OAM configuration.

One optimization may be considered is that once RAN node reports RAN Reader information during the Inventory or Command response procedure, that RAN Reader information associated with the Device ID information can be stored in the AIoTF or UDM. Such that AIoTF could select the RAN Reader based on the stored RAN Reader information.

Proposal 4: AIoTF may select the RAN Reader based on the stored RAN Reader information in AIoTF or UDM.

2) UE Reader selection for Topology 2 architecture

For UE Reader selection, similar to RAN Reader selection, almost all the solutions propose that CN selects the RAN Reader. Only very few solutions mentioned RAN may be involved in the UE Reader selection, but it is not very clear the motivation for RAN involvement. So it is proposed that AIoTF selects the UE Reader.

Proposal 5: AIoTF selects the UE Reader.

Regarding on the information used for UE Reader selection, the most straightforward way is that the AF provides UE Reader information (e.g. UE’s GPSI) which can be used for UE Reader selection by AIoTF. RAN assumed that one UE only has one UE Reader, so UE ID could be used to identify the UE Reader.

Proposal 6: AIoTF may use the AF provided UE ID (e.g. UE’s GPSI) for UE Reader selection.

Another way for UE Reader selection is similar to RAN Reader selection that the AF provided service area information could be used. AIoTF may determine UE Reader coverage information based on UE’s location information, and the mapping between AF provided service area information and UE Reader coverage information depends on implementation.

Proposal 7: AIoTF may select the UE Reader based on the AF provided service area information.

Proposal 8: AIoTF may determine UE Reader coverage information based on UE’ location information.

Similarly to the RAN Reader case, AIoTF may also select the UE Reader based on the stored UE Reader information in AIoTF or UDM.

Proposal 9: AIoTF may select the UE Reader based on the stored UE Reader information in AIoTF or UDM.

2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

8.1.2	Architecture to Support Topology 1

The principles and aspects in this clause are agreed to support Topology 1:

-	The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1:

-	Communicate with BS Reader (i.e. AIoT RAN), either directly or via an AMF, to trigger e.g. inventory request and command request).

NOTE 1:	There is no assumption about whether the AIoT RAN (i.e. BS Reader) also has gNB functionality for NR-Uu or not.

Editor's note:	The relationship between the AIoT RAN and (other) gNB functionality is FFS.

NOTE 2:	It is assumed Ambient IoT services can be deployed independently from existing deployments.

NOTE 3:	It is not expected a deployment will use both direct communication between a BS Reader and an AIoTF and indirect communication between a BS Reader and an AIoTF via an AMF.

When a BS Reader and the AIoTF communicate directly:

-	The AIOTF communicates with a BS Reader via a direct interface Nx.

-	Figure 8.1.2-1 below shows the aspects related to Topology 1 (direct path) reference architecture with other NFs removed.

Figure 8.1.2-1: Non-Roaming 5G System Architecture (Direct Path)

-	Figure 8.1.2-2 below shows the aspects related to Topology 1 (direct path) in reference point representation with other NFs removed.

Figure 8.1.2-2: Non-Roaming 5G System Architecture in reference point representation (Direct Path)

NOTE 4:	The protocol used over Nx will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

NOTE 5:	The protocol stack used between the AIoTF and the BS Reader will be concluded by RAN WG3.

-	Figure 8.1.2-3 below shows the aspects related to Topology 1 (direct path) protocol stack between the BS Reader and AIoTF.

Figure 8.1.2-3: Example Protocol Stack between AIoTF and AIoT Device for Topology 1 (Direct Path)

Editor's note:	The details of the protocol stack are FFS.

When a BS Reader and the AIoTF communicate indirectly via an AMF:

-	The AIoTF connects with a BS Reader via an AMF. The N2 interface between the BS Reader and AMF supports Ambient IoT services including delivery of inventory/command messages.

NOTE 6:	If network isolation is required an AMF instance is deployed for supporting the AIOTF communication with the BS Reader.

-	NGAP between the BS Reader and the AMF is enhanced to support Ambient IoT Services.

NOTE 7:	The details of the enhancements will be concluded by RAN WG3.

-	The AMF shall be enhanced to support Services which are used by an AIoTF for Ambient IoT Operations. New SBI based services on the AMF (to be used by the AIoTF) are introduced in 5GC.

-	The AMF passes information between BS Readers (e.g. operation requests and responses) and the AIOTF.

Editor's note:	The AMF enhancements are FFS.

Editor's note:	Details of the Service (e.g. whether it is a new service, whether the existing Namf service is enhanced is FFS).

-	Figure 8.1.2-4 below shows the aspects related to Topology 1 (indirect path via AMF) reference architecture with other NFs removed.

Figure 8.1.2-4: Non-Roaming 5G System Architecture (indirect Path via AMF)

-	Figure 8.1.2-5 below shows the aspects related to Topology 1 (indirect  Path via AMF) in reference point representation with other NFs removed.

Figure 8.1.2-5: Non-Roaming 5G System Architecture in reference point representation (indirect Path via AMF)

-	Figure 8.1.2-6 below shows the aspects related to Topology 1 (indirect path via AMF) protocol stack between the BS Reader and AIoTF.

Figure 8.1.2-6: Example Protocol Between AIoTF and AIoT Device for Topology 1 (indirect Path via AMF)

Editor's note:	The details of the protocol stack are FFS.

Editor's note:	How addressing works for UL traffic (i.e. how the BS Reader identifies the appropriate AMF to which to forward UL messages) in the indirect path via AMF is FFS.

-	For RAN Reader selection:

-	AIoTF selects the RAN Reader.

-	AIoTF may select the RAN Reader based on the AF provided service area information and RAN Reader coverage information.

-	RAN Reader coverage information are configured in the AIoTF via OAM configuration.

-	AIoTF may also select the RAN Reader based on the stored RAN Reader information in the AIoTF or UDM.

# * * * * Second change * * * *

8.1.3	Architecture to Support Topology 2

8.1.3.1	General

Two options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

Editor's note:	Whether a NAS-based option is needed in addition is FFS.

8.1.3.2	User-plane option

The following principles apply:

-	As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

Figure 8.1.3.2-1: User-plane architecture for Topology 2

Figure 8.1.3.2-2: Protocol stack for the user-plane architecture for Topology 2

NOTE 1:	Which transport protocol to use for AIoT-AP can be decided by CT WG1 in coordination with RAN WG3.

NOTE 2:	Security for AIoT-AP is assumed to be defined by SA WG3.

Editor's note:	Further details are FFS.

Editor's note:	The terminology, e.g. AIoT RAN is FFS.

8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC and a protocol between gNB/AIoT RAN and AIoTF. The related protocol stack options are shown in Figure 8.1.3.3-1 and Figure 8.1.3.3-2.

Editor's note:	Which option to use, i.e. whether gNB/AIoT RAN connect to AIoTF directly (Option A in Figure 8.1.3.3-1) or via AMF (Option B in Figure 8.1.3.3-2) is FFS.

Editor's note:	How addressing works for UL for Option B is FFS.

Editor's note:	Further details are FFS.

Figure 8.1.3.3-1: Protocol Stack for the RRC option (Option A)

Figure 8.1.3.3-2: Protocol Stack for the RRC option (Option B)

-	For UE Reader selection:

-	AIoTF selects the UE Reader.

-	AIoTF may use the AF provided UE ID (e.g. UE’s GPSI) for UE Reader selection.

-	AIoTF may select the UE Reader based on the AF provided service area information and UE Reader coverage information.

-	AIoTF may determine UE Reader coverage information based on UE’ location information.

-	AIoTF may also select the UE Reader based on the stored UE Reader information in the AIoTF or UDM.

# * * * * End of Changes * * * *