3GPP TSG-SA2 Meeting #166	S2-2411777

# Orlando, USA, November 18 – 22, 2024	(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5842 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for the Multi-modality awareness at RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the LS from RAN WG2(S2-2411300 / R2-2409272), RAN2 has agreed for Multi-modal awareness at RAN and multi-modality information needs to be provided by the CN. It is proposed to provide multi-modality information to the NG-RAN in order to support joint admission control, QoS flow to DRB mapping and traffic synchronization at RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.7.1.2 - Multi-modal Awareness ID is added to the QoS Profile to be provided to the NG-RAN §5.7.2.X(new) - New clause under the 5G QoS Parameters to add description on the Multi-modal Awareness ID §5.8.5.4 - Multi-modality information marking is added to the QER for the instruction from the SMF to the UPF. §5.37.1 - Add reference to the new clause for the support for Multi-modality awareness at RAN. §5.37.X(new) - New clause for the description to support for Multi-modality awareness at RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-modal awareness at RAN is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.2, 5.7.2.X (new), 5.8.5.4, 5.37.1, 5.37.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 5.7.1.2	QoS Profile

A QoS Flow may either be 'GBR' or 'Non-GBR' depending on its QoS profile. The QoS profile of a QoS Flow is sent to the (R)AN and it contains QoS parameters as described below (details of QoS parameters are described in clause 5.7.2):

-	For each QoS Flow, the QoS profile shall include the QoS parameters:

-	5G QoS Identifier (5QI); and

-	Allocation and Retention Priority (ARP).

-	For each QoS Flow, the QoS profile may also include the QoS parameters:

-	PDU Set QoS Parameters (described in clause 5.7.7).

-	Multi-modal Awareness ID (described in clause 5.7.2.X).

-	For each Non-GBR QoS Flow only, the QoS profile may also include the QoS parameter:

-	Reflective QoS Attribute (RQA).

-	For each GBR QoS Flow only, the QoS profile shall also include the QoS parameters:

-	Guaranteed Flow Bit Rate (GFBR) - UL and DL; and

-	Maximum Flow Bit Rate (MFBR) - UL and DL; and

-	In the case of a GBR QoS Flow only, the QoS profile may also include one or more of the QoS parameters:

-	Notification control;

-	Maximum Packet Loss Rate - UL and DL.

NOTE:	In this Release of the specification, the Maximum Packet Loss Rate (UL, DL) is only provided for a GBR QoS Flow belonging to voice media.

Each QoS profile has one corresponding QoS Flow identifier (QFI) which is not included in the QoS profile itself.

The usage of a dynamically assigned 5QI for a QoS Flow requires in addition the signalling of the complete 5G QoS characteristics (described in clause 5.7.3) as part of the QoS profile.

When a standardized or pre-configured 5QI is used for a QoS Flow, some of the 5G QoS characteristics may be signalled as part of the QoS profile (as described in clause 5.7.3).

* * * * Start of 2nd Change * * * *

5.7.2.X	Multi-modal Awareness ID

The Multi-modal Awareness ID is an identifier generated by the PCF based on the combination of AF identifier and the Multi-modal Service ID from the AF that refers to the multi-modal communication service.

Multi-modal Awareness ID(s) associated with each QoS Flow are provided to the NG-RAN in the QoS profile and the Multi-modal Awareness ID is included in the GTP-U header for packets belonging to the associated multi-modal service as described in clause 5.37.X.

* * * * Start of 3rd Change * * * *

5.8.5.4	QoS Enforcement Rule

The following table describes the QoS Enforcement Rule (QER) that defines how a packet shall be treated in terms of bit rate limitation and packet marking for QoS purposes. All Packet Detection Rules that refer to the same QER share the same QoS resources, e.g. MFBR.

Table 5.8.5.4-1: Attributes within QoS Enforcement Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this QER |  |
| Rule ID | Unique identifier to identify this information. |  |
| QoS Enforcement Rule correlation ID (NOTE 1) | An identity allowing the UP function to correlate multiple Sessions for the same UE and APN. | Is used to correlate QoS Enforcement Rules for APN-AMBR enforcement. |
| Gate status UL/DL | Instructs the UP function to let the flow pass or to block the flow. | Values are: open, close, close after measurement report (for termination action "discard"). |
| Maximum bitrate | The uplink/downlink maximum bitrate to be enforced for the packets. | This field may e.g. contain any one of: - APN-AMBR (for a QER that is referenced by all relevant Packet Detection Rules of all PDN Connections to an APN) (NOTE 1). - Session-AMBR (for a QER that is referenced by all relevant Packet Detection Rules of the PDU Session) - QoS Flow MBR (for a QER that is referenced by all Packet Detection Rules of a QoS Flow) - SDF MBR (for a QER that is referenced by the uplink/downlink Packet Detection Rule of a SDF) - Bearer MBR (for a QER that is referenced by all relevant Packet Detection Rules of a bearer) (NOTE 1). |
| Guaranteed bitrate | The uplink/downlink guaranteed bitrate authorized for the packets. | This field contains: - QoS Flow GBR (for a QER that is referenced by all Packet Detection Rules of a QoS Flow) - Bearer GBR (for a QER that is referenced by all relevant Packet Detection Rules of a bearer) (NOTE 1). |
| Averaging window | The time duration over which the Maximum and Guaranteed bitrate shall be calculated. | This is for counting the packets received during the time duration. |
| Down-link flow level marking | Flow level packet marking in the downlink. | For UPF, this is for controlling the setting of the RQI in the encapsulation header as described in clause 5.7.5.3. |
| QoS Flow ID | QoS Flow ID to be inserted by the UPF. | The UPF inserts the QFI value in the tunnel header of outgoing packets. |
| Paging Policy Indicator | Indicates the PPI value the UPF is required to insert in outgoing packets (see clause 5.4.3.2). | PPI applies only for DL traffic. The UPF inserts the PPI in the outer header of outgoing PDU. |
| Packet rate (NOTE 1) | Number of packets per time interval to be enforced. | This field contains any one of: - downlink packet rate for Serving PLMN Rate Control (the QER is referenced by all PDRs of the UE belonging to PDN connections using CIoT EPS Optimisations as described in TS 23.401 [26]). - uplink/downlink packet rate for APN Rate Control (the QER is referenced by all PDRs of the UE belonging to PDN connections to the same APN using CIoT EPS Optimisations as described in TS 23.401 [26]). |
| End of Data Burst Marking Indication | Indicates to the UPF to provide an End of Data Burst indication of the last PDU of a Data burst to the NG-RAN over GTP-U | NG-RAN can configure UE power management schemes like connected mode DRX when UPF provides an indication of the End of Data Burst, see clause 5.37.8.3. |
| PDU Set Information marking Indicator | Indicates the UPF to insert PDU Set Information related to packets belonging to a PDU Set into GTP-U header. | UPF identifies PDU Sets in DL traffic and forwards PDU Set related information of each PDU to the NG-RAN over GTP-U, as described in clause 5.37.5. |
| ECN marking for L4S indicator | Indicates the UPF to perform ECN marking for L4S for the corresponding QoS Flow. | UPF uses information sent by NG-RAN in GTP-U header extension to perform ECN marking for L4S for the corresponding direction. |
| Multi-modality information marking Indicator | Indicates the UPF to insert Multi-modality information to packets belonging to the Multi-modal service into GTP-U header. | UPF identifies PDUs for Multi-modal service in DL traffic and inserts Multi-modality information of each PDU to the NG-RAN over GTP-U, as described in clause 5.37.X. |
| NOTE 1: This parameter is only used for interworking with EPC. |  |  |

* * * * Start of 4th Change * * * *

5.37.1	General

This clause provides an overview of 5GS functionalities for support of XR services (AR/VR applications) and interactive media services that require high data rate and low latency communication, e.g. cloud gaming and tactile/multi-modal communication services according to service requirements documented in TS 22.261 [2]. The standardized 5QI characteristics for such interactive services are provided in Table 5.7.4-1 and TSCAI is used to describe the related traffic characteristics as defined in clause 5.27.2. Further enhancements for these interactive media services are as follows:

-	The 5GS may support QoS policy control for multi-modal traffic, see clause 5.37.2.

-	The 5GS may support network information exposure which can be based on ECN markings for L4S, see clause 5.37.3 or 5GS exposure API, see clause 5.37.4.

-	The 5GS may support PDU Set based QoS handling including PDU Set identification and marking, see clause 5.37.5.

-	The 5GS may ensure that the UL and DL packets together meet the requested round trip delay and also update the delay for UL and DL considering QoS monitoring results, see clause 5.37.6.

-	The 5GS may perform per-flow Packet Delay Variation (PDV) monitoring and policy control according to AF provided requirements, see clause 5.37.7.

-	The 5GC may provide traffic assistance information to the NG-RAN to enable Connected mode DRX power saving, see clause 5.37.8.

-	The 5GC may provide Multi-modality awareness information to the NG-RAN to support multi-modality awareness at RAN, see clause 5.37.X.

* * * * Start of 5th Change * * * *

5.37.X	Support for Multi-modality awareness at RAN

The following Multi-modality information can be provided by the CN to the NG-RAN in order to support multi-modality awareness at RAN:

-	Multi-modal Awareness ID;

-	Packet level synchronization information (e.g. Time stamp in the RTP/RTCP headers).

The PCF includes the Multi-modal Awareness ID in the PCC rule to the SMF. Based on the PCC rule, for a QoS Flow used for the multi-modal service, the SMF includes Multi-modal Awareness ID to the QoS Profile and instructs PSA UPF to perform Multi-modality information marking (i.e, Multi-modal Awareness ID and Packet level synchronization information).

NOTE 1:	The Multi-modal Awareness ID in the PCC rule is ensured to be unique within the operator's network by the PCF as described in clause 5.7.2.X.

NOTE 2:	Multiple Multi-modal Awareness IDs can be associated with a QoS Flow because the SMF can bind multiple SDFs into a single QoS Flow.

For each DL PDU received on N6 for which Multi-modality information marking is indicated from the SMF, the PSA UPF inserts the Multi-modality information into the GTP-U header as described in clause 5.8.5.4.

* * * * End of Changes * * * *