SA WG2 Meeting #166	S2-2411789

# 18 - 22 November, 2024, Orlando, USA		(revision of S2-2410804)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5657 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of UPF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, [ZTE, Nokia, Samsung] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 4.17.6.1 of TS 23.502, it captures that “the supported operator configurable UPF capabilities as described in clause 5.8.2.X of 23.501”, but the term “operator configurable UPF capability” is used in TS 23.501. Considering that the “operator configurable UPF capability” can typically include multiple UPF capabilities, so it proposes to use “operator configurable UPF capabilities” as the term. As described in clause 4.4.3.2 of TS 23.502, the N4 Association Update Procedure can also be initiated by the UPF or by the SMF to update the supported UPF features. The SMF can also consider the operator configurable UPF capabilities when discovering UPF from NRF. The NAT information exposure functionality and IP or MAC filter-based packet detection functionality can also be used as one of the criteria for UPF selection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the “Operator configurable UPF capability” to “Operator configurable UPF capabilities”. In clause 6.3.3.1, clarify that SMF may also be aware of Operator configurable UPF capabilities during N4 association update procedure. In clause 6.3.3.2, add the operator configurable UPF capabilities as one of the UPF capabilities requested by the SMF from NRF. In clause 6.3.3.3, the NAT information exposure functionality and IP or MAC filter-based packet detection functionality are added as the criteria for UPF selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent terminology with TS 23.502, and some UPF capabilities for UPF selection are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.21, 6.2.3, 6.2.6.2, 6.3.3.1, 6.3.3.2, 6.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 3: 1. Make clarification on partially supported standardized feature(s) in clause 5.8.2.21, to avoid mistaking partially supported standardized feature(s) as one of the example of operator defined non-standardized feature(s). 2. If the required and/or preferred UPF functionalities are received from UDM, the SMF may utilize the NRF to discover UPF(s) and select UPF by considering the required and/or preferred UPF functionalities. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8.2.21	Operator configurable UPF capabilities

UPF may support operator defined non-standardized feature(s) (such as hardware features (e.g. xPUs information (e.g. CPU/GPU information)), computing features (e.g. computing resource type, computing capability), and protection features (e.g. Firewall, DDoS)) or partially supported standardized feature(s). If the UPF supports such features, the UPF indicates the supported features to the SMF using the operator configurable UPF capabilities to the SMF via N4 association setup procedure or N4 association update procedure as described in clause 4.4.3 of TS 23.502 [3], or to the NRF via NF Service Registration procedure or NF service update procedure as described in clause 4.17 of TS 23.502 [3]. SMF can consider the operator configurable UPF capabilities for UPF selection as described in clause 6.3.3.

NOTE:	The operator configurable UPF capabilities can be used for example for partially supported standardized feature(s), e.g. to identify the supported certain option(s) of a standard feature.

# * * * * Next change * * * *

6.2.3	UPF

The User plane function (UPF) includes the following functionality. Some or all of the UPF functionalities may be supported in a single instance of a UPF:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable).

-	Allocation of UE IP address/prefix (if supported) in response to SMF request.

-	External PDU Session point of interconnect to Data Network.

-	Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session, support of traffic forwarding within a 5G VN group (UPF local switching, via N6, via N19)).

-	Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition, IP or MAC filter-based packet detection functionality).

-	User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

-	Lawful intercept (UP collection).

-	Traffic usage reporting.

-	QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

-	Uplink Traffic verification (SDF to QoS Flow mapping).

-	Transport level packet marking in the uplink and downlink.

-	Downlink packet buffering and downlink data notification triggering.

-	Sending and forwarding of one or more "end marker" to the source NG-RAN node.

-	Functionality to respond to Address Resolution Protocol (ARP) requests and / or IPv6 Neighbour Solicitation requests based on local cache information for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

-	Packet duplication in downlink direction and elimination in uplink direction in GTP-U layer.

-	NW-TT functionality.

-	High latency communication, see clause 5.31.8.

-	ATSSS Steering functionality to steer the MA PDU Session traffic, refer to clause 5.32.6.

NOTE:	Not all of the UPF functionalities are required to be supported in an instance of user plane function of a Network Slice.

-	Inter PLMN UP Security (IPUPS) functionality, specified in clause 5.8.2.14.

-	Event exposure, including exposure of network information, i.e. the QoS monitoring information, as specified in clause 5.8.2.18, events as specified in clause 5.2.26.2 of TS 23.502 [3], exposure of data collected for analytics, as specified in clause 5.2.26.2 of TS 23.502 [3] and exposure of the TSC management information as specified in clause 5.8.5.14.

-	Exposure of the UE information, e.g. UE IP address translation information as specified in clause 5.2.26.3 of TS 23.502 [3] and clause 4.15.10 of TS 23.502 [3] if Network address translation (i.e. NAT) functionality of the UE IP address is deployed within UPF.

-	Support of PDU Set Handling as defined in clause 5.37.5.

-	NAT information exposure functionality (if NAT is deployed within UPF) as described in clause 5.8.2.17.

-	Features indicated by the means of supported operator configurable UPF capabilities, as described in clause 5.8.2.21.

# * * * * Next change * * * *

6.2.6.2	NF profile

NF profile of NF instance maintained in an NRF includes the following information:

-	NF instance ID.

-	NF type.

-	PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN.

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID.

-	FQDN or IP address of NF.

-	NF capacity information.

-	NF priority information.

NOTE 1:	This parameter is used for AMF selection, if applicable, as specified in clause 6.3.5. See clause 6.1.6.2.2 of TS 29.510 [58] for its detailed use.

-	NF Set ID.

-	NF Service Set ID of the NF service instance.

-	NF Specific Service authorization information.

-	if applicable, Names of supported services.

-	Endpoint Address(es) of instance(s) of each supported service.

-	Identification of stored data/information.

NOTE 2:	This is only applicable for a UDR profile. See applicable input parameters for Nnrf_NFManagement_NFRegister service operation in clause 5.2.7.2.2 of TS 23.502 [3]. This information applicability to other NF profiles is implementation specific.

-	Other service parameter, e.g. DNN or DNN list, notification endpoint for each type of notification that the NF service is interested in receiving.

-	Location information or serving scope for the NF instance.

NOTE 3:	This information is operator specific. Examples of such information can be geographical location, data centre.

-	TAI(s).

-	NF load information.

-	Routing Indicator, Home Network Public Key identifier, for UDM and AUSF.

-	For UDM, AUSF and NSSAAF in the case of access to an SNPN using credentials owned by a Credentials Holder with AAA Server, identification of Credentials Holder (i.e. the realm of the Network Specific Identifier based SUPI).

-	For UDM and AUSF, and if UDM/AUSF is used for access to an SNPN using credentials owned by a Credentials Holder, identification of Credentials Holder (i.e. the realm if Network Specific Identifier based SUPI is used or the MCC and MNC if IMSI based SUPI is used); see clause 5.30.2.1.

-	For AUSF and NSSAAF in the case of SNPN Onboarding using a DCS with AAA server, identification of DCS (i.e. the realm of the Network Specific Identifier based SUPI).

-	For UDM and AUSF, and if UDM/AUSF is used as DCS in the case of SNPN Onboarding, identification of DCS (i.e. the realm if Network Specific Identifier based SUPI, or the MCC and MNC if IMSI based SUPI).

-	One or more GUAMI(s), in the case of AMF.

-	For the UPF, see clause 5.2.7.2.2 of TS 23.502 [3].

-	UDM Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of internal group identifiers, range(s) of external group identifiers for UDM.

-	UDR Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers for UDR.

-	AUSF Group ID, range(s) of SUPIs for AUSF.

-	PCF Group ID, range(s) of SUPIs for PCF.

-	HSS Group ID, set(s) of IMPIs, set(s) of IMPU, set(s) of IMSIs, set(s) of PSIs, set(s) of MSISDN for HSS.

-	For NWDAF, the following information are supported:

-	Analytics ID(s) (possibly per service).

-	NWDAF Serving Area information (refer to clause 6.3.13).

-	Supported Analytics Delay per Analytics ID (if available).

-	NF types of the NF data sources, NF Set IDs of the NF data sources, if available.

-	Analytics aggregation capability (if available).

-	Analytics metadata provisioning capability (if available).

-	ML model Filter information parameters include S-NSSAI(s) and Area(s) of Interest for the trained ML model(s) per Analytics ID(s).

-	ML Model Interoperability indicator (if available) per Analytics ID(s).

-	FL capability information per analytics ID including FL capability type (i.e. FL server and/or FL client, if available).

-	Time interval supporting FL (if available).

-	Accuracy checking capability for ML model accuracy monitoring or Analytics Accuracy Monitoring (if available).

-	Roaming exchange capability (if available).

NOTE 4:	The NWDAF's Serving Area information is common to all its supported Analytics IDs.

NOTE 5:	The Analytics IDs supported by the NWDAF may be associated with a Supported Analytics Delay i.e. the Analytics report can be generated with a time (including data collection delay and inference delay) in less than or equal to the Supported Analytics Delay.

NOTE 6:	The determination of Supported Analytics Delay, and how the NWDAF avoid updating its Supported Analytics Delay in NRF frequently is NWDAF implementation specific.

-	Event ID(s) supported by AFs, in the case of NEF.

-	Event Exposure service supported event ID(s) by UPF.

-	Application Identifier(s) supported by AFs, in the case of NEF.

-	Range(s) of External Identifiers, or range(s) of External Group Identifiers, or the domain names served by the NEF, in the case of NEF.

NOTE 7:	This is applicable when NEF exposes AF information for analytics purpose as detailed in TS 23.288 [86].

NOTE 8:	It is expected service authorization information is usually provided by OA&M system, and it can also be included in the NF profile in the case that e.g. an NF instance has an exceptional service authorization information.

NOTE 9:	The NRF may store a mapping between UDM Group ID and SUPI(s), UDR Group ID and SUPI(s), AUSF Group ID and SUPI(s) and PCF Group ID and SUPI(s), to enable discovery of UDM, UDR, AUSF and PCF using SUPI, SUPI ranges as specified in clause 6.3 or interact with UDR to resolve the UDM Group ID/UDR Group ID/AUSF Group ID/PCF Group ID based on UE identity, e.g. SUPI (see clause 6.3.1 for details).

-	IP domain list as described in clause 6.1.6.2.21 of TS 29.510 [58], Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, Range(s) of SUPIs or Range(s) of GPSIs or a BSF Group ID, in the case of BSF.

-	SCP Domain the NF belongs to.

-	DCCF Serving Area information, NF types of the data sources, NF Set IDs of the data sources, if available, in the case of DCCF.

-	Supported DNAI list, in the case of SMF.

-	For SNPN, capability to support SNPN Onboarding in the case of AMF and capability to support User Plane Remote Provisioning in the case of SMF.

-	IP address range, DNAI for UPF.

-	Supported operator configurable UPF capabilities (as described in clause 5.8.2.21) in the case of UPF.

-	Supported DNS security protocols, in the case of EASDF.

-	Additional V2X related NF profile parameters are defined in TS 23.287 [121].

-	Additional ProSe related NF profile parameters are defined in TS 23.304 [128].

-	Additional MBS related NF profile parameters are defined in TS 23.247 [129].

-	Additional UAS related NF profile parameters are defined in TS 23.256 [136].

-	Additional Ranging based services and Sidelink Positioning related NF profile parameters are defined in TS 23.586 [180].

-	For additional information in PCF profile, see clause 5.2.7.2.2 of TS 23.502 [3].

# * * * * Next change * * * *

6.3.3.1	Overview

The selection and reselection of the UPF for PDU session establishment, UE mobility or UE traffic offloading are performed by the SMF by considering UPF deployment scenarios such as centrally located UPF and distributed UPF located close to or at the Access Network site. The selection of the UPF shall also enable deployment of UPF with different capabilities, e.g. UPFs supporting no or a subset of optional functionalities.

The SMF may be made aware of UPF capabilities (e.g. operator configurable UPF capabilities, NAT information exposure functionality, IP or MAC filter-based packet detection functionality) during N4 association setup procedure or N4 association update procedure as described in clause 4.4.3 of TS 23.502 [3], or the SMF may utilize the NRF to discover UPF(s) that supports those capabilities as described in clause 4.17 of TS 23.502 [3].

When the UPF selection for PDU session establishment takes place in home routed roaming case, the UPF(s) in home PLMN is selected by SMF(s) in HPLMN, and the UPF(s) in the VPLMN is selected by SMF(s) in VPLMN. The exact set of parameters used for the selection mechanism is deployment specific and controlled by the operator configuration.

The UPF selection for PDU session establishment, UE mobility or UE traffic offloading involves:

-	A step of SMF Provisioning of available UPF(s) (details are described in clause 6.3.3.2). This step may take place while there is no PDU Session to establish and is followed by N4 Node Level procedures defined in clause 4.4.3 of TS 23.502 [3] where the UPF and the SMF may exchange information such as the support of optional functionalities and capabilities.

-	A step of selection of an UPF for a particular PDU Session (details are described in clause 6.3.3.3) which is followed by N4 session management procedures defined in clause 4.4.1 of TS 23.502 [3].

The selection and reselection of the UPF is also performed by an NF (other than the SMF) in order to collect the data from the UPF as defined in clause 5.8.2.17. In this case, the related dedicated UPF is discovered and selected as follows:

-	When the NF consumer or SCP directly subscribes to the UPF (if allowed by the conditions defined in clause 5.8.2.17), the NF consumer or SCP queries the NRF including the related discovery parameters. The NRF returns the UPF(s) which meet(s) the discovery request.

-	When the NF consumer or SCP shall subscribe via the SMF, the NF consumer gets the serving SMF information from the UDM per SUPI, DNN and S-NSSAI. After that, the NF consumer sends a subscription to the indicated SMF and the SMF identifies the related UPF(s) using the parameters of the subscription (e.g. target flow description, AoI, etc.) and transfers the related event subscription information to the identified UPF(s). If the NF consumer does not know the SUPI but only the UE IP address, it may need to invoke the BSF to get the SUPI corresponding to the triplet (UE IP address, DNN and S-NSSAI).

# * * * * Next change * * * *

6.3.3.2	SMF Provisioning of available UPF(s)

SMF may be locally configured with the information about the available UPFs, e.g. by OAM system when a UPF is instantiated or removed, or the SMF may become aware of a UPF via a UPF initiated N4 Association establishment (as described in clause 4.4.3 of TS 23.502 [3]).

NOTE 1:	UPF information can be updated e.g. by OAM system any time after the initial provisioning, or UPF itself updates its information to the SMF via N4 node level procedures anytime after N4 Association establishment.

The UPF selection functionality in the SMF may optionally utilize the NRF to discover UPF(s). In this case, the SMF issues a request to the NRF that may include following parameters: DNN, S-NSSAI, SMF Area Identity, the requested functionalities and capabilities (e.g. ATSSS steering capabilities, functionality associated with high data rate low latency service, NAT information exposure functionality, IP or MAC filter-based packet detection functionality, operator configurable UPF capabilities, etc.). In its answer, the NRF provides the NF profile(s) that include(s) the IP address(es) or the FQDN of the N4 interface of corresponding UPF(s) to the SMF. If required and/or preferred UPF functionalities are received from UDM (see clause 5.2.3.3.1 of TS 23.502 [3]), it will be considered by the SMF for discovering UPF(s).

UPFs may be associated with an SMF Area Identity in the NRF. This allows limiting the SMF provisioning of UPF(s) using NRF to those UPF(s) associated with a certain SMF Area Identity. This can e.g. be used in the case that an SMF is only allowed to control UPF(s) configured in NRF as belonging to a certain SMF Area Identity.

The NRF may be configured by OAM with information on the available UPF(s) or the UPF(s) may register its/their NF profile(s) in the NRF. This is further defined in clause 4.17 of TS 23.502 [3].

# * * * * Next change * * * *

6.3.3.3	Selection of an UPF for a particular PDU Session

The following parameter(s) and information may be considered by the SMF for UPF selection and re-selection:

-	UPF's dynamic load.

-	Analytics (i.e. statistics or predictions) for UPF load, Service Experience analytics and/or DN Performance analytics per UP path (including UPF and/or DNAI and/or AS instance) and UE related analytics (UE mobility, UE communication, and expected UE behavioural parameters) as received from NWDAF (see TS 23.288 [86]), if NWDAF is deployed.

-	UPF's relative static capacity among UPFs supporting the same DNN.

-	UPF location available at the SMF.

-	UE location information.

-	Capability of the UPF and the functionality required for the particular UE session: An appropriate UPF can be selected by matching the functionality and features required for an UE.

-	Data Network Name (DNN).

-	PDU Session Type (i.e. IPv4, IPv6, IPv4v6, Ethernet Type or Unstructured Type) and if applicable, the static IP address/prefix.

-	SSC mode selected for the PDU Session.

-	UE subscription profile in UDM.

-	DNAI as included in the PCC Rules and described in clause 5.6.7.

-	Local operator policies.

-	S-NSSAI.

-	Access technology being used by the UE.

-	Information related to user plane topology and user plane terminations, that may be deduced from:

-	5G-AN-provided identities (e.g. CellID, TAI), available UPF(s) and DNAI(s);

-	Identifiers (i.e. a FQDN and/or IP address(es)) of N3 terminations provided by a W-AGF or a TNGF or a TWIF;

NOTE 1:	A W-AGF or a TNGF may provide Identifiers of its N3 terminations when forwarding over N2 uplink NAS signalling to the 5GC. The AMF may relay this information to the SMF, as part of session management signalling for a new PDU Session.

-	Information regarding the user plane interfaces of UPF(s). This information may be acquired by the SMF using N4;

-	Information regarding the N3 User Plane termination(s) of the AN serving the UE. This may be deduced from 5G-AN-provided identities (e.g. CellID, TAI);

-	Information regarding the N9 User Plane termination(s) of UPF(s) if needed;

-	Information regarding the User plane termination(s) corresponding to DNAI(s).

-	RSN, support for redundant GTP-U path or support for redundant transport path in the transport layer (as in clause 5.33.2) when redundant UP handling is applicable.

-	Information regarding the ATSSS Steering Capability of the UE session (e.g. any combination of ATSSS-LL capability, MPTCP capability, MPQUIC capability) and information on the UPF support of RTT measurements without PMF.

-	Support for UPF allocation of IP address/prefix.

-	Support of the IPUPS functionality, specified in clause 5.8.2.14.

-	Support for High latency communication (see clause 5.31.8).

-	Support for functionality associated with high data rate low latency services, eXtended Reality (XR) and interactive media services, specified in clause 5.37 (for example, ECN marking for L4S, specified in clause 5.37.3, PDU Set Marking, specified in clause 5.37.5, UE power saving management, specified in clause 5.37.8).

-	User Plane Latency Requirements within AF request (see clause 5.6.7.1 and clause 6.3.6 of TS 23.548 [130]).

-	List of supported Event ID(s) for exposure of UPF-related information via service based interface (see clause 7.2.29 and clause 5.2.26.2 of TS 23.502 [3]).

-	Information regarding required and/or preferred UPF functionalities (see clause 5.2.3.3.1 of TS 23.502 [3]). If received from UDM, the SMF selects a PSA UPF supporting the required UPF functionalities and the best set of preferred functionalities based on their priorities.

-	Support of NAT information exposure functionality.

-	Support of IP or MAC filter-based packet detection functionality.

-	Supported operator configurable UPF capabilities as described in clause 5.8.2.21.

NOTE 2:	How the SMF determines information about the user plane network topology from information listed above, and what information is considered by the SMF, is based on operator configuration.

NOTE 3:	In this release the SMF uses no additional parameters for UPF selection for a PDU Session serving TSC or Deterministic Networking. If a PDU Session needs to connect to a specific UPF hosting a specific TSN 5GS bridge or 5GS router, this can be achieved e.g. by using a dedicated DNN/S-NSSAI combination.

If there is an existing PDU Session, and the SMF receives another PDU Session request to the same DNN and S-NSSAI, and if the SMF determines that interworking with EPC is supported for this PDU Session (as specified in clause 4.11.5 of TS 23.502 [3]), the SMF should select the same UPF if it supports all capabilities required for the new PDU Session. Otherwise, if the SMF determines that interworking with EPC is not supported for the new PDU Session or the UPF of the existing PDU Session does not support all capabilities required for the new PDU Session, a different UPF may be selected according to operator policy.

For the same DNN and S-NSSAI if different UPFs are selected at 5GC, when the UE is moved to EPC network, there is no requirement to enforce APN-AMBR. Whether and how to apply APN-AMBR for the PDN Connection associated with this DNN/APN is implementation dependent, e.g. possibly only AMBR enforcement per PDU Session applies.

# * * * * End of changes * * * *