3GPP TSG-WG SA2 Meeting #166	S2-2411818

# Orlando, USA, 18 – 22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3837 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PWS support for NB-IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The LS R2-2409243 to SA2 mentioned below: A revised WID was approved in RAN#105 (Sept 2024) to further enhance the support of NB-IoT and eMTC for NTN in RP-242397, with the following with the following new objective added: - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome. SA2 needs to update corresponding specfications to support the PWS for NB-IoT NTN. Current SA2 has designed a complete warning message delivery mechanism and procedure as described in TS 23.041. The MME is supported to be triggered by the CBC (Cell Broadcast Centre) for Warning message delivery and notify the eNodeB of sending PWS notification. The warning message delivery mechanism and procedure in SA2 are common for all PWS services (e.g., ETWS, CMAS (aka WEA), KPAS and EU-Alert warning messages). The general design for the warning message delivery mechanism and procedure doesn’t differentiate specific UE type and specific RAT type, it can be applied to all kinds of UE that support PWS feature. Therefore, there is no significant updates for PWS support NB-IoT from the warning message delivery mechanism and procedure. It just only revises the “not support” to “support” in above table to apply to NB-IoT directly. The left work relies on RAN groups on the PWS notification on the Uu interface enhancement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Warning message delivery: - Change “No” to “yes” for PWS support NB-IoT, apply CP and UP both, optional to UE and NW (as indicated 36.331, PWS feature is operional for UE support) - Add NOTE to state that: PWS support NB-IoT is not applied when UE is operating eDRX, PSM or S&F mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Doesn’t support PWS for NB-IoT which is not met the requirment of RAN WID(RP-242397). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

Annex M (informative):
Functions and procedures over NB-IoT RAT

In the case of conflict between the information in this Annex and other information in the main body of the present document, the information in the main body takes precedence.

The following tables list the functions and procedures that are:

-	Supported or not supported over NB-IoT RAT, including whether for CP CIoT EPS Optimisation only, UP CIoT EPS Optimisation only or both.

-	Optional for the UE and/or network when using NB-IoT RAT.

NOTE:	The tables M-1 to M-5 are ordered by clause number according to the present specification. The table M-6 is ordered by clause number according to TS 23.682 [74]. The notation "CP/UP/Both" indicates whether a particular item is supported for CP CIoT EPS Optimisation only, UP CIoT EPS Optimisation only or both.

Table M-1: Clause 4 Concepts

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 4.3.3 | IP header compression |  |  |  |  |
|  | RoHC (NAS) | Yes | CP | Optional | Optional |
|  | RoHC (AS) | Yes | UP | Optional | Optional |
| 4.3.5 | Mobility management functions | Yes | Both |  |  |
|  | Inter-RAT idle mode NAS mobility (WB-E_UTRAN, UTRAN, GERAN) | Yes | Both |  |  |
|  | Reachability management | Yes | Both | ISR Optional | ISR Optional |
|  | TA list management (2) | Yes | Both |  |  |
|  | ISR | Yes | Both |  | Optional |
|  | Mobility restrictions | Yes | Both |  |  |
|  | IMS voice over PS session supported indication | No | N/A | N/A | N/A |
|  | Voice domain preference and UE usage setting | No | N/A | N/A | N/A |
|  | Preferred and supported network behaviour | Yes | Both |  |  |
| 4.3.7.1a.2 | GTP-C Overload Control | Yes | CP | N/A |  |
| 4.3.7.4 | MME control of overload | Yes | Both | N/A |  |
|  | Extended Access Barring | No | N/A | N/A | N/A |
| 4.3.7.4.2.7 | Control Plane data specific NAS level congestion control | Yes | CP |  |  |
| 4.3.12 | IMS emergency session | No | N/A | N/A | N/A |
| 4.3.12a | Support of Restricted Local Operator Service | No | N/A | N/A | N/A |
| 4.3.13 | Closed Subscriber Group | No | N/A | N/A | N/A |
| 4.3.17 | Support for MTC |  |  |  |  |
|  | Low access priority | Yes | Both |  | Optional |
|  | Non-IP data delivery (NIDD) | Yes | Both |  |  |
|  | NIDD SCEF | Yes | CP | N/A (1) | Optional (3) |
|  | NIDD SGi | Yes | Both | N/A (1) | Optional (3) |
| 4.3.18 | Multimedia Priority Service | No | N/A | N/A | N/A |
| 4.3.22 | Power Save Mode | Yes | Both | Optional | Optional |
| 4.3.27a | Restriction of use of Enhanced Coverage for voice centric UE | No | N/A | N/A | N/A |
| 4.3.27b | Restriction of use of Enhanced Coverage for data centric UE | No | N/A | N/A | N/A |
| 4.3.28 | Restriction of use of Enhanced Coverage | Yes | Both |  | Optional |
| 4.3.29 | 3GPP PS Data Off | Yes | Both | Optional | Optional |
| 4.7 | QoS |  |  |  |  |
|  | PDN connection to SCEF | Yes | CP |  | Optional |
|  | Dedicated bearer | No | N/A | N/A | N/A |
|  | GBR bearer | No | N/A | N/A | N/A |
|  | Non-GBR bearer | Yes | UP |  |  |
|  | Rate control (Serving PLMN) | Yes | CP |  | Optional |
|  | Rate control (APN) | Yes | Both |  | Optional |
| 4.7.8 | Inter-UE QoS for NB-IoT UEs using Control Plane CIoT EPS Optimisation | Yes | Yes | N/A | Optional |
| 4.10 | CIoT EPS Optimisation |  |  |  |  |
|  | UP | Yes | UP | Optional | Optional |
|  | CP | Yes | CP |  |  |
| NOTE 1: Whether the non-IP PDN connection is provided via SCEF or over SGi is transparent to the UE. NOTE 2: Required for inter-RAT idle mode mobility. NOTE 3: At least one of NIDD SCEF and NIDD SGi is required. |  |  |  |  |  |

Table M-2: Clause 5.3 Authentication, security and location management

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 5.3.1 | IP address allocation (1) | Yes | Both | Not required for NIDD |  |
| 5.3.2 | Attach procedure | Yes | Both |  |  |
|  | Normal attach | Yes | Both | Optional (2) |  |
|  | Attach without PDN connectivity | Yes | Both | Optional (2) | Optional |
|  | Emergency attach | No | N/A | N/A | N/A |
|  | SMS transfer without Combined Attach | Yes | Both |  | Optional |
| 5.3.3 | TAU procedure | Yes (3) | Both |  |  |
| 5.3.4 | Service request procedures | Yes | Both (4) |  |  |
| 5.3.4A | Connection Suspend procedure | Yes | UP |  |  |
| 5.3.4B | Data transport in CP CIoT EPS Optimisation | Yes | CP |  |  |
|  | Reliable data delivery with hop-by-hop acknowledgement | Yes | CP | N/A | Optional |
| 5.3.5 | S1 release procedure | Yes (5) |  |  |  |
| 5.3.5A | Connection resume procedure | Yes | UP |  |  |
| 5.3.6A | PDN GW pause of charging procedure | Yes | Both | N/A | Optional |
| 5.3.7 | GUTI reallocation procedure | Yes | Both |  |  |
| 5.3.8 | Detach procedure | Yes (6) | Both |  |  |
| 5.3.10 | Security function | Yes | Both |  |  |
|  | AS security | Yes | UP |  |  |
|  | NAS security | Yes | Both |  |  |
| 5.3.14 | UE radio capability match request | No (7) | N/A | N/A | N/A |
| NOTE 1: Incl. attach without PDN connectivity. NOTE 2: At least one of Normal attach and Attach without PDN connectivity is required. NOTE 3: Incl. inter RAT. NOTE 4: Also supported with CP CIoT EPS Optimisation when the UE/MME also support S1-U data transfer or UP CIoT EPS Optimisation. NOTE 5: Releases the S11-U bearer in the case of CP CIoT EPS Optimisation. RRC Connection release is used with the UE. NOTE 6: ISR aspects optional. NOTE 7: IMS Voice not supported over NB-IoT. |  |  |  |  |  |

Table M-3: Clause 5.4 Session management, QoS and interaction with PCC

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 5.4.1 | Dedicated bearer activation | No | N/A | N/A | N/A |
| 5.4.2 | Bearer modification with QoS update | Yes | UP | Optional | Optional |
| 5.4.3 | PDN GW initiated bearer modification without bearer QoS update | Yes | UP |  |  |
| 5.4.4 | Bearer deactivation | Yes | UP |  |  |
| 5.4.5 | UE-requested bearer resource modification | Yes | Both | Optional with CP CIoT EPS Optimisation |  |
| 5.4.7 | E-UTRAN initiated ERAB modification procedure | No (1) | N/A | N/A | N/A |
| 5.4.8 | E-UTRAN initiated UE context modification procedure | No (1) | N/A | N/A | N/A |
| NOTE 1: Dual connectivity not supported over NB-IoT RAT. |  |  |  |  |  |

Table M-4: Clauses 5.5 and 5.6 Handover and NACC

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 5.5 | Handover | No |  |  |  |
| 5.5.1 | Intra-E-UTRAN handover | No | N/A | N/A | N/A |
| 5.5.2 | Inter RAT handover | No | N/A | N/A | N/A |
| 5.6 | Network assisted cell change | No | N/A | N/A | N/A |

Table M-5: Misc.

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 5.8 | MBMS (3) | (3) | (3) | (3) | (3) |
| 5.10 | Multiple PDN support and PDN activation for UEs supporting Attach without PDN connectivity | Yes (1) | Both |  |  |
| 5.11 | UE capability handling | Yes | Both |  |  |
| 5.12 | Warning message delivery | Yes(4) | Both | Optional | Optional |
| 5.13 | DRX and UE-specific DRX parameter handling | Yes (2) | Both |  |  |
| 5.13a | Extended idle mode DRX | Yes | Both | Optional | Optional |
| 5.15 | RAN information management procedures | No | N/A | N/A | N/A |
| 5.16 | MME-initiated procedure on UE's CSG membership change | No | N/A | N/A | N/A |
| 5.17 | HeNB multicast packet forwarding function | No | N/A | N/A | N/A |
| NOTE 1: Multiple PDN optional. NOTE 2: UE-specific DRX not supported in NB-IoT RAT. NOTE 3: See Table M-6. |  |  |  |  |  |

Table M-6:MTC/CIoT-related items in TS 23.682 [74]

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 4.5.14.3 | Reliable Data Service | Yes | CP | Optional | Optional |
| 4.5.15, 5.14 | Support of Packet Flow Description management via SCEF | Yes | Both | N/A | Optional |
| 4.5.16 | MSISDN-less MO-SMS via T4 | Yes | Both | Optional | Optional |
| 4.5.18 | MBMS user service for UEs using power saving functions | Yes | Both | Optional | Optional |
| 4.5.19 | Enhancements to Location Services for CIoT | Yes | Both | Optional | Optional |
| NOTE: This table lists MTC/CIoT-related items in TS 23.682 [74] not otherwise specified in the present specification. |  |  |  |  |  |

* * * End of Changes * * * *