3GPP TSG-WG SA2 Meeting #166	S2-2411922

# Orlando, USA, 18th Nov – 22nd Nov, 2024		(revision of S2-24010478)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5790 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of PDU Set based handling when no DL PDU Set QoS parameters are received |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the last meeting, KI#1 on other PDU Set handling has been concluded, specifically how to support PDU Set based handling when no DL PDU Set QoS parameters are received. The CR proposes to add the conclusion in TS 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds principles of KI#1 conclusion into TS 23.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to perform PDU Set based handling if no DL PDU Set QoS parameters are received is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.7.1, 5.7.7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.7.7.1	General

PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN.

The following PDU Set QoS Parameters are specified:

1.	PDU Set Delay Budget (PSDB).

2.	PDU Set Error Rate (PSER).

3.	PDU Set Integrated Handling Information (PSIHI).

4.	PDU Set Information Marking Only (PSMO)

One of the following shall be sent to the NG-RAN to enable PDU Set based handling: 1) PSIHI, 2) PSDB and PSER, 3) PSIHI, PSDB and PSER or 4) PSMO. For a given QoS Flow, the values of PSDB, PSER and PSIHI can be different for UL and DL. The PSMO can only be provided when neither the PSIHI nor the combination of PSDB and PSER is available.

The QoS Profile (see clause 5.7.1.2) may include the PDU Set QoS Parameters described in this clause for UL and/or DL direction. The PCF determines the PDU Set QoS Parameters based on information provided by AF and/or local configuration. The PDU Set QoS parameters are sent to the SMF as part of PCC rule. The SMF sends them to NG-RAN as part of the QoS Profile. In case no PDU Set QoS parameters are sent to the SMF as part of PCC rule but a Protocol Description is received, the SMF can send the PSMO to the NG-RAN if the SMF decides to enable PDU Set Information Marking whenever possible based on the local configuration.

If the NG-RAN receives PDU Set QoS Parameters, it enables the PDU Set based QoS handling and applies PDU Set QoS Parameters as described in TS 38.300 [27], TS 38.413 [34] and TS 38.331 [51].

# * * * * Second change * * * *

#### 5.7.7.x	PDU Set Information Marking Only

The PDU Set Information Marking Only (PSMO) indicates that PDU Set Information Marking is enabled while there are no specific requirements for the handling of PDU Sets in the NG-RAN. PSMO is an optional parameter generated by the SMF. A QoS Flow can only be associated with the PSMO for the DL direction.

# * * * * End of changes * * * *