3GPP TSG-WG SA2 Meeting #166	S2-2411970

18th – 22nd November, 2024, Orlando, US

Source:	Huawei, HiSilicon

Title:	Discussion about IP (re)allocation during Multi-hop U2N Relay Reselection

Document for:	Discussion

Agenda Item:	19.7.2

Work Item / Release:	5G_ProSe_Ph3 / Rel-19

Abstract: Discuss whether and how to trigger U2N Relay to (re)allocate IP address/prefix during Multi-hop U2N Relay Reselection.

# 1. Introduction

## 1.1. Overview

In the latest TS 23.304, two types of situations for parent relay reselection are supported, i.e. for a single 5G ProSe Remote UE, and for a 5G ProSe Intermediate UE-to-Network Relay with multiple child relays or 5G ProSe Remote UEs. For the case when a new 5G ProSe UE-to-Network Relay is used after the relay reselection, IP address/prefix changes may happen (in case of Layer-3 UE-to-Network relay).

However, the current enhancement of Layer-2 link modification procedure for multi-hop U2N Relay is insufficient for the case mentioned above.

This paper first discusses the problem of the current procedure in some specific situation and then gives out some possible solutions.

# 2. Discussion

As described in TS 23.304 clause 6.4.3.8.3:

##### 6.4.3.8.3 Relay reselection and mobility support There are two types of situations for parent relay reselection, i.e. for a single 5G ProSe Remote UE, and for a 5G ProSe Intermediate UE-to-Network Relay with multiple child relays or 5G ProSe Remote UEs. For a single 5G ProSe Remote UE, when the connection with an old parent relay (either a 5G ProSe Intermediate UE-to-Network Relay or a 5G ProSe UE-to-Network Relay) is lost or the link quality drops below a configured threshold, the 5G ProSe Remote UE triggers the reselection of the relays that offers the same service, identified by the same RSC. Figure 6.4.3.8.3-1: Relay reselection for 5G ProSe Remote UE with 5G ProSe Multi-hop UE-to-Network Relays Figure 6.4.3.8.3-1 depicts the Relay reselection operation for this situation. When the 5G ProSe Remote UE has lost the connection or has an imminent connection loss/deteriorated connection with the 5G ProSe Intermediate UE-to-Network Relay-1, the 5G ProSe Remote UE performs relay reselection by using the relay discovery procedure in clause 6.3.2.5. The 5G ProSe Remote UE discovered the 5G ProSe Intermediate UE-to-Network Relay-2 that offers the same RSC. In case there are multiple 5G ProSe Intermediate UE-to-Network Relays discovered, the 5G ProSe Intermediate UE-to-Network Relay that offers connection to the same 5G ProSe UE-to-Network Relay is prioritized. 5G ProSe Remote UE establishes the Layer-2 link with the 5G ProSe Intermediate UE-to-Network Relay-2 as specified in clause 6.4.3.8.1. After the connection establishment, the Intermediate UE-to-Network Relay-2 uses the Layer-2 Link Modification procedure to update the 5G ProSe UE-to-Network Relay regarding the 5G ProSe Remote UE. If the same 5G ProSe UE-to-Network Relay is used, it switches the connection/routing context for the 5G ProSe Remote UE from the Layer-2 link with the 5G Intermediate UE-to-Network Relay-1 to that of the Layer-2 link with the 5G ProSe Intermediate UE-to-Network Relay-2. The 5G ProSe Remote UE tears down the link with the 5G ProSe Intermediate UE-to-Network Relay-1 (or the link failure is detected), which will trigger the 5G ProSe Intermediate UE-to-Network Relay-1 to update the 5G ProSe UE-to-Network Relay to remove its association with 5G ProSe Remote UE. If the same 5G ProSe UE-to-Network Relay is selected after the relay reselection, the 5G ProSe Remote UE can keep its IP address/prefix (in case of Layer-3 UE-to-Network relay). If a new 5G ProSe UE-to-Network Relay is used after the relay reselection, the procedures as that described in clause 6.4.3.8.1 will be executed. In this case, IP address/prefix changes may happen (in case of Layer-3 UE-to-Network relay). The relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure.

Observation# 1): The solution works for the scenario shown in Figure 6.4.3.8.3-1. But except for the situation shown in Figure 6.4.3.8.3-1, there is another situation that also correspond to the parent relay reselection type of “a 5G ProSe Intermediate UE-to-Network Relay with multiple child relays or 5G ProSe Remote UEs.”

Intermediate R0

Intermediate R1

Intermediate R2

Intermediate R3

UE1

U2N Relay1

U2N Relay2

Figure 2.1 Parent Relay reselection between Intermediate U2N Relay and its parent Relay

As shown in Figure 2.1, the Intermediate U2N Relay that directly connects to the Remote UE1 (R0) is not changed while the parent Relay (R3→R2) of an intermediate U2N Relay (R1) is changed and a new Root U2N Relay (Relay2) is selected.

In this case, “the procedures as that described in clause 6.4.3.8.1 will be executed”, i.e. the Layer-2 link modification procedure for multi-hop U2N Relay is executed as highlighted:

##### 6.4.3.8.1 Layer-2 Link establishment and management (based on Model A discovery) Each of the 5G ProSe Intermediate UE-to-Network Relay establishes a Layer-2 Link with its immediate parent Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, before it can serve the 5G ProSe Remote UE, or a child Intermediate UE-to-Network Relay. RSC based configuration for the 5G ProSe Intermediate UE-to-Network Relay indicates whether it should establish the Layer-2 Link with a parent relay before it can participate in the discover operation. Layer-2 link management procedures as defined in clause 6.4.3.6 for 5G ProSe UE-to-Network Relay operation are used. When establishing the Layer-2 link with its parent relay (either a 5G ProSe UE-to-Network Relay or another 5G ProSe Intermediate UE-to-Network Relay), the 5G ProSe Intermediate UE-to-Network Relay takes the role of the 5G ProSe Remote UE and its parent relay takes the role of 5G ProSe UE-to-Network Relay. The Layer-2 link is established per RSC. The 5G ProSe Remote UE uses the procedure defined in clause 6.4.5.6 to establish a Layer-2 Link with the selected 5G ProSe Intermediate UE-to-Network Relay. The Intermediate UE-to-Network Relay takes the role of 5G ProSe UE-to-Network Relay in the procedure. NOTE: In the case of Layer-3 UE-to-Network Relay operation, only the root 5G ProSe UE-to-Network Relay needs to establish the PDU session with the network. When an Intermediate UE-to-Network Relay has a child Intermediate UE-to-Network Relay or a 5G ProSe Remote UE connected, it updates the Layer-2 link with its parent relay, using the procedure defined in clause 6.4.3.4, with the following enhancements: - The link modification procedure is enhanced to inform the parent relay, e.g. another 5G ProSe Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, of the User Info of the child relay(s) or the Remote UE(s). The Link Modification Request message contains a list of 5G ProSe Remote UE (Remote User ID, Remote UE Info) and the corresponding PC5 QoS Flow associated with each 5G ProSe Remote UE. The Remote User ID and Remote UE Info are as defined in clause 6.5.1.1. An Intermediate UE-to-Network Relay triggers a link modification procedure towards its parent relay if a new Remote User ID is received from its child relay during the link modification procedure. The 5G ProSe UE-to-Network Relay's Remote UE reporting procedure includes all the Remote UE information reported by its child relays.

Observation# 2): The Remote User ID and Remote UE Info of Remote UE1 is informed by R2 (which has multiple IM relay & Remote UE1 connected) to the Root U2N Relay2, where the Remote UE Info is IP Info for L3 U2N Relay.

The IP info of Remote UE1 was allocated by the ole Root U2N Relay1 as described in clause 6.4.3.8.2:

##### 6.4.3.8.2 IP address/prefix management for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay When IP based PDU session types are used by the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay needs to act as an IP Router. The actual IP address allocation is performed by the 5G ProSe UE-to-Network Relay, as defined in clause 6.5.1. To support DHCP based IP address configuration, the 5G ProSe Intermediate UE-to-Network Relay acts as a DHCPv4 or DHCPv6 proxy. To support IPv6 SLAAC, the 5G ProSe Intermediate UE-to-Network Relay also relays the Router Solicitation and Router Announcement messages between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay. The 5G ProSe Intermediate UE-to-Network Relay builds its local IP routing table based on the Remote UE info from the link modification procedures as described in clause 6.4.3.8.1. The local IP routing table is per RSC and used based on RSC of the link the packet is received.

Observation# 3): In the case shown in Figure 2.1, the new Root U2N Relay2 receives the User Info ID of the Remote UE1 and the IP address/prefix of the Remote UE1 which was allocated by the old Root U2N Relay. Besides, the Remote UE1 may not notice the link between R1 and R2/R3 has changed as currently no other enhancement/procedure was defined for the L2 Link Modification.

This leads to a disruption of IP communication because Remote UE1 does not have a correct IP addr/prefix allocated by U2N Relay1.

Thus, the current enhancement of L2 Link Modification for multi-hop Relay is insufficient for Relay reselection.

The solution could be simple, either R1 informs the Remote UE(s) about the path switch related to Root Relay change and triggers UE1 to request for a new IP address/prefix, or the new U2N Relay triggers the (re-)allocation of IP address/prefix.

Option# 1): Clarify that the Intermediate U2N Relay (R1) also informs Remote UE(s) when it decides that the Root U2N Relay is changed. That is,

The intermediate U2N Relay R1 stores the Root Relay Info that it has established link to. When the PC5 connection between R1 and its parent Relay changes, it determines whether the Root U2N Relay is changed. If so, R1 informs UE1 so as to trigger UE1 to request for a new IP address from the new Root U2N Relay. R1 initiates Link Modification as described in clause 6.4.3.8.1 after it receives the new IP Info of UE1.

Option# 1A): R1 informs UE1 by sending Link Modification message which includes an indication that Root U2N Relay is changed.

Option# 1B): R1 informs UE1 by sending a DHCP FORCERENEW message.

Option# 2): Clarify that the Intermediate U2N Relay (R1) informs Remote UE(s) when it decides that the its parent Intermediate Relay or U2N Relay is changed, the Remote UE decides whether the Root U2N Relay is changed. That is,

R1 informs UE1 by sending Link Modification message which includes the User Info ID of the connected Root U2N Relay. R1 learns the User Info ID of Root U2N Relay from the Discovery procedure. When the Remote UE decides the Root U2N Relay is changed, it requests for a new IP address from the new Root U2N Relay.

Option# 3): Clarify that the new Root U2N Relay (Relay2) decides if a new Remote UE is connected and triggers the Root U2N Relay to (re-)allocate IP address/prefix to the Remote UE.

The new Root U2N Relay checks if Remote UE1 is newly connected, e.g. based on the User Info ID or based on whether the IP is allocated by itself to the Remote UE1. If it is newly connected, the new Root U2N Relay needs to allocate a new IP address/prefix and sends it to Remote UE1 through PC5 signalling, e.g. Link Modification Ack or Link Identifier Update, including the allocated IP and the associated User Info ID of Remote UE1.

Option# 4): Clarify that the Intermediate U2N Relay (R1) request for a new IP address/prefix for the Remote UE when it decides that the Root U2N Relay is changed. Then R1 sends PC5 signalling or DHCP FORCERENEW to trigger Remote UE to update its IP address/prefix.

Evaluation:

For Option#1A, the Link Modification message is enhanced to include a new indication.

For Option#1B, the DHCP message is reused, but the Intermediate Relay not only just acts as DHCP Proxy. The Remote UE is also required to support of handling FORCERENEW message.

For Option#2, the Link Modification procedure is triggered whether the Root U2N Relay is changed or not, which may cause additional PC5 signalling overhead.

For Option#3, it saves the message overhead as there is no need for the Remote UE to initiate IP request and Link Modification procedure. Link Modification Ack message is enhanced with new function, i.e. to force reallocate IP address/prefix. Besides, it can be tricky how the U2N Relay decides the Remote UE is “newly connected”. E.g., the Link Modification procedure is also initiated by R3 to delete Remote UE1. Even if the Root Relay is not changed, how does it consider the Remote UE1 as “newly connected”?

For Option#4, it saves the message overhead as the Intermediate U2N Relay requests an IP address/prefix for Remote UE(s) in advance. But the Intermediate U2N Relay is enhanced to carry mixed function of DHCP client & proxy & server, or SLAAC client & router & server, and the enhancement included in Option#1A/1B is also required.

To sum up, Option#1A is an easy and direct way. Option#2 is simpler for the Intermediate Relay and may also be used for link management after Model B discovery which needs the path information. The implementation of Option#3 and Option#4 can be complicated.

# 3. Proposal

Proposal# 1): Take Option#1A to solve the discussed problem.