3GPP TSG-SA WG2#166	S2-2411974

Orlando, US, November 18 – 22, 2024	(Revision of S2-240xxxx)

Source:	Sony

Title:	KI#2: Interim Conclusion on Registration Management

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution proposes interim conclusion on Registration Management.

# 1.	Discussion

Several solutions in TR 23.700-13 refers to an initial step 0, that assumes Onboarding/Registration of AIoT Devices, other solutions provide details, more or less elaborate, for an onboarding/registration procedure. The solutions that provide information related to registration of AIoT devices can be divided into different categories/options.

Option 1: No registration procedure.

Registration of devices in the network that does not involve the AIoT device. E.g. solution 7 proposes: “Bulk Introduction of Devices to the Network Request sent by AF to NEF. Bulk Introduction of Devices to the Network Request includes AF ID, list of device IDs, Registration Type. The Registration type can be Group Initial Registration.”

This option assumes that the AIoT device has been pre-configured will all parameters needed to communicate with the network and that there is no need for further configurations by the network. As the AIoT device has no knowledge if the device is allowed to communicate in the network, all AIoT device must assume that they shall communicate if triggered.

Option 2: “One-way” registration.

The registration of an AIoT device is done by sending a write command to the AIoT device. E.g. solution 39 proposes that the command includes a “frequency configuration”. The AIoT device(s) that receives the command must store the configuration update and send an acknowledgement as a response. After receiving the configuration update, the AIoT device is allowed to respond to AIoT service requests i.e., Inventory or Command service.

Option 3: Registration procedure.

The registration procedure is triggered by the network e.g. with a registration poll, activation signal or Inventory (solution 8, 9, 10, 16, 18, 24, 27, 29, 36 and 37). Solution 30 and 33 proposes that registration procedure is performed only by AIoT devices that have this capability. Details on the registration procedure are different between the solution proposals, but all solution involves updating the AIoT device with further parameters/configuration. After performing the registration procedure, the AIoT device is allowed to respond to AIoT service requests i.e., Inventory or Command service.

Proposal 1: The final conclusion on the registration procedure/management must consider other working groups conclusions. E.g. SA3 might conclude that an initial authorization procedure is required, or security parameters must be provided after the device is authorized. Initial authorization and security configuration is traditionally part of a registration procedure.

Proposal 2: Agree on some interim conclusions/principles/options in this meeting. This could be that the interim conclusions keep several optional until SA2 understands other WGs requirements.

# 2.	Text proposal

It is proposed to agree the following changes to TR 23.700-13:

# >>>>BEGINNING OF CHANGES<<<<

### 8.2	Interim Conclusion on Key Issue #2

### 8.2.x	AIoT device Registration

Editor’s Note: Conclusions on registration management have strong dependence to other working groups e.g. SA3. Conclusions in other working groups may require a registration procedure. Several options are listed below, but it is expected that not all are specified.

The following aspects and principles are agreed for the interim conclusion on AIoT device registration management:

Opt1: No Registration.

- An AF provides the AIOTF with a list of device IDs. These AIoT devices shall be allowed to communicate in the network, and responses from them shall be forwarded to the AF.

- The AF may also provide AIOTF or other NF with parameters needed for the communication between the AIoT device and the network

NOTE 1: Details of the parameters are to be defined during the normative work

- It is assumed that the AIoT device has all parameters pre-configured before deployed and no further configuration is needed.

Opt2: “One-way” Registration.

- An AF provides the AIOTF with a list of device IDs. These AIoT devices shall be allowed to communicate in the network, and responses from them shall be forwarded to the AF.

- The AF may also provide AIOTF or other NF with parameters needed for the communication between the AIoT device and the network.

- AIoT is not pre-configured with all parameters before deployed.

- AIoT device is registered/configured by the AIOTF using write command and the device acknowledge the command. The write command includes parameters to be stored in the AIoT device.

NOTE 2: Details of the parameters provided to the AIOTF and AIoT device are to be defined during the normative work.

Opt3: Registration procedure triggered by network.

- AIOTF may or may not have a list of device IDs prior to recieving a registration request for an AIoT device,

- The AF may have provided AIOTF or other NF with parameters needed for the registration/communication between the AIoT device and the network prior to recieveing the registration request. Or the AIOTF can on demand retrieve the information/parameters from the AF.

- An AIoT device is pre-configured with parameters needed to perform registration.

- AIoT device detects a Registration trigger and responds with a registration request message.

- AIOTF configures the authorized AIoT device with parameters needed to communicate.

NOTE 3: Details of the parameters to the AIOTF and AIoT device are be defined during the normative work

NOTE 4: The Authorization may be done by the CN or 3rd party AAA-server, details to be specified by SA3.

# >>>>END OF CHANGES<<<<