3GPP TSG-SA WG2 Meeting #166	S2-2411987

# Orlando, USA, November 18-22, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1521 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility procedure for UE-Satellite-UE communication in IMS – continuation of optimized media routing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the approved 5GSAT_Ph3_ARCH work item and the conclusion of FS_5GSAT_Ph3_ARCH captured in clause 8.3 of TR 23.700-29, this CR adds description on Mobility procedure of UE-Satellite-UE communication in IMS focusing on continuation of optimized media routing. This CR is written based on the following principle as well as the above-mentioned conclusion. - Use of the N5 interface between IMS and 5GC is assumed. - IMS AS, requested by P-CSCF via SIP MESSAGE, sends SIP re-INVITE to complete IMS AGW relocation. (P-CSCF does not need to act as SIP B2B UA.) - No UE impact. - No new SDP attribute is introduced. (A new SIP header is introduced for conveying satellite ID.) - Subscription data is not considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Mobility procedure of UE-Satellite-UE communication in IMS focusing on continuation of optimized media routing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.5.2 (new), A.5.2.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented after TS 23.228 CR 1428. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

AX.5.2	IMS AGW relocation and media routing path change due to change of satellites

AX.5.2.1	Continued optimized media routing procedure

Figure AX.5.2.1-1 depicts a signalling flow diagram for continuation of optimized media routing after change of satellites that serve a UE. The figure is written in such a way that change of satellite occurs in the originating network for the purpose of the explanation, while change of satellite can occur in the terminating network as well.

NOTE 1:	IMS entities not relevant for the procedure are omitted below for brevity of the description.

Use of the N5 interface between IMS and 5GC is assumed.

Based on the procedure described in clause AX.5.1, P-CSCF is expected to subscribe from 5GC for the early and the late notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic by using a scheme defined in TS 23.501 [93] and TS 23.502 [94].

Figure AX.5.2.1-1:	Continued optimized media routing procedure

The steps in the call flow are as follows:

1.	P-CSCF receives the early notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic as defined in TS 23.502 [94]. This early notification contains satellite ID of a target satellite that has gNB to which the UE gets connected and indicates that 5GC is prepared to change the user plane paths for optimized media routing to the one through this target satellite.

2.	P-CSCF determines that optimized media routing continues to be possible based on the satellite ID received in step 1 for the originating network and the satellite ID stored for the terminating network. P-CSCF determines to continue activating optimized media routing.

NOTE 2:	How P-CSCF uses the satellite IDs to derive whether the two satellites are connected and whether optimized media routing is possible are not specified.

If P-CSCF determines that optimized media routing cannot continue, P-CSCF follows the ground fallback procedure as defined in clause AX.5.2.2 for subsequent steps.

3.	P-CSCF requests IMS AGW on the target satellite to configure the IP address allocated in UE, which P-CSCF has stored, to be used by the IMS AGW on the target satellite as the destination of media traffic towards the UE and to reserve an IP address in the IMS AGW on the target satellite to be used by the UE as the destination of media traffic. In addition, the P-CSCF requests the IMS AGW on the target satellite to configure context information other than IP addresses of the connection point towards the UE based on the corresponding context in the IMS AGW on the source satellite. This step 3 is performed according to clause 8.2 of TS 23.334 [74].

4.	P-CSCF requests IMS AGW on the target satellite to configure the IP address allocated in IMS AGW on satellite in the terminating network, which P-CSCF has stored, to be used by the IMS AGW on the target satellite as the destination of media traffic towards the terminating network and to reserve an IP address in the IMS AGW on the target satellite to be used by the IMS AGW on satellite in the terminating network as the destination of media traffic towards the originating network. In addition, the P-CSCF requests the IMS AGW on the target satellite to configure context information other than IP addresses of the connection point towards the terminating network based on the corresponding context in the IMS AGW on the source satellite. This step 4 is performed according to clause 8.2 of TS 23.334 [74].

5.	P-CSCF replies as defined in TS 23.502 [94] to the early notification received in step 1 by indicating that the change of the user plane paths for optimized media routing to the one through the target satellite should be performed. This reply also contains the IP address allocated in IMS AGW on the target satellite to be used by UE as the destination of media traffic. SMF in 5GC establishes ULCL and L-PSA on the target satellite with the ULCL filter towards the L-PSA containing this IP address according to clause 4.3.5.7 of TS 23.502 [94] and updates the UE filter for media traffic to contain this IP address according to clause 4.3.3.2 of TS 23.502 [94].

6.	P-CSCF receives the late notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic as defined in TS 23.502 [94]. This late notification indicates that 5GC has prepared the user plane path for optimized media routing through the target satellite while implying the user plane path for optimized media routing through the source satellite is still maintained as long as active traffic exists.

NOTE 3:	The following steps 7 to 18 are used only for the purpose of updating the setting in UE in the originating network and in IMS AGW in the terminating network with regards to the destination of media traffic towards IMS AGW in the originating network.

7.	P-CSCF sends SIP MESSAGE to IMS AS to request it to send SIP re-INVITE to the terminating network. This SIP MESSAGE contains the IP address allocated in IMS AGW on the target satellite to be used by the terminating network as the destination of media traffic. This SIP MESSAGE also contains the satellite ID of the target satellite.

8.	IMS AS sends SIP re-INVITE to the terminating network. This SIP re-INVITE contains the IP address allocated in IMS AGW on the target satellite to be used by the terminating network as the destination of media traffic. This SIP re-INVITE also contains a SIP header for conveying the satellite ID of the target satellite.

9.	P-CSCF requests IMS AGW on satellite to configure the IP address received in step 8 to be used by the IMS AGW on satellite as the destination of media traffic towards the originating network. This step 9 is performed so far according to clause 8.4 of TS 23.334 [74]. In addition, P-CSCF stores the satellite ID of the target satellite in the originating network for future use.

10.	P-CSCF sends SIP re-INVITE to UE.

11. UE sends SIP 200 OK to P-CSCF.

12.	P-CSCF sends SIP 200 OK to the originating network. This SIP 200 OK also contains a SIP header for conveying the satellite ID of the satellite in the terminating network.

13. IMS AS sends SIP re-INVITE to P-CSCF. This SIP re-INVITE also contains a SIP header for conveying the satellite ID of the satellite in the terminating network.

14.	P-CSCF sends SIP re-INVITE to UE. This SIP re-INVITE contains the IP address allocated in IMS AGW on the target satellite to be used by UE as the destination of media traffic.

15.	UE sends SIP 200 OK to P-CSCF.

16.	P-CSCF sends SIP 200 OK to IMS AS.

17.	SMF in 5GC releases ULCL and L-PSA on the source satellite according to NOTEs 7 and 8 and steps 11 and 12 in clause 4.3.5.7 of TS 23.502 [94].

18. P-CSCF releases IMS AGW on the source satellite sometime after receiving SIP 200 OK in step 15. This step 18 is performed according to clause 8.5 of TS 23.334 [74].

## ---End of the Change---