SA WG2 Meeting #166	S2-2412030

18 - 22 November, 2024, Orlando , US

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1297 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editor's Note removal for VFL procedures. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions for KI#2: 5GC Support for Vertical Federated Learning in clause 8.2 of TR23.700-84, this CR aims to specify the high-level functionality for vertical federated learning between AF and NWDAF(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The consumer of analytics does not need to aware of whether analytics is provided using VFL or other mechanism. Thus, there is no need to register and discover VFL server. Editor’s Note：Whether NWDAF as VFL server registers to NRF with VFL capability information is needed is FFS. Editor’s Note：Whether AF as VFL server registers to NRF with VFL capability information is needed is FFS. Editor’s Note：Whether a discovery of an AF acting as VFL server is required and how it can be done is FFS. Editor’s Note: The content of the VFL status report is FFS. It is slightly unclear the motivation to introduce additional NEF's functionality than that agreed in October. Thus, NEF's role is limited to that agreed in October. VFL server can send separate message to each VFL client, even when VFL server is untrusted AF. Editor´s note: It is ffs whether the VFL client can only reduce the requirements (e.g. samples) or also add new requirements. It is also ffs whether step 1 needs to be repeated if requirements have been reduced by the VFL client. Editor's note: It is ffs whether the NEF stores selection criteria received during discovery and includes them in subsequent messages or if the AF needs to provide related parameters. Editor's note: The NEF procedures need to be confirmed. Editor´s note: It is ffs whether .the VFL server sends a separate message for each VFL client towards the NEF. Usage of temporary NWDAF IDs needs to be added. It is postponed to next meeting. Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS. Sample information is needed for each training iteration because the samples maybe changed frequently. Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training. Editor’s note: The report may include ID(s) of sample(s) corresponding to the intermediate training result. Terminology is aligned. Editor’s Note: Terminology will be aligned in the future. Editor's note: Terminology should be alligned related among VFL related descriptions, in particular related to intermediate results and VFL model correlation ID. It is out of 3GPP scope. Editor’s Note: Whether weight of feature is computed by VFL server is FFS. The NWDAF can collect input data. Editor's note: It is ffs whether the NWDAF as VFL client also uses the collected input data to update its model. All the trusted AF part is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL functionality is not correctly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

## 5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

-	If the analytics are related to UE(s) and if NWDAF instances indicate weights for TAIs in their NF profile (see clause 6.3.13 of TS 23.501 [2]), the NWDAF service consumer may use the weights for TAIs to decide which NWDAF to select.

-	If the NWDAF service consumer needs to discover an NWDAF containing an AnLF with analytics accuracy checking capability, the consumer may query NRF providing also the analytics accuracy checking capability in the discovery request.

-	If the NWDAF service consumer needs to discover an NWDAF containing AnLF which can use the Model provided by the specific NWDAF containing MTLF (e.g., in case of analytics context transfer), the consumer should discover the NWDAF(s) whose vendor ID is in the ML Model Interoperability indicator of the NWDAF containing MTLF.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types or to collect data from particular NWDAF Serving Area, the consumer may query NRF providing the NF Set IDs or NF types or Area of Interest in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources or NWDAF Serving Area information as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	When one or more trained ML Models are available for one or more Analytics ID(s) the NWDAF containing MTLF shall include the Analytics ID(s) that is(are) supported per service in the registration towards NRF. The NWDAF containing MTLF may wait to register in NRF the above services until at least one trained model is available. The NWDAF containing MTLF may provide to the NRF a list of Analytics IDs corresponding to the trained ML Models and possibly the ML Model Filter Information for the trained ML Model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML Model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. If the NWDAF containing MTLF supports ML Model interoperability, the NWDAF containing MTLF includes, in the registration to the NRF, an ML Model Interoperability indicator for each Analytics ID.

Editor's note:	How to indicate supporting model training for the LMF-based AI/ML positioning as defined in TS 23.273 [39] by the MTLF to NRF, e.g., using a specific analytics ID or a new indication is FFS.

Editor's note:	For model training for LMF-based AI/ML positioning, whether Positioning case information (e.g. Uplink/downlink, or case 2b, 3b) can be included in NF profile as an optional parameter is FFS.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML Models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML Models requested by the NWDAFs from the vendors in the list.

NOTE 4:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (e.g. an NWDAF containing AnLF, an NWDAF containing MTLF as FL server or FL client) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required and Vendor ID. The NRF returns one or more candidate instances of NWDAF containing MTLF to the NF consumer and each candidate instance of NWDAF containing MTLF includes the Analytics ID(s), possibly the ML Model Filter Information for the available trained ML Models and ML Model Interoperability indicator, if available.

-	If the NWDAF service consumer needs to discover an NWDAF containing an MTLF with ML Model accuracy checking capability, the consumer may query NRF also providing the ML Model accuracy checking capability in the discovery request.

In order to discover an NWDAF containing MTLF with Horizontal Federated Learning (HFL) capability via NRF, in addition to the procedures described above for discovering NWDAF containing MTLF:

-	An NWDAF containing MTLF supporting HFL as a server shall additionally include FL capability type (i.e. HFL server), and may include Time interval supporting HFL as HFL capability information during the registration in NRF.

-	An NWDAF containing MTLF supporting HFL as a client shall additionally include FL capability type (i.e. HFL client), and may include Time interval supporting HFL as HFL capability information during the registration in NRF, and it may also include, NF type(s) and NWDAF Serving Area information and/or NF set ID(s) of the data source(s) where data can be collected as input for local model training.

NOTE 5:	An NWDAF containing MTLF may indicate to support both HFL server and HFL client in the HFL capability for specific Analytics ID. The FL capability type only applies for HFL but continues to be termed FL capability type to maintain backward compatibility with previous releases where only HFL was supported but the term FL capability type was used

-	During the discovery of NWDAF containing MTLF as HFL server, a consumer (e.g. a NWDAF containing MTLF) may include in the request the FL capability type as HFL server and may include Time Period of Interest and ML Model Filter information for the trained ML Model(s) per Analytics ID(s), if available. The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

-	During the discovery of NWDAF containing MTLF as HFL client, a consumer (e.g. an HFL server) may include in the request FL capability type as FL client and may include Time Period of Interest, a list of NF type(s) and/or NF set ID(s) of the data source(s). The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

NOTE 6:	The service consumer to discover an NWDAF containing MTLF with HFL capability is limited to NWDAF containing MTLF in this Release.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the NWDAF with roaming exchange capability (RE-NWDAF) to request analytics or input data is discovered via the NRF. A consumer in the same PLMN as the RE-NWDAF discovers the RE-NWDAF(s) by querying for NWDAF(s) where the roaming exchange capability is indicated in its (their) NF profile. A consumer in a peer PLMN (i.e. RE-NWDAF) discovers the RE-NWDAF(s) by querying for NWDAF(s) in the target PLMN that is (are) supporting the specific services defined for roaming. A RE-NWDAF discovers the RE-NWDAF(s) in a different PLMN (i.e. HPLMN or VPLMN) using the procedure defined in clause 4.17.5 (if delegated discovery is not used) or clause 4.17.10 (if delegated discovery is used) of TS 23.502 [3], where the detailed parameters are determined based on the analytics request or subscription from the consumer 5GC NF, operator policy, user consent and/or local configuration.

In order to support VFL training and inference, the NWDAF

shall include its VFL capability information per supported Analytics ID during registering to NRF. The VFL capability information includes VFL capability type (i.e. VFL client) and Time interval supporting VFL if available. To discover NWDAF supporting VFL from NRF

, the

consumer should consider the VFL capability information, the detailed procedure and parameters of NWDAF registration and discovery for VFL are as defined in clause 6.2H.2.1.

Editor's note:	Whether the VFL Interoperability information (e.g. type of supported VFL training method(s), vendor information, and feature related information) needs to be part of the NF profile registered in NRF is FFS.

Editor’s note: It is FFS whether and how to enhance the discovery of VFL client that can do VFL with the VFL server.

Editor's note:	Details of ML Model handling, supported features, Feature alignment are FFS.

* * *Next Change * * *

## 5.X	AF Discovery and Selection for VFL

The AF discovery and selection is defined in the clause 6.3.25, TS 23.501 [2]. In addition to support VFL training and inference, the following factors may be considered for AF discovery and selection:

-	VFL capability information per supported AnalyticsID, which includes:

-	VFL capability type (i.e. VFL client)

-	optional Time interval supporting VFL.

A trusted AF registers the above factor in NRF, for an untrusted AF the NEF is configured via OAM to register this factor into NRF. For the discovery of a trusted AF or untrusted AF, the consumer may select an AF instance considering the above factor. The detailed procedure and parameters of AF registration and discovery for VFL are as defined in clause 6.2H.2.1.

* * *Next Change * * *

## 6.2H	Vertical Federated Learning among NWDAFs and AFs

### 6.2H.1	General

This clause specifies procedures for Vertical Federated learning where AFs and NWDAF can either act as VFL servers or VFL clients. Procedures for registration and discovery, for VFL training preparation, for VFL training, and for VFL inference are covered.

### 6.2H.2	Procedures

#### 6.2H.2.1	Registration and Discovery procedure for Vertical Federated Learning

##### 6.2H.2.1.1	Registration and Discovery procedure for Vertical Federated Learning when NWDAF is acting as the VFL server

Figure 6.2H.2.1.1-1: Registration and Discovery procedure for Vertical Federated Learning when NWDAF is acting as VFL server and NWDAF(s) and/or AF(s) are the VFL clients

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is NWDAF.

1b. NWDAF

as VFL client registers to NRF with its NF profile, which includes

NF Type (i.e. NWDAF type), analytics ID(s), service area if available and VFL capability information per analytics ID. The VFL capability information includes VFL capability type (i.e. VFL client) and optional Time interval supporting VFL.

1c. When untrusted AF as VFL client, it shall register to the NEF via OAM configuration: Analyics ID(s) and its VFL capability information per supported analytics ID. The AF’s VFL capability information includes VFL capability type (i.e. VFL client) and optional Time interval supporting VFL. Then NEF updates NEF profile to NRF including associated AF ID and AF’s VFL capability information.

2. The NRF receives the registrations from VFL server and VFL client(s), and stores their NF profile.

3. The NRF sends registration response to VFL server and VFL client(s).

Steps 4 to 6 are the NWDAF and AF Discovery procedures when the VFL server is NWDAF.

4-6.	NWDAF as the VFL server determines that the ML Model requires VFL based on operator policy, Analytics ID, and Service Area.

NOTE:	Step 4 in Figure 6.2H.2.1-1 may be triggered at the VFL Server NWDAF by VFL server NWDAF itself or a request from a consumer.

T

he VFL Server NWDAF discovers other NWDAF(s) and/or AF(s) as VFL Client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: NF type (i.e. NWDAF type, AF type, or NEF type), Analytics ID, VFL capability type (i.e.VFL client), Time Period of Interest and and optional Service Area.

##### 6.2H.2.1.2	Registration and Discovery procedure for Vertical Federated Learning when AF is acting as the VFL server

The procedure below shows registration and discovery for VFL training and inference when the AF is the AF server, the AF can be trusted or untrused one. There can be multiple NWDAFs as VFL clients.

Editor´s note: The following is FFS: a deployment scenario that can also be agreed between operator and application service provider is when untrusted AF is the VFL server and only one NWDAF will be the VFL client. In such case the VFL Server can be configured to go directly to preparation procedure in clause 6.2H.2.2.

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is AF.

- Same as the step 1b, in clause 6.2H.2.1, NWDAF as VFL client registers to NRF with its NF profile. The NWDAF registers with the analytics IDs it supports.

NOTE x: The AF can use unstandardized values of the analytics ID.  The unstandardized values can be used by the AF as the VFL server to intiate the VFL training VFL inference and need to be supported by NWDAFs acting as VFL clients. It is the operator´s responsibility to guarantee that unstndardized analytics ID values within a PLMN are unique.

Steps 2-9 are the NWDAF Discovery procedures when the VFL server is an untrusted AF.

4. If untrusted AF acting as the VFL server, it determines that VFL operations are required and the NWDAF(s) as VFL client(s) are required. The AF sends a discovery request for the VFL client(s) to the NEF and provides selection criteria: analytics ID, required NF type (i.e. NWDAF type), VFL capability type (i.e. VFL client), Time Period of Interest and optional Service Area.

Editor's note:	Whether additional selection criteria are required is FFS (e.g. vendor information and feature related information).

5. The NEF checks based on configured policies whether the AF is entitled to request a VFL client for the analytics ID.

6-7. The NEF discovers VFL client (i.e. NWDAF) on behalf of the AF from the NRF by invoking the Nnrf_NFDiscovery_Request using the selection criteria provided by the AF as defined in step 4.

Editor’s note: It is FFS, whether NEF should perform VFL NWDAF selection (or shortlisting) or it should only discover NWDAF client candidates without any further selection.

8. 	The NEF selects an NWDAFs capable as acting as VFL clients and matching the received selection criteria. The NEF anonymizes NWDAF instances ID(s) and assigns temporary NWDAF ID(s) for each selected NWDAF intance as VFL client. The NEF stores the temporary NWDAF ID(s) together with information how to reach the NWDAFs

9. The NEF sends the temporary NWDAF ID(s) to the untrusted AF.

10. The AF stores the received temporary VFL client ID and uses it subsequent interactions with the NEF to indicate the target VFL client.

NOTE: If the VFL client NWDAFs are preconfigured to the NEF and untrusted AF, the Discovery procedure can be skipped.

Editor's note:	Which services are used in step 4 and step 9 are FFS.

Editor's note: How to subsequently handles the Temporary NWDAF ID (e.g. removing and possibly notifying the update of Temporary NWDAF IDs) is FFS.

* * *Next Change * * *

#### 6.2H.2.2	Preparation procedure for Vertical Federated Learning

##### 6.2H.2.2.0	General

Preparation procedure is used to check if the VFL Client(s) can meet the ML Model training requirement. The procedure includes the negotiation, between server and client(s) to enable interoperability, sample alignment and optionally feature alignment. This is performed after VFL client discovery is finished as clause 6.2H.2.1

NOTE 1: 	Features can be non-specified, privacy protected and in this case feature alignment is a simple alignment using info registered in NRF, e.g. data sources info or proprietary Feature ID).

Editor´s note: Possible refinement of the selection of features during the preparation phase is FFS.

NOTE 2:	Vertical Federated Learning preparation procedure can be skipped if the VFL Server can decide which VFL Client(s) support the VFL procedure to be performed, e.g. based on local configuration.

##### 6.2H.2.2.1	Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Editor's note:	Whether the VFL Interoperability information (e.g. type of supported VFL training method(s)) needs to be part of the NF profile registered in NRF is FFS.

Editor´s Note: Whether the sample IDs are included as part of the MLModelTraining or a separate Nnwdaf service is defined is FFS.

Editor´s Note: The preparation procedure when an untrustrd AF is part of the procedure is FFS.

Editor´s note:	For UEs as samples, additional discussion is needed on whether UE needs to be registerd or not.

1.	The VFL Server sends a Federated Learning preparation request to each of the VFL Client(s), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag. The Server may add a list of sample IDs, Analytics ID and optionally Feature ID.

2.	Each VFL Client checks if it can meet the ML Model training requirement. If it cannot meet the requirements for any reason, it may decide which requirements it can agree to.

Editor´s note:	Clarifications which parameters are regarded as requirements and can be modified by the VFL client is required

- Each VFL Client invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate to the VFL Server whether it accepts the requirements, whether it requests new requirements (e.g., list of available sample IDs) or whether it will not join. The client includes the requirements it can accept or may add a reason if it cannot join the FL process.

4.	The VFL Server NWDAF determines the VFL Client(s) and sample IDs to be involved in the FL procedures based on the information received in step 6 in Figure 6.2H.2.1-1 (if performed) and other information received in step 3 (if available).

##### 6.2H.2.2.2	Preparation procedure for Vertical Federated Learning when the untrusted AF as the VFL server

This clause specifies the preparation (including sample alignment) procedure for untrusted AF-initiated VFL scenarios between AF and NWDAF(s) within a single PLMN.

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL Server

Editor´s Note: Whether the sample IDs are included as part of the MLModelTraining or a separate Nnwdaf service is defined is FFS.

  - The untrusted AF as VFL server sends VFL preparation request to the candidate VFL client(s), via NEF with the ML Preparation Flag to check if the VFL client(s) can meet the ML Model training requirement (e.g. Analytics ID, Sample alignment requirement, VFL Availability time requirement (time span needed for the VFL process), etc.). Sample alignment requirement includes a list of sample IDs targeted for VFL training as defined by the AF, and Temporal NWDAF ID that assigned in Step in Clause 6.2H.1.2. The AF sends a separate message for each VFL client towards the NEF.

  - The NEF maps the external UE IDs (e.g. GPSIs) to the internal UE IDs (e.g. SUPIs). The NEF may send VFL preparation request to each candidate VFL client using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service when multiple candidate VFL clients are involved with the same information as provided in step 1 in 6.2H.2.2.1.

Editor’s Note:	Whether to extend the existing ML Preparation Flag for reuse or introduce a new flag dedicated for VFL preparation is FFS.

Editor's note:	Whether to use ML Model interoperability information or introduce new identifiers for interoperability among NWDAF and AFs is FFS.

Editor’s Note:	The specific service name for the untrusted AF case is FFS.

  - Same as step 1 in 6.2H.2.2-1.

  - Same as step 2 in 6.2H.2.2-1.

  - Same as step 3 in 6.2H.2.2-1.

  - [optional] When multiple candidate VFL clients are involved, the NEF may aggregate the response from each candidate VFL client and send a VFL Preparation response message to the untrusted AF.

* * *Next Change * * *

#### 6.2H.2.3 Training Procedure for Vertical Federated Learning

##### 6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server.

Editor's Note:  How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor’s Note:	It is FFS if the term “backward local ML model training information” needs to be changed into “intermediate results”.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

NOTE: VFL Server determines to perform VFL training based on the internal trigger or local configuration.

The steps 2-6 are repeated until the training termination condition is reached.

2.	To start VFL training, the VFL server allocates VFL correlation ID. The VFL server sends a request to start the VFL training to each of the selected VFL clients. The VFL server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF clients(s) and Naf_VFLTraining_Subscribe or Naf_VFLTrainingInfo_Request to the selected AF clients(s). The request includes VFL correlation ID, VFL capability type (VFL Client).

If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing the backward local ML model training information to each of the VFL clients for next round of VFL training, round ID, and ID(s) of sample(s) corresponding to the intermediate training result.

Editor´s Note: Additional Parameters to be provided in the request are FFS.

Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

Editor’s Note:	It is FFS which service is used between VFL server and VFL clients for VFL training.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and based on possible backward local ML model training information distributed by the VFL server in the previous training iteration, and computes and reports the client intermediate training result of the local ML model to the VFL server. The report may include ID(s) of sample(s) corresponding to the intermediate training result.

Editor´s note: The following is FFS and may depend on the service design: When the clients reports the client intermediate training result, it also includes the corresponding VFL correlation ID.

5.	The VFL server may collect the local data and generate its own local intermediate training result. The NWDAF acting as VFL Server computes the backward local ML model training information (e.g. gradient information or loss information) based on the client intermediate training result received in step 4, its own local intermediate results and the label. The backward local ML model training information is used for updating the models of VFL clients. Different backward local ML model training information may be computed for different VFL clients, respectively.

The VFL server may also compute the ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

Editor’s Note: Whether VFL server and VFL clients share feature information is FFS.

6.	[Optional] The NWDAF acting as VFL server evaluates (e.g., based on the convergence of a loss function or loss value, the pre-set iteration number is reached) whether VFL Training process converged. If not, the NWDAF acting as a VFL Server determines another round of VFL training is required and repeats step 2 - 6. If yes, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

Editor’s Note: Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS

Editor’s Note: the following is FFS. The VFL server determines with which VFL Client(s) to continue the VFL. The VFL server may provide to the VFL clients any sample ID(s) if changed

7.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process. The VFL Server and each VFL Client, stores the VFL correlation ID and latest information related to their locally trained Models

NOTE: The VFL correlation ID is used later for inference.

Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS.

Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS.

NOTE 1:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

* * *Next Change * * *

##### 6.2H.2.3.2	Vertical Federated Learning (VFL) training procedures with untrusted AF as VFL server

Editor's note:	It is ffs whether to shift the trusted AF as server case to the NWDAF as server case. However, it would need to be considered that the trusted AF only interacts with NWDAFs as clients.

Editor's note:	Where the same messages appear in NWDAF as server and AF as server callflows, the description should be replaced by cross-references.

Editor´s note:	The callflow should be compared with the NWDAF as server callflow to have the same steps where applicable

Figure 6.2H.2.3.2-1: VFL model training with AF as VFL server

- Same as Steps 1 in clause 6.2H.2.3.1.

- 2

.	An untrusted AF identifies the VFL client using the temporary NWDAF ID assigned in the discovery procedure (see Clause 6.2H.2.1.1) and sends the request to the NEF. The NEF maps external identifiers to internal identifiers, then forward to NWDAF clients based on the temporal NWDAF IDs. The AF can send separate message for each NWDAF.

Editor's note:	Additional parameters, e.g. to indicate features and samples, are FFS. More clarifications about the intermediate results might be required; if loss information and gradient information is contained in intermediate results is FFS.

3-5.	Same as Steps 2-5 in clause 6.2H.2.3.1.

6.	For each NWDAF/MTLF as VFL training client, the NEF forwards the intermediate results to the AF, and maps external identifiers to internal identifiers.

7.	Same as Steps 6-7 in clause 6.2H.2.3.1.

8. If AF determines to terminate the VFL process, the AF sends VFL termination message to NEF, including Temporal NWDAF ID. The NEF maps external identifiers to internal identifiers, then forward to NWDAF clients based on the temporal NWDAF IDs.

9.  Same as Steps 7 in clause 6.2H.2.3.1.

Editor's note:	Which services are used between VFL server, NEF and VFL client is FFS and message names need to be confirmed.

* * *Next Change * * *

#### 6.2H.2.4	Inference procedure for vertical federated learning

##### 6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Editor's note:	If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS.

1.	VFL server decides to initiate the VFL inference procedure with the VFL clients based on internal logic. VFL Server selects clients(s) using information stored in the VFL training process. The server may select some or no clients, e.g., depending on their contribution to the training result and the current status of the client.

NOTE: The trigger of VFL inference may be Analytics request from consumer.

If the VFL server is NWDAF and the VFL client(s) are NWDAF(s), VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients including the list of sample IDs (e.g., UE IDs), VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

Editor's note:	It is FFS additional parameters are needed to send from the VFL server to VFL client.

Editor's note:	It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying.

When the VFL server is NWDAF and the VFL client(s) are untrusted AF(s), VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes the UE ID(s), the VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used. If the AF is untrusted, the VFL server NWDAF sends the request to the NEF, and NEF forwards the request to the untrusted AF.

Editor's note:	Which service is used between the VFL server and VFL client is FFS.

2.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

3.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

4.	VFL Client sends the client intermediate local inference results to the VFL server including the VFL correlation ID, and corresponding sample IDs.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 3, step 4 may be repeated.

5.	The VFL server may collects its local data and generate the intermediate local inference results. The VFL Server combines all the intermediate local inference results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate results when generates the combined inference output.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

* * *Next Change * * *

##### 6.2H.2.4.2	Inference procedure for vertical federated learning with AF as VFL server

Editor's note:	It is ffs whether to shift the trusted AF as server case to the NWDAF as server case. However, it would need to be considered that the trusted AF only interacts with NWDAFs as clients. It also needs to be considered that the interfence trigger may be different

Editor's note:	Where the same messages appear in NWDAF as server and AF as server callflows, the description should be replaced by cross-references.

Editor´s note:	The callflow should be compared with the NWDAF as server callflow to have the same steps where applicable

Figure 6.2H.2.4.2-1: inference procedure for vertical federated learning with untrusted AF as VFL server.

- The inference procedure may be triggered by internal service logic of the AF acting as VFL server, or by a request or subscription for information from a 5GC entity, for instance for service experience information. The NEF forwards the request or subscription for information to the AF.

- AF acting as VFL server decides to initiate the VFL inference procedure with the VFL clients and for each client sends an inference request or subscription to the NEF including the Target of Analytics Reporting (e.g. UE ID(s)), possible analytics filters (see clause 6.1.3), and VFL correlation ID, and temporary NWDAF ID obtained in the discovery procedure (see Clause 6.2H.2.1.1). The NEF translates the external IDs to internal IDs, and forward the request for NWDAFs based on Temporal NWDAF ID.

2-4.	Same as Steps 1-4 in clause 6.2H.2.4.1.

5. The NEF translates the internal ID and external ID and forwards the intermediate results to AF.

Editor's note:	Additional parameters, e.g. to indicate features, are FFS.

6. Same as Step 5 in clause 6.2H.2.4.1.

Editor's note:	Whether the VFL client needs to retrieve stored local models is FFS.

# >>>>END OF CHANGES<<<<