3GPP TSG-WG2 Meeting #166	S2-2412037

18 – 22 November 2024, Orlando, USA

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1246 | rev | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 – Update of VFL training and inference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Ericsson, Apple, ETRI, InterDigital], Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solving multiple Editor’s Notes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes in 5.4: 1) Added that Vertical Federated learning is a machine learning techinique working without exchanging/sharing of local data set nor ML models 2) Editor's note: Support for AF acting as VFL server selecting multiple NWDAFs is FFS. Is solved by introducing: - An AF acting as VFL server discovers and selects VFL client(s) (NWDAF(s)) to participate in a VFL procedure. 3) Editor's note: Details regarding Sample alignment and features alignment functionality or whether the functionality needs to be specified are FFS. Is solved in procedure procedure for preparation phase Editor's note: For an NWDAF impacts of the split into AnLF and MTLF are FFS. are solved in procedure clauses, where NWDAFs being part of inference have earlier been part of training. 4) Adding where the procedures are described Changes in 6.2F.2: Clarifying that some parameters are needed for VFL. Changes in 6.2H Added Preparation procedure Alignments including Sample and feature alignment is done in this phase |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL not implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.4, 6.2C.1; 6.2F.2, 6.2H.1, 6.2H.2, 6.2H.2.1, 6.2H.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 6.2G.2 and underlying sub clauses (changed text in this clause and underlying clauses, new sub clauses and moved sub clauses of 6.2H into this clause). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 5 has the following changes: - Clarify that Feature alignment is not optional under the assumption that no feature alignment is carried out during discovery (from NRF) - Clarify that in the Preparation Request that the VFL server includes a Feature Information Container which may include Feature ID and additional information - Clarify that in the Preparation Response that the VFL client includes a list of supported features - For the case where AF is the VFL server clarify that the NEF may create a common set of samples from the list of samples provided by each VFL client. |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AI/ML	Artificial Intelligence/Machine Learning

DCAF	Data Collection Application Function

FL	Federated Learning

RE-NWDAF	Roaming Exchange Network Data Analytics Function

VFL	Vertical Federated Learning

HFL	Horizontal Federated Learning

* * * Next Change * * *

## 5.3	Federated Learning (FL) among multiple NWDAFs

This clause describes Horizontal Federated Learning.

Federated learning among multiple NWDAFs is a machine learning technique in core network that trains an ML Model across multiple decentralized entities holding local data set, without exchanging/sharing local data set. This approach stands in contrast to centralized machine learning techniques where all the local datasets are uploaded to one server, thus allowing to address critical issues such as data privacy, data security, data access rights.

NOTE 1:	Horizontal Federated Learning is supported among multiple NWDAFs, which means the local data set in different FL client NWDAFs have the same feature space for different samples (e.g. UE IDs).

For Federated Learning supported by multiple NWDAFs containing MTLF, there is one NWDAF containing MTLF acting as FL server (called FL server NWDAF for short) and multiple NWDAFs containing MTLF acting as FL client (called FL client NWDAF for short), the main functionality includes:

FL server NWDAF:

-	discovers and selects FL client NWDAFs to participant in an FL procedure

-	requests FL client NWDAFs to do local model training and to report local model information.

-	generates global ML Model by aggregating local model information from FL client NWDAFs.

-	sends the global ML Model back to FL client NWDAFs to perform an additional training iteration if needed.

FL client NWDAF:

-	locally trains ML Model as tasked by the FL server NWDAF with the available local data set, which includes the data that may not be allowed to be shared with other FL client NWDAFs due to e.g. data privacy, data security, data access rights.

-	reports the trained local ML Model information to the FL server NWDAF.

-	receives the global ML Model from FL server NWDAF and perform an additional training iteration if needed.

FL server NWDAF or FL client NWDAF register to NRF with their FL capability information as described in clause 5.2.

The NWDAF containing MTLF determines to train an ML Model either based on local configuration or when it receives a request from NWDAF containing AnLF. The NWDAF containing MTLF further determines whether the ML Model should be trained via FL mechanism based on Analytic ID, Service Area/DNAI or when data can not be obtained directly from data producer NF (e.g. due to data privacy, data security). The NWDAF containing AnLF is not aware whether the ML Model is trained based on FL or not.

If the NWDAF containing MTLF can act as an FL server for the ML Model training, then FL procedure is initiated by the NWDAF containing MTLF as FL server NWDAF directly.

If the NWDAF containing MTLF determines to train an ML Model based on local configuration and the FL mechanism is required, but the NWDAF containing MTLF can't act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The FL server NWDAF may determine to initiate FL procedure before providing the ML Model.

If the ML Model training is triggered by the request from NWDAF containing AnLF, the NWDAF containing MTLF determines the FL mechanism is required but it can not act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The Notification Target Address and the Notification Correlation ID from the NWDAF containing AnLF is provided in the request message sent to the FL server NWDAF. The FL server NWDAF may determine to initiate FL procedure before providing the ML Model. The FL server NWDAF sends the ML Model information to the notification endpoint (e.g. the NWDAF containing AnLF) after the ML Model training success.

NOTE 2:	The security procedure on authorizating FL server to initiate FL procedure on the FL client(s) is described in Annex X, clause X.9 of TS 33.501 [49]. The security procedure authorizing an MTLF to request ML Models on behalf of an AnLF to another MTLF (e.g., FL server NWDAF) is described in Annex X, clause X.10 of TS 33.501 [49].

Before FL procedure is initiated by FL server NWDAF, appropriate FL client NWDAFs should be discovered by FL server NWDAF as described in clause 5.2.

When starting an FL procedure, the FL server NWDAF is to provide an initial model to each FL client NWDAF, and then each FL client NWDAF is to perform local model training using its local data set. The detailed procedure for FL among Multiple NWDAFs is described in clause 6.2C.

* * * Next Change * * *

## 6.2C	Federated Learning among Multiple NWDAFs

### 6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Horizontal Federated Learning technique to train an ML Model.

* * * Next Change * * *

### 6.2F.2	Contents of ML Model Training

The consumers of the ML Model training services (i.e. an NWDAF containing MTLF) may provide the input parameters in Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service operations as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be trained.

-	For HFL,ML Model Interoperability Information as defined in clause 6.2A.2.

-	(Only for Nnwdaf_MLModelTraining_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the NWDAF containing MTLF with the subscription.

-	[OPTIONAL] For HFL, ML Model Information (as defined in clause 6.2A.2).

-	[OPTIONAL] For HFL, ML Model file.

NOTE 1:	It is up to NWDAF implementation to determine whether to include ML Model file in input parameters considering ML Model file size, etc.

-	[OPTIONAL] For HFL, ML Model identifier: identifies the provided ML Model.

-	[OPTIONAL] ML Preparation Flag: identifies whether the request is for preparing Federated Learning or executing Federated Learning.

-	[OPTIONAL] ML Model Accuracy Check Flag: identifies that the request is for using the local training data as the testing dataset to calculate the Model Accuracy of the global ML Model provided by the NWDAF service consumer acting as the FL Server NWDAF.

-	[OPTIONAL] For HFL, ML Correlation ID: identifies the Federated Learning procedure for training the ML Model. This parameter is included when the service is used for Federated Learning.

-	[OPTIONAL] For HFL, Data Availability requirement. This is the requirement on data availability for the ML Model training. e.g. FL Server NWDAF sends the requirement in preparation request to a FL Client NWDAF for selecting the FL Client NWDAF which can meet the data availability requirement. The following may be included:

-	Event ID list to be collected for local model training.

-	Dataset statistical properties as defined in clause 6.1.3.

-	Time window of the data samples.

-	Minimum number of data samples.

-	[OPTIONAL] FL Availability time requirement. This is the requirement on availability time for the ML Model training, e.g. FL Server NWDAF sends the requirement in preparation request to FL Client NWDAF for selecting the FL Client NWDAF which is available in the required time for training ML Model.

-	[OPTIONAL] Training Filter Information: enables to select which data for the ML Model training is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the Training Filter Information are the same as or subset of parameter types in the ML Model Filter Information which are defined in clause 6.2A.2.

-	[OPTIONAL] Target of Training Reporting: indicates the object(s) for which data for ML Model training is requested, i.e. group of UEs identified by a list of Internal-Group-Ids or any UE (i.e. all UEs).

-	[OPTIONAL] Use case context: indicates the context of use of ML Model.

-	[OPTIONAL] Training Reporting Information with the following parameters:

-	Maximum response time: indicates maximum time for waiting notifications (i.e. model training results).

-	[OPTIONAL] Iteration round ID: indicates the iteration round number of current ML Model training.

-	[OPTIONAL] Expiry time.

-	[OPTIONAL] Indication of skipping the current FL round.

-	For VFL, VFL model ID: identifies the Vertical Federated Learning procedure while training the ML Model and once the training ends, the ML Model for inference purposes.

Editor's note:	Whether new information for interoperatbility when using VFL, such as the VFL Interoperability information, convergence reports or whether inference needs new indicators or not is FFS.

The NWDAF containing MTLF provides to the consumer of the ML Model training service operations as described in clause 7.10 and clause 7.11, the output information in notification or response as listed below:

-	(Only for Nnwdaf_MLModelTraining_Notify) The Notification Correlation Information.

-	[OPTIONAL] ML Model Information (as defined in clause 6.2A.2).

-	[OPTIONAL] For HFL, ML Model identifier: identifies the provisioned ML Model.

-	[OPTIONAL] Global ML Model Accuracy information: The model metric value of the global ML Model and optionally the used metric, which is calculate by the FL Client NWDAF using the local training data as the testing dataset.

[OPTIONAL] Status report of FL training: Accuracy information of local model and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.), which are generated by the FL Client NWDAF during FL procedure.

NOTE 2:	The parameters in Training Input Data Information are up to the implementation.

-	[OPTIONAL] ML Correlation ID. This parameter may be included when the service is used for Federated Learning.

-	[OPTIONAL] Iteration round ID: indicates the iteration round number of ML Model training indicated by the FL Server NWDAF.

-	[OPTIONAL] Delay Event Notification with the following parameters:

-	delay event indication: this parameter indicates that FL Client NWDAF is not able to complete the training of the interim local ML Model within the maximum response time provided by the FL Server NWDAF.

-	[OPTIONAL] cause code (e.g. local ML Model training failure, more time necessary for local ML Model training, etc.).

-	[OPTIONAL] Expected time to complete the training: Indicates to the FL Server NWDAF that expected remaining training time and may be provided with Delay Event Notification.

-	For VFL, VFL correlation ID: identifies the Vertical Federated Learning procedure while training the ML Model and once the training ends, the ML Model for inference purposes.

Editor's note:	Whether new information for interoperatbility when using VFL, such as the VFL Interoperability information, convergence reports or whether inference needs new indicators or not is FFS.

* * * Next Change * * *

#### 6.2H.2.2	Preparation procedure for Vertical Federated Learning

#### 6.2H.2.2.0	General

Preparation procedure is used to check if the VFL Client(s) can meet the ML Model training requirement. The procedure includes the negotiation, between server and client(s) to enable interoperability, sample alignment and feature alignment.

NOTE 1: 	Features can be non-specified, privacy protected e.g. data sources info or proprietary Feature ID).

Editor´s note: Possible refinement of the selection of features during the preparation phase is FFS.

NOTE 2:	Vertical Federated Learning preparation procedure can be skipped if the VFL Server can decide which VFL Client(s) support the VFL procedure to be performed, e.g. based on local configuration.

#### 6.2H.2.2.1	Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Editor´s note:	For UEs as samples, additional discussion is needed on whether UE needs to be registerd or not.

1.	The VFL Server sends a Federated Learning preparation request to each of the VFL Client(s), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag. The Server may add Sample Alignment Requirements which includes a list of sample IDs, Analytics ID and Feature Information container which includes Feature ID(s) and additional information.

2.	Each VFL Client checks if it can meet the ML Model training requirement. If it cannot meet the requirements for any reason, it may decide which requirements it can agree to.

Editor´s note:	Clarifications which parameters are regarded as requirements and can be modified by the VFL client is required

3.	Each VFL Client invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate to the VFL Server whether it accepts the requirements, whether it requests new requirements or whether it will not join. The client includes the requirements it can accept or may add a reason if it cannot join the FL process. The response also includes supported Features within a Feature Information Container.

If the notify or response includes new requested requirements, procedure from step 1 is repeated.

Editor´s note: It is ffs whether the VFL client can only reduce the requirements (e.g. samples) or also add new requirements. It is also ffs whether step 1 needs to be repeated if requirements have been reduced by the VFL client.

4.	The VFL Server NWDAF determines the VFL Client(s) to be involved in the FL procedures based on the information received in step 6 in Figure 6.2H.2.1-1 (if performed) and other information received in step 3 (if available).

#### 6.2H.2.2.2	Preparation procedure for Vertical Federated Learning when the AF as the VFL server

This clause specifies the preparation (including sample alignment) procedure for AF-initiated VFL scenarios between AF and NWDAF(s) within a single PLMN.

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when AF is the VFL Server

Editor´s Note: Whether the sample IDs are included as part of the MLModelTraining or a separate Nnwdaf service is defined is FFS.

  - The AF as VFL server sends VFL preparation request to the candidate VFL client(s) with the ML Preparation Flag to check if the VFL client(s) can meet the ML Model training requirement (e.g. Analytics ID, Sample alignment requirement, VFL Availability time requirement (time span needed for the VFL process), etc.). Sample alignment requirement includes a list of sample IDs targeted for VFL training and Feature Information container including Feature ID(s) and additional information as defined by the AF.

Editor´s note:	It is ffs whether .the VFL server sends a separate message for each VFL client towards the NEF. Usage of temporary NWDAF IDs needs to be added.

  - The NEF maps the external UE IDs (e.g. GPSIs) to the internal UE IDs (e.g. SUPIs). The NEF may send VFL preparation request to each candidate VFL client using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service when multiple candidate VFL clients are involved with the same information as provided in step 1 in 6.2H.2.2.1.

Editor’s Note:	Whether to extend the existing ML Preparation Flag for reuse or introduce a new flag dedicated for VFL preparation is FFS.

Editor’s Note:	The specific service name for the untrusted AF case is FFS.

  - Same as step 1 in 6.2H.2.2-1.

  - Same as step 2 in 6.2H.2.2-1.

  - Same as step 3 in 6.2H.2.2-1.

  - When multiple candidate VFL clients are involved, the NEF may aggregate the response from each candidate VFL client and send a VFL Preparation response message to the untrusted AF. In such a scenario the NEF may respond to the VFL server with a common sample list supported by all VFL clients that have provided a response in step 5.

* * * End of Changes * * *