3GPP TSG-SA2 Meeting #166	S2-2412073

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024				   (revision of S2-2410447)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1497 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the IMS standalone sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | AC.10.2.2 "Adding Application DC to Standalone IMS DC Session with Bootstrap DC" has severe issues that make it unusable: - It is unclear why the terminating UE (and possibly the user) should be involved in the IMS session before the actual DC application is selected and the ADC session is initiated by the originating UE. - It is unclear whether the callee should be alerted for the incoming BDC-only session. If the callee is alerted, he/she is not aware of the purpose of the session until the caller initiates the actual ADC. If the callee is not alerted, the UE-B remains reserved for the IMS session without the knowledge and permission from the user. - Similar user experience as intended here can be achieved by using the procedure in AC.10.2.1 for downloading the DC application(s) to the UE-A, and once the caller has selected the application, using the procedure in AC.10.2.3 to initiate the ADC session with callee. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to remove the current description of AC.10.2.2 "Adding Application DC to Standalone IMS DC Session with Bootstrap DC" and replace it with Originating Standalone Bootstrap DC Setup as described in clause AC.10.2.1, following with the Combined Bootstrap DC and Application DC Setup with the terminating UE as described in clause AC.10.2.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect, infeasible procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.10.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

### AC.10.2.2	Adding Application DC to Standalone IMS DC Session with Bootstrap DC

After finishing the Originating Standalone Bootstrap DC Setup and downloading the DC application(s) as described in clause AC.10.2.1, the originating UE may initiate a combined Bootstrap DC and Application DC Setup with the terminating UE as described in clause AC.10.2.3.

*************************** End of Changes ****************************