3GPP SA WG2 Meeting #166	S2-2412216

# Orlando, US, 18-22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0508 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifiers for 5G ProSe Multi-hop UE-to-UE Relay for non-IP PDU type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution adds the description of Identifiers to be used during discovery procedure for Multi-hop UE-to-UE Relay discovery for non-IP type. Furthermore for Model B, for the Discovery response messages to be propagated back to the Source End UE, if the first hop Relay allocates different Source Layer-2 IDs for each soliciation messages, so in order for the next hop relay to send back the response to the correct Source Layer-2 ID of the first hop Relay, it will further again need to allocate different Source Layer-2 IDs for each of the soliciation messages received. Possibly leading to higher probability of collisions. One of the better alternates can be to simply including discoverer’s source-layer-2 ID, which can be used by the last hop relay when sending the response message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of the Identifiers need to be present in the 5G Prose Discovery messages and corresponding changes in the Discovery procedures. Minor changes in toher clauses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.6.x(new) ; 6.3.2.6.3.1; 6.3.2.6.3.2; 6.7.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (ALL new text)

#### 5.8.6.x	Identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of non-IP PDU type

The 5G ProSe UE-to-UE Relay Discovery Announcement message (Model A) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and contains the following parameters for multi-hop relaying in addition to as described in clause 5.3.2.4.2:

For each user info in the Direct Discovery set:

-	 Hop count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop.

-	 (Optional) Hop-Limit: value that indicates the limit of the number of hops of the message.

-	 Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message.

The 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the source 5G ProSe End UE, and includes the following parameters for multi-hop relaying in addition to as described in clause 5.3.2.4.2:

-	Hop count: indicates the number of hops that the message is already relayed. It is set to 0 initially by the Discoverer End UE and is increased by 1 per hop.

-	(Optional) Hop-Limit: value that indicates the limit of the number of hops of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than or equal to the (pre)configured maximum number of hops.

The 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC, and includes the following parameters for multi-hop relaying in addition to as described in clause 5.3.2.4.2:

When a 5G ProSe UE-to-UE Relay self-selects a Source Layer-2 ID for a received 5G ProSe UE-to-UE Relay Discovery Solicitation message, it selects a different Source Layer-2 ID values for each 5G ProSe UE-to-UE Relay Discovery Solicitation message, so that the 5G ProSe UE-to-UE Relay can correlate the 5G ProSe UE-to-UE Relay Discovery Response message with the 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay can determine the destination layer-2 ID of the next-hop 5G Prose UE (i.e. discoverer 5G ProSe End UE or the neighbour 5G ProSe UE-to-UE Relay that sent the 5G ProSe UE-to-UE Relay Discovery Solicitation) based on the destination Layer-2 ID of the received 5G ProSe UE-to-UE Relay Discovery Response message.

-	Hop count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop.

-	(Optional) Hop-Limit: value that indicates the limit of the number of hops of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than or equal to the (pre)configured maximum number of hops.

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes discoverer source layer-2 ID.

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC includes the following parameters for multi-hop relaying in addition to as described in clause 5.3.2.4.2:

-    Hop count: indicates the number of hops that the message is already relayed.

-    Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes discoverer source layer-2 ID.

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the target 5G ProSe End UE matching the RSC includes the following the following parameters for multi-hop relaying in addition to as described in clause 5.3.2.4.2:

-    Hop count: indicates the number of hops that the message is already relayed.

-    Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes discoverer source layer-2 ID.

CHANGE 2

6.3.2.6.3.1	Procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model A

Depicted in Figure 6.3.2.6.3.1-1 is the procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model A.

Figure 6.3.2.6.3.1-1: 5G ProSe multi-hop UE-to-UE Relay Discovery of non-IP PDU type with Model A

1.	The 5G ProSe UE-to-UE Relay has discovered End UEs in proximity and obtains the Direct discovery set from End UEs in proximity per RSC. (e.g. via a previous 5G ProSe UE-to-UE Relay Discovery or via secure PC5 connection between 5G ProSe U2U Relay and 5G ProSe End UE (refer to TS 33.503 [29])). 5G ProSe UE-to-UE Relay may also get the maximum number of hops from End UEs.

2.	The 5G ProSe UE-to-UE Relay sends a UE-to-UE Relay Discovery Announcement message. The UE-to-UE Relay Discovery Announcement message contains the User Info ID of itself, Direct Discovery set including list of protected user info of End UEs, a list of paths indicated by an (ordered) list of User Info ID of Relays (initially it only contains the User Info ID of UE-to-UE Relay1 in the figure), hop count (which is set to 1 initially) corresponding to each user info in the Direct Discovery set and optionally maximum number of hops corresponding to each user info in the Direct Discovery set. Each path corresponds to a user info in the Direct Discovery set.

The UE-to-UE Relay Discovery Announcement message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

3.	After receiving UE-to-UE Relay Discovery Announcement message(s) from another 5G ProSe UE-to-UE relay, a 5G ProSe UE-to-UE Relay checks the received Direct Discovery set. For each protected user info in the received Direct Discovery set, if the path does not include the User Info ID of the 5G ProSe UE-to-UE Relay and the hop count is less than the corresponding maximum number of hops, the relay will add the protected user info into its own Direct Discovery set (if it is not added before) and add its own User Info ID into the path of the corresponding protected user info and increase the hop count by 1. If the relay already has the protected user info in its own Direct Discovery set with the hop count less than or equal to the received one, then it is up to relay's implementation to determine if add the protected user info again.

NOTE:	Based on implementation, Relay UEs can determine if two protected user infos are identical, e.g., by comparing the protected user info bit by bit if it is encrypted.

A 5G ProSe UE-to-UE Relay may associate the User Info ID and Layer-2 ID of neighbour UE-to-UE Relay(s) according to the Announcement message. The association may be used in the subsequent Link Management procedures.

4.	A 5G ProSe End UE or a 5G ProSe UE-to-UE Relay monitors Announcement message from a 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs and 5G ProSe UE-to-UE Relays determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

CHANGE 3

6.3.2.6.3.2	Procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model B

Depicted in Figure 6.3.2.6.3.2-1 is the procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model B.

Figure 6.3.2.6.3.2-1: Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model B

1.	The Source End UE determines the maximum number of hops for discovery based on policy configuration (i.e. a mapping between maximum number of hops and RSC) or QoS parameters.

If the maximum number of hops is determined based on RSC, the Source End UE may not include the maximum number of hops in the Solicitation message. Otherwise, the Source End UE includes the maximum number of hops in the Solicitation message.

2a.	The Source End UE sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the information as described in clause 5.8.6.x.

3a.	If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s) of a 5G ProSe UE-to-UE Relay, the 5G ProSe UE-to-UE Relay may decide to send a 5G ProSe UE-to-UE Relay Discovery Solicitation message.

The 5G ProSe UE-to-UE Relay should drop the received Solicitation message when its own User Info ID is contained in the Path information of received Solicitation message, or if the hop count (i.e., the number of Relays included in the message) has reached the maximum number of hops of the received Solicitation message. The maximum number of hops may be obtained from the Solicitation message or may be decided based on RSC if it is not obtained from the message.

If the same Direct Discovery Set is received from different 5G ProSe-enabled UEs, the 5G ProSe UE-to-UE Relay may select a Solicitation message to be sent to the next hop based on various criteria (e.g., hop count, delay, channel quality of received messages, etc.).

NOTE 1:	If the Source End UE does not receive any response after a timeout, based on application requirement, it may increase the maximum number of hops and send the discovery message again.

NOTE 2:	Based on implementation, Relay UEs can decide to send the discovery message with the same direct discovery set at most once in order to control the total number of discovery messages transferred among Relays. To determine the identical direct discovery set, the Relay UE can compare the direct discovery set bit by bit if it is encrypted, or check the source End UE and target End UE User info if the direct discovery set is not encrypted.

4a.	A 5G ProSe UE-to-UE Relay sends a Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID of Relays in the path that has sent the Solicitation message. The hop count is increased by 1.

The 5G ProSe UE-to-UE Relay assigns unique Layer-2 ID for sending Solicitation message corresponding to each received Solicitation message that it decides to forward, as described in clause 5.8.6.x.

2b.-6b.	Another transmission path of the Solicitation message.

7-8. If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s) of the Target 5G ProSe End UE, and the Target 5G ProSe End UE matches the user info (i.e. Application Layer ID) of the discoveree 5G ProSe End UE contained in the solicitation message, then the Target 5G ProSe End UE responds to the 5G ProSe UE-to-UE Relay with a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the information as described in clause 5.8.6.x.

The Target 5G ProSe End UE may choose a path and sends Response message to 5G ProSe UE-to-UE Relay(s) based on e.g. the PC5 signal strength of each solicitation message received, the number of hops to the Source End UE, the path information, order in which solicitation message were received, etc.

9-10.	A 5G ProSe UE-to-UE Relay sends a 5G ProSe UE-to-UE Relay Discovery Response message. A 5G ProSe UE-to-UE Relay may associate the User Info ID and Layer-2 ID of neighbour UE-to-UE Relay(s) according to the Response message. The association may be used in the subsequent Link Management procedures.

11.	The Source End UE may perform the UE-to-UE Relay(s)/path selection for subsequent Link Management procedures based on e.g. the PC5 signal strength of the received Response messages, the number of hops to Target 5G ProSe End UE.

CHANGE 4

##### 6.7.5.2.2	5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-UE Relay for Non-IP Type PDU

Figure 6.7.5.2.2-1: Layer-2 link establishment via 5G ProSe Layer-3 UE-to-UE Relay for Non-IP Type PDU

1.	5G ProSe Layer-3 End UE1 and 5G ProSe Layer-3 End UE2 are authorized for multi-hop UE-to-UE Relay service as End UE and are provisioned with parameters for discovery and connection setup with other UEs via multi-hop UE-to-UE Relay services.

5G ProSe Layer-3 UE-to-UE Relays are authorized for multi-hop UE-to-UE Relay service as Relay UE and are provisioned with parameters for discovery and connection setup with other UEs and relay UEs via multi-hop UE-to-UE Relay services (as described in clause 5.1.5.1). The provisioned parameter may include parameters such as RSC (Relay service Code)(s), list of PLMN, user info of UE for application which are allowed at multi-hop relay connection.

2.	It is assumed that the source 5G ProSe Layer-3 End UE1 selects a relay path to the Target End UE2 from the discovery procedure which is captured in clause 6.3.2.4.

3.	The source 5G ProSe Layer-3 End UE1 initiates the unicast Layer-2 link establishment procedure with the 5G ProSe Layer-3 UE-to-UE Relays by sending a Direct Communication Request message to the UE-to-UE Relay. The parameters included in the Direct Communication Request message are described in clause 6.4.3.10.2.

If there is already a PC5 link with the same RSC been established between the End UE and the UE-to-UE Relay or between UE-to-UE Relays, a Link Modification Request message is sent instead of Direct Communication Request message. The parameters included in the LMR message are described in clause 6.4.3.10.2.

4.	(Optional) When the security protection is enabled, the source 5G ProSe Layer-3 End UE sends the parameters as described in clause 6.4.3.10.2 to the 5G ProSe Layer-3 UE-to-UE Relay.

5.	A UE-to-UE Relay sends a Direct Communication Request or Link Modification Request message to the next UE-to-UE Relay according to the path information in the received Direct Communication Request or Link Modification Request message.

The Source Layer-2 ID of the Direct Communication Request message is self-assigned by the UE-to-UE Relay and the Destination Layer-2 ID is the unicast Layer-2 ID associated with the User Info ID of the next hop UE-to-UE Relay. This association may be determined by UE-to-UE relay during discovery procedure in Step 2.

7.	A UE-to-UE Relay may know it is the last Relay in the path (e.g., according to the User Info ID of Relays in the received Direct Communication Request/Link Modification Request message). The UE-to-UE Relay sends a Direct Communication Request/Link Modification Request message to the target 5G ProSe Layer-3 End UE.

The Destination Layer-2 ID is the unicast Layer-2 ID of target End UE or a broadcast Layer-2 ID. Unicast Layer-2 ID is used if the Layer-2 ID of the Target End UE is known to the UE-to-UE Relay.

9.	The target End UE sends a Direct Communication Accept message to the UE-to-UE Relay it has successfully established security with. The DCA message may include the path information.

10-11.	After receiving a DCA message, the UE-to-UE Relay (e.g. Relay2) sends a DCA message to the next UE-to-UE Relay (e.g. Relay1) or Source End UE it has successfully established security with. The UE-to-UE Relay may decide the next UE-to-UE Relay (e.g. Relay1) according to the received DCA message or from which it previously received a corresponding DCR message.

For Ethernet communication, the 5G ProSe Layer-3 UE-to-UE Relay maintains the association between PC5 links and Ethernet MAC addresses received from the 5G ProSe Layer-3 End UE.

For Unstructured traffic communication, for each pair of source and target 5G ProSe Layer-3 End UEs, the 5G ProSe Layer-3 UE-to-UE Relay maintains the 1:1 mapping between two PC5 links to the (next hop to) source 5G ProSe Layer-3 End UE and the (next hop to) target 5G ProSe Layer-3 End UE.

End of Changes