3GPP TSG-SA2 Meeting #166	S2-2412309

Orlando, USA, November 18 – 22, 2024

Source:	Google

Title:	Discussion on the use of device identifiers in URSP and NAS SM messages

Agenda Item:	19.8.2

Document for:	Discussion and Decision

# 1.	Background

In the past few meetings, a number of questions related to KI#4 have remained unresolved. A few of these questions are listed below.

- Should URSP rules be enhanced to include device identifiers as traffic descriptors?

- Which NAS SM messages should carry device identifiers?

- Which entity, if any, restricts the maximum number of non-3GPP devices that can connect via a UE or 5G-RG?

In the sequel, we provide our views on the questions listed above.

# 2.	Device identifier as traffic descriptor

One of the key architectural assumptions made during the study phase is as follows:

-	Subscriber/subscription information will not be moved into a user profile and information from the user profile should not be used to override information in a subscription. For example, the slices and DNNs that are available to the UE do not change based on the user of the UE.

Observation 1: In the context of KI#4, “user of the UE” refers to the non-3GPP device connecting via the UE (or 5G-RG). Based on this assumption, it seems reasonable to conclude that it is up to UE implementation to enable its non-3GPP devices connecting via a UE/5G-RG to access a particular combination of the slices and DNNs that are part of the UE/5G-RG’s subscription, subject to operator policy and QoS requirements.

Proposal 1: Device identifiers should not be used to determine if a particular network slice/DNN is available.

Observation 2: The chief purpose of assigning a device identifier to a non-3GPP device connecting via a UE or 5G-RG is to enable the network to provide differentiated QoS. Fortunately existing traffic descriptors already provide a rich array of choices that can be used to select an appropriate slice/DNN for the QoS profile denoted by a device identifier. For example, the “connection capability” traffic descriptor provides for 16 standardized traffic categories defined in TS 24.526, according to the requirements present in GSMA PRD NG.135. Moreover, the “application identity” traffic descriptor allows for multiple OS-dependent identifiers for classifying application traffic. Either TD or both or some other TD (e.g., FQDN) can be associated to a non-3GPP device connecting via a UE/5G-RG based on UE implementation.

Proposal 2: Existing traffic descriptors in URSP are sufficient to achieve traffic differentiation. A new traffic descriptor, corresponding to device identifier, is not pursued.

# 3.	NAS SM message for carrying device identifier

During the study phase, it was agreed that the UE/5G-RG may send device identifier and user plane address of the non-3GPP device to the SMF in a NAS-SM message. In the last meeting (SA2#165), there appeared to be a broad consensus that the PDU session modification request message should carry the device identifier. However, it was not resolved whether the PDU session establishment request message should also carry the device identifier.

Observation 3: In our view, a device identifier is useful only if it is associated with a user plane address. Otherwise, the network cannot configure packet filters to provide differentiated QoS. In the case of IP addresses, the user plane address is not available at the time the UE sends the PDU session establishment request. Even for MAC addresses, it is possible that the MAC address is not available (e.g., when the PDU session is established before the non-3GPP device becomes active). In any case, there appears to be no loss of functionality if only the PDU session modification request carries the device identifier and associated user plane address, albeit at some overhead cost of additional signalling.

Proposal 3: No change to PDU session establishment procedure is needed for providing differentiated QoS for non-3GPP device connecting via a UE/5G-RG.

# 4.	Restricting maximum number of non-3GPP devices

A number of options have been identified to restrict the maximum number of non-3GPP devices per UE/5G-RG. A summary of these options are provided below.

Option 1, AMF/UDM + UE/5G-RG: The AMF signals the maximum number of device identifiers to the UE, based on subscription information in the UDM, and the enforcement of the limit is done by the UE or 5G-RG.

Option 2, AMF/UDM: The AMF reads the maximum number of device identifiers from the subscription information in the UDM, and enforces the limit.

Option 3: PCF/UDR: The UDR is provisioned bwith the maximum number of device identifier by the AF, and the SM-PCF is responsible for enforcement.

Option 4, implicit limit: The maximum number of simultaneously active device identifiers is implicitly enforced by limiting the number of device identifiers that are configured at the UDR.

Observation 4: Regarding Option 2-3, for network based enforcement options (Option 2 and Option 3) to work, it becomes necessary for the UE/5G-RG to signal the activity status of each non-3GPP device that the UE/5G-RG serves. Such an approach will lead to excessive signaling without any obvious benefit.

Moreover, the network based approaches, work by preventing the last request (which would exceed the quota) from being successfully connected to the network. However, it should be up to the UE/5G-RG to decide which devices should be connected based on user preference when quota restrictions become active. For these reasons, we find the network based approaches problematic.

Proposal 4: Network based enforcement options (Option 2 and Option 3) are not feasible.

Observation 5: Option 1violates the spirit of the conclusion in the TR that the UE is configured with device identifiers using mechanisms that are out-of-scope of 3GPP.

Proposal 5: The mechanism of AMF signaling the maximum number of device identifiers to the UE is not pursued.

Observation 6: In practice, the determination of whether a non-3GPP device is active or not can only be reliably performed by the UE/5G-RG. It seems reasonable that such mechanisms can also be relied on to configure the maximum number of simultaneously active non-3GPP devices.

Proposal 6: Based on observations 4-6, we propose a new option (Option 5, third party provisioning + UE/5G-RG): The provisioning mechanism of device identifiers is also used to configure the maximum number, and the enforcement is done by the UE/5G-RG.

Option 5 does not require any network level device identifier allocation and network level enforcement. It also provides UE/5G-RG with the flexibility to connect the devices they care about, when the maximum limit applies.

Observation 7: Option 4 is also feasible, even though it does impose a somewhat artificial constraint on how many device identifiers can be configured. But we note that Option 4 has no specification impact, since it can be achieved by suitable configuration in an operator’s network.

Proposal 7: Either Option 4 or Option 5 or both are agreed to limit the maximum of simultaneously active non-3GPP devices that can be served by a UE/5G-RG.

# 5.	Conclusions

In this document, we have provided our views on some outstanding questions related to KI#4. Our proposals are summarized below. A draft CR implementing Option 5 can be found in S2-24xxxxx.

Proposal 1: Device identifiers should not be used to determine if a particular network slice/DNN is available.

Proposal 2: Existing traffic descriptors in URSP are sufficient to achieve traffic differentiation. A new traffic descriptor, corresponding to device identifier, is not pursued.

Proposal 3: No change to PDU session establishment procedure is needed for providing differentiated QoS for non-3GPP device connecting via a UE/5G-RG.

Proposal 4: Network based enforcement options (Option 2 and Option 3) are not feasible.

Proposal 5: The mechanism of AMF signaling the maximum number of device identifiers to the UE is not pursued.

Proposal 6: Based on observations 4-6, we propose a new option (Option 5, third party provisioning + UE/5G-RG): The provisioning mechanism of device identifiers is also used to configure the maximum number, and the enforcement is done by the UE/5G-RG.

Proposal 7: Either Option 4 or Option 5 or both are agreed to limit the maximum of simultaneously active non-3GPP devices that can be served by a UE/5G-RG.